| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0372 |
| --- | --- |

| Title: | CE6: Fast DST-7/DCT-8 based on DFT and matrix multiplication (test 6.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. NASER E. Francois, F. Le Léannec | Tel: Email: | Karam.naser@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution provides the description and experimental results of CE6-2.2.

# Introduction

In CE6-2.2, fast implementation of DCT8 and DST7 for the sizes 32x32 and 16x16 is proposed. It is based on DFT decomposition where length 2N+1 DFT is employed to implement length N DCT8/DST7. That is, length 32 DCT8/DST7 is implemented by length 65 DFT and length 16 DCT8/DST7 is implemented by length 33 DFT. Furthermore, length 65 DFT is decomposed into 5x13 2D DFT and length 33 DFT is decomposed into 3x11 2D DFT using prime factor algorithm.

In summary, the method comprises the following steps:

- N2 calls to DFT_RI involving N1-DFTs

- 1 call to DFT_RI_IO involving N2-DFTs

- (N1+1)/2 calls to DFT_IO involving N2-DFTs

- Plus permutations of input/output

where (N1, N2) are (3, 11) for 16x16 DCT8/DST7 and (5, 13) for 32x32 DCT8/DST7. RI and IO stands for real input and imaginary output (resp.). Further details are found in ‎[1].

# Implementation

In the following, DFT basis functions are represented as:

DFT = R+jI

where R is the real part (cosine part) and I is the imaginary part (sine part).

It should be noted that R is even symmetric, and I is odd symmetric. In addition, the first basis function of R is all ones, which requires only the summation to compute the first real coefficients, whereas the first basis function of I is all zeros, which set the first imaginary coefficient to zero without requiring any mathematical operation.

When multiplying an input vector v by R, the following algorithm is used:

ALGORITHM 1 Mult_DFTr

Input: vector v of length n and real part DFT matrix R (nxn)

Output: vector V of length (n+1)/2

- Sum values: 
for i=1:(n-1)/2
   s(i-1) = v(i)+v(n-i)

- Computing first coefficient:
V(0) = v(0)+sum(s)

- Computing the rest of coefficient:
V = v(0) + R(1:(n-1)/2),1:(n-1)/2)*s

# additions = (n-1)/2 + (n-1)/2 + (n-1)/2*(n-1)/2 + (n-1)/2 = 3*(n-1)/2 + (n-1)^2/4

# multiplications = (n-1)/2*(n-1)/2 = (n-1)^2/4

ALGORITHM 2 Mult_DFTi

Input: vector v of length n and imaginary part DFT matrix I (nxn)

Output: vector V of length (n+1)/2

- Diff values: 
for i=1:(n-1)/2
   d(i-1) = v(i)-v(n-i)

- Computing first coefficient:
V(0) = 0

- Computing the rest of coefficients:
V(i) = R(1:(n-1)/2,1:(n-1)/2)*d

# additions = (n-1)/2 + (n-1)/2*(n-1)/2 = (n-1)/2 + (n-1)^2/4

# multiplications = (n-1)/2*(n-1)/2 = (n-1)^2/4

Otherwise, when multiplying an input vector v by I, the following algorithm is used:

## DFT_RI

For a real input vector (r), the real DFT coefficients Cr are computed by multiplying r by the real part R and the imaginary coefficients Ci are computed by multiplying r by the imaginary part I of DFT. That is:

Cr = r x R

Ci = r x I

The number of operations is as follows:

- Additions: 3*(n-1)/2 + (n-1)^2/4 + (n-1)/2 + (n-1)^2/4 = 2*(n-1)  + (n-1)^2/2

- Multiplications: (n-1)^2/4 + (n-1)^2/4 = (n-1)^2/2

## DFT_RI_IO

For a real input vector (r), the imaginary coefficients Ci are computed by multiplying r by the imaginary part I of DFT. That is:

Ci = r x I

The number of operations is as follows:

- Additions: (n-1)/2 + (n-1)^2/4

- Multiplications: (n-1)^2/4

## DFT_ IO

For a complex input vector (r+ji), the imaginary coefficients Ci are computed by as follows:

Ci = r x I + i x R

From this equation, it can be observed that n additions are required in addition to Mult_DFTr and Mult_DFTi. Furthermore, since the output coefficients are scaled by a transform scaling factor, the first coefficients require on more multiplication.

Therefore, the number of operations is as follows:

- Additions: 2n-1 + (n-1)^2/4 + (n-1)/2 + (n-1)^2/4 + n = (7n-3)/2 + (n-1)^2/2

- Multiplications: (n-1)^2/4 + (n-1)^2/4 + 1 = (n-1)^2/2 + 1

It should be noted that the scaling can be done at the quantization level. Therefore, the +1 addition can be removed and the total number of multiplications becomes:

- Multiplications: (n-1)^2/4 + (n-1)^2/4 + 1 = (n-1)^2/2

## Total number of operations

Summing up the number of operations required by each process, the table below shows the total number of operations required to implement the DFT based fast DCT8/DST7 transforms.

| Process | Additions | Multiplications |
| --- | --- | --- |
| N2 DFT_RI(N1) | N2*( 2(N1-1) + (N1-1)^2/2) | N2*((N1-1)^2/2) |
| 1 DFT_RI_IO(N2) | (N2-1)/2 + (N2-1)^2/4 | (N2-1)^2/4 |
| (N1-1)/2 DFT_IO(N2) | (N1-1)/2*( (7*N2-3)/2 + (N2-1)^2/2) | (N1-1)/2*((N2-1)^2/2) |

Using this table, the 16x16 DCT8/DST7 requires the following number of operations:

| Transform | (Adds)/(VTM_Adds) | (Mults)/(VTM_Mults) |
| --- | --- | --- |
| 16x16 DCT8/DST7 | 183/240=76.25% | 97/256=37.89% |
| 32x32 DCT8/DST7 | 482/992=48.59% | 284/1024=27.73% |

Therefore, the total number of additions can be reduced to 76% and 49% and the number of multiplications is reduced to 38% and 28% of their original values in case of transform sizes of 16x16 and 32x32 (resp.).

# Memory requirement

The overall memory requirement of MTS implementation as proposed in this contribution is given here, in comparison with the current design of VTM. The number of matrix coefficients of each DFT matrix is provided in the table below:

| Matrix | # coefficients |
| --- | --- |
| DFT_r_3 | 1 |
| DFT_i_3 | 1 |
| DFT_r_5 | 4 |
| DFT_i_5 | 4 |
| DFT_r_11 | 25 |
| DFT_i_11 | 25 |
| DFT_r_13 | 36 |
| DFT_i_33 | 36 |
| Sum | 132 |

The method is applicable to 16x16 and 32x32 DCT8/DST7, where 4x4 and 8x8 DCT8/DST7 remain the same. Therefore, 80=16+64 matrix coefficients are required to store DCT8/DST7 of sizes 4x4 and 8x8.

Test CE6-2.2a considers 16 bits representation of DFT matrices. Therefore, 264=132*2 bytes are required, whereas test CE6-2.2b considers 8 bits representation, which requires 132 bytes. With 8 bits representation of transform matrices, the total memory required for test CE6-2.2a is 264+80=344 bytes and 132+80=212 bytes for test CE6-2.2b. This respectively  represents 25.29% and 15.59% of the current memory consumed for MTS in VTM-3.0 software.

It should be noted that the memory required for the core transform (4096 Bytes) remains unchanged. Therefore, the total memory for the transform design is 4439 bytes and 4308 bytes for test CE6-2.2a and test CE6-2.2b.

# BitDepth Restriction

The proposed method consists of two stages of DFT, each of which performs matrix multiplications with DFT matrices. The current design restricts the internal bit depth not to exceed 16 bits (signed representation) so that multiple instruction can be easily deployed.

The first stage DFT matrices are up-shifted to either 16 bits integer representation (test CE6-2.2a) or 8 bits integer representation (test CE6-2.2b). The intermediate coefficients, i.e. after the first stage, are down shifted by the same up-shift value plus ceil(log2(n)) where n is the length of first stage DFT. This is done to ensure the same depth of the coefficients and the input signal to the first stage.

# Simulation results

## Common Testing Conditions

Table 1: Results of common testing condition with 16 bits representation of DFT matrices (test CE6-2.2a)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.14% | 0.17% | 122% | 114% |
| Class A2 | 0.13% | 0.08% | 0.06% | 118% | 112% |
| Class B | 0.08% | 0.02% | 0.03% | 119% | 114% |
| Class C | 0.03% | 0.02% | -0.06% | 114% | 110% |
| Class E | 0.05% | 0.05% | 0.06% | 117% | 109% |
| Overall | 0.08% | 0.05% | 0.04% | 118% | 112% |
| Class D | 0.02% | 0.04% | 0.02% | 111% | 108% |
| Class F | 0.02% | 0.06% | 0.12% | 113% | 106% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.04% | 0.21% | 121% | 108% |
| Class A2 | 0.02% | -0.04% | 0.09% | 116% | 106% |
| Class B | 0.03% | 0.18% | 0.10% | 115% | 103% |
| Class C | 0.00% | 0.04% | 0.12% | 112% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.07% | 0.12% | 116% | 105% |
| Class D | 0.00% | -0.42% | -0.05% | 110% | 95% |
| Class F | 0.01% | -0.05% | 0.17% | 106% | 86% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | 0.00% | -0.11% | 101% | 99% |
| Class C | 0.00% | 0.22% | 0.17% | 101% | 100% |
| Class E | 0.00% | 0.56% | -0.71% | 100% | 100% |
| Overall | 0.04% | 0.22% | -0.17% | 101% | 100% |
| Class D | -0.05% | -0.12% | -0.49% | 100% | 101% |
| Class F | 0.06% | 0.47% | -0.15% | 100% | 101% |

Table 2: Results of common testing condition with 8 bits representation of DFT matrices (test CE6-2.2b)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.10% | 0.20% | 122% | 114% |
| Class A2 | 0.14% | 0.07% | 0.07% | 118% | 111% |
| Class B | 0.09% | 0.06% | 0.09% | 119% | 114% |
| Class C | 0.03% | 0.07% | 0.00% | 114% | 110% |
| Class E | 0.08% | -0.04% | 0.10% | 117% | 110% |
| Overall | 0.09% | 0.05% | 0.09% | 118% | 112% |
| Class D | 0.04% | 0.02% | 0.02% | 111% | 108% |
| Class F | 0.02% | 0.11% | 0.18% | 112% | 106% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.15% | 0.11% | 120% | 107% |
| Class A2 | -0.01% | 0.11% | 0.22% | 115% | 105% |
| Class B | 0.00% | 0.27% | -0.07% | 115% | 103% |
| Class C | 0.01% | -0.14% | -0.01% | 112% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.11% | 0.04% | 116% | 104% |
| Class D | 0.04% | -0.27% | -0.27% | 110% | 95% |
| Class F | 0.02% | -0.06% | 0.14% | 106% | 86% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | 0.00% | -0.11% | 101% | 99% |
| Class C | 0.00% | 0.22% | 0.17% | 101% | 100% |
| Class E | 0.00% | 0.56% | -0.71% | 100% | 100% |
| Overall | 0.04% | 0.22% | -0.17% | 101% | 100% |
| Class D | -0.05% | -0.12% | -0.49% | 100% | 101% |
| Class F | 0.06% | 0.47% | -0.15% | 100% | 101% |

## Common Testing Conditions with Inter-MTS

Table 3: Results of common testing condition with 16 bits representation of DFT matrices (CE6-2.2a)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0_MTS3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.00% | 0.20% | #DIV/0! | #DIV/0! |
| Class A2 | 0.06% | -0.11% | 0.02% | #DIV/0! | #DIV/0! |
| Class B | 0.04% | -0.05% | 0.13% | #DIV/0! | #DIV/0! |
| Class C | 0.00% | 0.16% | 0.13% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.00% | 0.12% | #DIV/0! | #DIV/0! |
| Class D | 0.07% | -0.38% | -0.11% | #DIV/0! | #DIV/0! |
| Class F | 0.04% | -0.08% | 0.24% | #DIV/0! | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0_MTS3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.04% | -0.40% | #DIV/0! | #DIV/0! |
| Class C | 0.08% | -0.07% | 0.16% | #DIV/0! | #DIV/0! |
| Class E | 0.03% | 0.68% | -0.25% | #DIV/0! | #DIV/0! |
| Overall | 0.07% | 0.13% | -0.18% | #DIV/0! | #DIV/0! |
| Class D | 0.09% | 0.39% | -0.60% | #DIV/0! | #DIV/0! |
| Class F | -0.14% | -0.11% | -0.02% | #DIV/0! | #DIV/0! |

Results of common testing condition with 8 bits representation of DFT matrices (CE6-2.2b)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0_MTS3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.08% | 0.11% | #DIV/0! | #DIV/0! |
| Class A2 | 0.06% | 0.01% | 0.08% | #DIV/0! | #DIV/0! |
| Class B | 0.05% | 0.16% | -0.04% | #DIV/0! | #DIV/0! |
| Class C | 0.02% | -0.02% | 0.07% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.07% | 0.04% | #DIV/0! | #DIV/0! |
| Class D | 0.05% | -0.21% | -0.06% | #DIV/0! | #DIV/0! |
| Class F | 0.04% | -0.10% | 0.14% | #DIV/0! | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0_MTS3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | 0.35% | 0.05% | #DIV/0! | #DIV/0! |
| Class C | 0.05% | 0.14% | 0.30% | #DIV/0! | #DIV/0! |
| Class E | 0.20% | 0.37% | 0.04% | #DIV/0! | #DIV/0! |
| Overall | 0.10% | 0.28% | 0.13% | #DIV/0! | #DIV/0! |
| Class D | 0.08% | -0.27% | -0.32% | #DIV/0! | #DIV/0! |
| Class F | -0.09% | 0.51% | 0.16% | #DIV/0! | #DIV/0! |

# Complexity metrics

| Metrics | Proposed method |
| --- | --- |
| Operation counts for individual transforms. | - 16x16: 140 Adds, 98 Mults - 32x32: 386 Adds, 286 Mults |
| Memory requirements for storing transform cores (in bytes). | - Test CE6-2.2a: 4439 - Test CE6-2.2a: 4308 |
| Specify the minimum bit-precision required by the transforms - To represent transform coefficients or additional parameters - Precision of arithmetic operations needed during transform computation - For storing intermediate data representation (e.g., between two stages) | The same as VTM-3.0 - 8 bits for CE6-2.2b and 16 bits for CE6-2.2a - 25 bits - 16 bits |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. | None |
| List of all combinations of transform types (e.g., DCT-2 and DST-7) and transforms block sizes (e.g., 4x4, 4x8, 8x4 and 8x8) used for primary transformation. | Same as VTM-3.0 |
| List of all combinations of transforms block size and transform type used for secondary transformation. | The same as those for primary transform. |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations) | Sec. ‎2.4 |
| Worst-case analysis (e.g., multiplications per pixel) | Same as VTM-3.0 |

# Conclusion

In this contribution, the use of a matrix multiplication-based implementation of DCT8/DST7 is studied. It is employed the DFT decomposition and prime factor algorithm for reducing both the memory and number of operations required for computing DCT8/DST7 for sizes of 16x16 and 32x32. The experimental results showed a negligible loss in BD rate in CTC.

# References

- Karam Naser, Gagan Rath, Edouard Francois, “CE6-related: fast implementation of MTS transforms using matrix multiplication” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-L0412

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).