| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0383 |
| --- | --- |

| Title: | Non-CE3: Table size reduction and bit width limitation for CCLM implementation. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Patrice Onno Christophe Gisquet Guillaume Laroche Jonathan Taquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes to modify the derivation of the linear model parameters for the CCLM mode.

In one aspect of the proposal, the two tables of 512 integers coded on 16 bits used for deriving the  and  parameters of the CCLM mode in the VTM 3.0 are reduced to two tables of  128 (or 256) entries where each integer value is coded on 8 bits. It is asserted that the results provided by the modifications are respectively 0.01%/0.00% (or 0.01/-0.01%) for the AI/RA configurations while reducing the storage of the table by 8 (or by 4).

In a second aspect of the proposal, the  parameter is coded on a maximum number of 6 bits in order to limit the bit width of the multiplication in the CCLM prediction loop. It is asserted that the results provided by this modification above the first aspect are respectively 0.02%/0.01% for the AI/RA configurations.

# Introduction

During the 12th JVET meeting, the Least Mean Square (LMS) algorithm, used for the derivation of the linear model α and β of the CLLM mode, was removed. These parameters were derived by minimizing the regression error between the neighbouring Luma and Chroma samples.

Now in VTM3.0 and after the adoption of the contribution JVET-L0191, the minimum and maximum Luma samples and their corresponding Chroma samples are used to obtain the linear model and derive the  and  parameters from the straight line passing through these points. This method enables to avoid the costly regression error minimization algorithm of LMS where all Luma and all Chroma samples from the outer border are used for the derivation.

The following Figure 1 illustrates the new method for deriving the parameters of the CCLM mode in VTM 3.0,

Y=αx+β

A

B

Figure 1 Illustration of the straight line between minimum and maximum Luma value

where the linear model parameters α and β are obtained according to the following equation (1):

 = (yB- yA) /(xB – xA)                                                                                            (1)

 = yA – .xAβ= ∑Cn-α·∑LnN                                                                                                         (2)

In order not to apply any floating point operation for the prediction loop and the derivation of the  parameter, integer arithmetic is used where only integer multiplications and bit shift operation are applied.

In a first aspect, for the prediction loop, the following formula (3) is used instead:

Cc =  (( *Lc) >> k) + 

where Cc is the Chroma predicted value corresponding to the Luma Lc value.

In a second aspect and regarding the division to derive the  parameters according to the formula (3), two tables (called g_aiLMDivTableHigh[] and g_aiLMDivTableLow[]in VTM 3.0) are stored in memory to represent all the possible values of the numerator (yB-yA) of equation (1).  In VTM3.0, these two tables have 512 entries and they are used for implementing the integer arithmetic. Each entry corresponds to an integer that is currently represented on 16 bits in VTM3.0. The storage of these two tables represents a significant amount of memory (2 KBytes) that can be significantly reduced without impacting the performance.

# Proposed method

## Table size reduction

In order to minimize the memory needed to store these tables, the size of the two tables (called g_aiLMDivTableHigh[] and g_aiLMDivTableLow[] in the VTM 3.0 reference software) used for the determination of the  parameters are modified. Their sizes are reduced from 512 entries to 128 (or 256 entries).

In addition, the number of bits to represent each integer are reduced from k=16 bits to k=8 bits.

As a result, the total memory to store the two tables is divided by 8 (or by 4 if we consider table size of 256).

## Restriction of the number of bits to represent “”

With the derivation process of  and  described in VTM3.0, the value of  could reach up to 17 bits when k=8.  A primary reason to modify the value of is to limit the bit width of the multiplication in the prediction formula (3) of the CCLM mode:

If the luma samples (Lc in formula (3)) are coded on 10 bits, it means that the core loop of prediction is required to be able to handle multiplication of up to 10 bits time 17 bits, which is computationally complex. In this proposal, the value of  is modified, in the above formula (2), so that the result of the multiplications in the prediction loop does not exceed 16 bits.

In order to achieve this, it is proposed to limit the representation of  on L=6 bits in the above formula (3).  The 6 bits can represent up to 64 values which gives enough accuracy to maintain a good performance as presented in the next section.

# Experimental results

The proposed method have been evaluated on top of VTM3.0 by using the Common Test conditions of JVET-L1010.

Please note that in the following decoding times are not reliable in the attached results.

## Table size of 256 and entry coded on 8 bits.

The proposed method is implemented on top of VTM 3.0 where the two tables have a size of 256 entries coded on 8 bits.

The proposed method respectively gives an average BDR loss of 0.01% and -0.01% for AI and RA configuration.

Full results on LDP and LDB are provided in the attached Excel sheets.

## Table size of 128 and entry coded on 8 bits.

The proposed method is implemented on top of VTM 3.0 where the two tables have a size of 128 entries coded on 8 bits

The proposed method respectively gives an average BDR loss of 0.01% and 0.00% for AI and RA configurations.

Full results on LDP and LDB are provided in the attached Excel sheets.

## Table size of 128, entries coded on 8 bits + representation of “” on 6 bits.

The proposed method is implemented on top of VTM 3.0 where the two tables have a size of 128 entries coded on 8 bits with the additional restriction of having only a 6-bit representation for 

The proposed method respectively gives an average Y-BDR loss of 0.02% for AI and 0.01% for RA configuration.

Full results on LDP and LDB are provided in the attached Excel sheets.

# Conclusion

This contribution proposes two simplifications to derive the  and  parameters of the linear model for the CCLM mode.

The first modification is about the table size reduction to perform the integer arithmetic. The results show that by reducing the size of the two tables by 8 (or 4) and having integer values represented on 8-bits the performance of the VTM 3.0 is are hardly affected.

The second modification is about the restriction of L=6 bits in order to limit the bit width of the multiplication in the prediction loop. Again, the results of this addition restriction in VTM3.0 show a negligible impact in terms of coding efficiency.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).