| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0387-v2 |
| --- | --- |

| Title: | AHG14: Updates on Intra Refresh Proposal |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jean-Marc Thiesse, Didier Nicholson, David Gommelet VITEC 99 rue Pierre Semard 92320 Chatillon, France | Tel: Email: | +33 1 46 73 06 06 jean-marc.thiesse@vitec.com didier.nicholson@vitec.com david.gommelet@vitec.com |
| Source: | VITEC |  |  |

_____________________________

# Abstract

This contribution reports an update of the software modifications for the study of progressive intra refresh (AHG 14). This code has been made available in the AHG Gitlab repository. Both encoder-only modifications and a normative solution have been reported to VTM 3.0.1 with handling of new tools adopted at Macao meeting. The corresponding performance are in the same range as the results presented at Macao meeting with a loss of 21.29% in average for the encoder-only modifications against a low-delay B reference with a comparable Intra period of 32. This significant loss demonstrates that a better handling of progressive intra refresh in VVC is relevant. The normative proposal brings an improvement of 5.26% against the encoder-only modification solution by adding a straightforward signalization to derive the position of the intra refresh area and then exploit this position for decoding process.

In order to proceed on the AHG 14 activity, it is recommended to agree on using the proposed testing conditions for next intra refresh assessment and to integrate the encoder-only modifications for intra-refresh support on VTM 4.

# Introduction

A proposal for normative handling of progressive intra refresh was made at Ljubljana [1] and lead to the creation of the Ad hoc Group 14 which aims at studying the opportunity of integrating such a normative solution. At Macao, [2] provides a great overview of the issues to tackle to have an efficient support of intra refresh within VVC. We respectively proposed in [3] an encoder-only implementation of intra refresh and in [4] an implementation of the proposed intra refresh support. This new contribution provides an update of these two contributions with integration within VTM 3. In particular, triangle tool handling has been tackled.

The code corresponding to this contribution is available in AHG 14 Gitlab: https://vcgit.hhi.fraunhofer.de/jvet-l-ahg-14/VVCSoftware_VTM/tree/AHG14-L0160.  The modifications for encoder-only intra refresh are available through JVET_M0387_INTRA_REFRESH_ENC_ONLY define while the modifications for proposed normative solution are available through JVET_M0387_INTRA_REFRESH_NORMATIVE define.

# Encoder-only intra refresh

Figure 1: Intra Refresh principle.

The software modifications to support Intra Refresh functionality at the encoder-level in VTM software include:

- Intra prediction mode forced on coding unit on column basis.

- Constrained Intra Prediction enabled to ensure reconstruction of Intra CU (Constrained Intra Prediction as define in HEVC is used since it is available in VTM while it is not yet included in the VVC draft).

- Motion vectors constrained to point within the clean area while taking into account an additional margin to avoid motion compensation filters error spreading (5 pixels for instance).

- Removing of former reference pictures when re-looping the Intra column.

- Disabling of Adaptive Loop Filter at CTU level for CTU including Intra refresh pixels.

The additional parameters added to the configuration file are as follows:

IntraRefreshType			  : 1			# Intra Refresh type flag 0: disable, 1: enc-only, 2: normative

IntraRefreshStart			  : 0			# Position of the first column of Intra refresh (in pixels)

IntraRefreshDirection		  : 0			# Direction of Intra Refresh 0: left to right, 1: right to left

IntraRefreshPeriod			  : 32		# Period of the Intra Refresh

# Proposed normative intra refresh

## Description of the method

The proposed solution adds the minimum signalization needed for deriving the position of Intra Refreshed pixels and then exploits this position knowledge to improve the quality within this low-latency use-case. The modifications at the encoder side involved:

- Avoid the transmission of pred_mode_flag for Coding Unit with at least one pixel within the Intra Refresh Area.

- Enable the Constrained Intra Prediction (CIP) tool adaptively depending on the position and direction of the Intra Refresh Area.

- Disabling of Deblocking Filter at the Intra Refresh boundary.

Other possible modifications (not tested yet), as also suggested in [1], include:

- Take care of the Intra Refresh position for the motion vectors derivation.

- Define two deltas QP by slice depending on the Intra Refresh Area position.

## Signalization

The proposed additional signalization is:

- at SPS level:

  - sps_intra_refresh_enabled_flag specifies if the Intra Refresh tool is enabled.

- at PPS level:

  - pps_intra_refresh_mode specifies the mode of Intra Refresh (Column or Line for instance)

  - pps_intra_refresh_size  specifies the size of the refresh pattern (width of the Column, height of the Line, width x height for block-based refresh). This can be signaled as a multiple of the CU min size (0 for 8 pixels, 1 for 16 pixels, …).

  - pps_intra_refresh_delta_qp specifies a delta QP for the Intra Refresh blocks. This allows to optimize the encoder quality balancing between Intra Refresh blocks and others.

- at Tiles-group level:

  - tile_group_intra_refresh_direction specifies the direction of the refresh (right to left/left to right for Column, top to bottom/bottom to top for Line, …). This is used to determine motion vectors constraints during the encoding/decoding.

  - tile_group_intra_refresh_position specifies the position of the Intra Refresh blocks, this is given according to pps_intra_refresh_size.

The associated draft modifications are proposed in annex.

# Results

## Test Conditions

It is proposed to use the following test conditions for intra refresh assessment within AHG14. The associated configuration files are attached to the contribution.

Note: this has not been used for the following results but it may be fair to remove the first Intra picture from bitrate and PSNR computation in order to catch the real intra refresh performance.

### Intra refresh

The Low Delay B 10 configuration file is used as a basis, with the two following modifications:

- The period of Intra Refresh is set at 32 to match the RA Intra Period. The size of the column for Intra Refresh is then computed automatically to match this. For information, the resulting refresh sizes are reported in following table:

| Resolution | Column Size |
| --- | --- |
| 3840x2160p | 120 |
| 1920x1080p | 64 |
| 1280x720p | 40 |
| 832x480p | 32 |
| 704x576p | 24 |
| 416x240p | 16 |

- The GOP size is set to 1 instead of 4 with a QP offset of 1 in order to be close to an Intra Refresh scenario where the goal is to keep the same picture bits size along the sequence.

### Reference (non-“intra refresh”)

In order to characterize the intra refresh efficiency as requested in AHG 14 mandates, the intra refresh implementation is compared against a reference which is built similarly upon the Low Delay B 10 configuration file with two modifications:

- Intra period of 32 together with DecodingRefreshType set to 1 in order to keep it comparable in term of amount of Intra pixels.

- Same GOP size modification (GOPSize = 1) with a QP offset of 1 between I and B picture.

## Results for encoder-only modifications against reference

|  | Low delay B Intra Refresh Encoder Only Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 12,53% | 23,35% | 35,68% | 93% | #DIV/0! |
| Class C | 18,75% | 24,36% | 26,79% | 94% | #DIV/0! |
| Class E | 39,27% | 43,52% | 51,34% | 91% | #DIV/0! |
| Overall | 21,29% | 28,73% | 36,63% | 93% | #DIV/0! |
| Class D | 22,34% | 33,67% | 36,18% | 86% | #DIV/0! |
| Class F | 37,68% | 39,80% | 42,51% | 133% | #DIV/0! |

## Results for proposed normative solution against encoder-only

|  | Low delay B Normative Intra Refresh Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Low delay B Enc-only Intra Refresh VTM3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #DIV/0! |
| Class C | -5,12% | -8,00% | -9,01% | 110% | #DIV/0! |
| Class E | -6,59% | -7,69% | -9,05% | 106% | #DIV/0! |
| Overall | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #DIV/0! |
| Class D | -5,18% | -7,74% | -8,50% | 108% | #DIV/0! |
| Class F | -8,03% | -10,09% | -10,93% | 79% | #DIV/0! |

# Conclusion

It is recommended to integrate the encoder only modifications within the next VTM software version and to use the proposed test conditions for further AHG testing.

# Acknowledgement

The research leading to this work has been supported by the FUI23 EFIGI project, BPIFrance and Region Ile de France.

# References

[1] J.-M. Thiesse, D.Nicholson, “Improved Cyclic Intra Refresh”, JVET-K0212, July 2018.

[2] K.Kazui, “AHG14: Study of methods for progressive intra refresh”, JVET-L0079, October 2018.

[3]  J.-M. Thiesse, D.Nicholson, D.Gommelet “AHG14: Intra Refresh Test conditions and Anchors generation Proposal”, JVET-L0160, October 2018.

[4] J.-M. Thiesse, D.Nicholson, D.Gommelet “AHG14: Normative Intra Refresh Proposal”, JVET-L0161, October 2018.

# Patent rights declaration(s)

VITEC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex: Syntax modifications

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| intra_only_constraint_flag | u(1) |
| max_bitdepth_constraint_idc | u(4) |
| max_chroma_format_constraint_idc | u(2) |
| frame_only_constraint_flag | u(1) |
| no_qtbtt_dual_tree_intra constraint_flag | u(1) |
| no_sao_constraint_flag | u(1) |
| no_alf_constraint_flag | u(1) |
| no_pcm_constraint_flag | u(1) |
| no_temporal_mvp_constraint_flag | u(1) |
| no_sbtmvp_constraint_flag | u(1) |
| no_amvr_constraint_flag | u(1) |
| no_cclm_constraint_flag | u(1) |
| no_affine_motion_constraint_flag | u(1) |
| no_ladf_constraint_flag | u(1) |
| no_dep_quant_constraint_flag | u(1) |
| no_sign_data_hiding_constraint_flag | u(1) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| qtbtt_dual_tree_intra_flag | ue(v) |
| log2_ctu_size_minus2 | ue(v) |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| partition_constraints_override_enabled_flag | ue(v) |
| sps_log2_diff_min_qt_min_cb_intra_tile_group_luma | ue(v) |
| sps_log2_diff_min_qt_min_cb_inter_tile_group | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_tile_groups | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_tile_groups_luma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_tile_groups_luma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_tile_group_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_tile_group_luma | ue(v) |
| } |  |
| if( sps_max_mtt_hierarchy_depth_inter_tile_groups != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_tile_group | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_tile_group | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_tile_group_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_tile_groups_chroma | ue(v) |
| if ( sps_max_mtt_hierarchy_depth_intra_tile_groups_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_tile_group_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_tile_group_chroma | ue(v) |
| } |  |
| } |  |
| sps_intra_refresh_enabled_flag | u(1) |
| sps_sao_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disabled_flag | u(1) |
| } |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset | ue(v) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_bdof_enabled_flag | u(1) |
| sps_cclm_enabled_flag | u(1) |
| sps_mts_intra_enabled_flag | u(1) |
| sps_mts_inter_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) |  |
| sps_affine_type_flag | u(1) |
| sps_gbi_enabled_flag | u(1) |
| sps_cpr_enabled_flag | u(1) |
| sps_ciip_enabled_flag | u(1) |
| sps_triangle_enabled_flag | u(1) |
| sps_ladf_enabled_flag | u(1) |
| if ( sps_ladf_enabled_flag ) { |  |
| sps_num_ladf_intervals_minus2 | u(2) |
| sps_ladf_lowest_interval_qp_offset | se(v) |
| for( i = 0; i < sps_num_ladf_intervals_minus2 + 1; i++ ) { |  |
| sps_ladf_qp_offset[ i ] | se(v) |
| sps_ladf_delta_threshold_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| init_qp_minus26 | se(v) |
| transform_skip_enabled_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| if( cu_qp_delta_enabled_flag ) |  |
| diff_cu_qp_delta_depth | ue(v) |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| if (sps_intra_refresh_enabled_flag){ |  |
| pps_intra_refresh_mode | u(1) |
| pps_intra_refresh_size | ue(v) |
| pps_intra_refresh_delta_qp | se(v) |
| } |  |
| pps_tile_group_chroma_qp_offsets_present_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

### Tile group header syntax

#### General tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| tile_group_address | u(v) |
| num_tiles_in_tile_group_minus1 | ue(v) |
| } |  |
| tile_group_type | ue(v) |
| tile_group_pic_order_cnt_lsb | u(v) |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | ue(v) |
| if( partition_constraints_override_flag ) { |  |
| tile_group_log2_diff_min_qt_min_cb_luma | ue(v) |
| tile_group_max_mtt_hierarchy_depth_luma | ue(v) |
| if( tile_group_max_mtt_hierarchy_depth_luma != 0 ) |  |
| tile_group_log2_diff_max_bt_min_qt_luma | ue(v) |
| tile_group_log2_diff_max_tt_min_qt_luma | ue(v) |
| } |  |
| if( tile_group_type = = I && qtbtt_dual_tree_intra_flag ) { |  |
| tile_group_log2_diff_min_qt_min_cb_chroma | ue(v) |
| tile_group_max_mtt_hierarchy_depth_chroma | ue(v) |
| if( tile_group_max_mtt_hierarchy_depth_chroma != 0 ) |  |
| tile_group_log2_diff_max_bt_min_qt_chroma | ue(v) |
| tile_group_log2_diff_max_tt_min_qt_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if ( tile_group_type != I ) { |  |
| if (sps_intra_refresh_enabled_flag ){ |  |
| tile_group_intra_refresh_direction | u(1) |
| tile_group_intra_refresh_position | ue(v) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| tile_group_temporal_mvp_enabled_flag | u(1) |
| if( tile_group_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( tile_group_temporal_mvp_enabled_flag ) { |  |
| if( tile_group_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enable_flag ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| } |  |
| tile_group_qp_delta | se(v) |
| if( pps_tile_group_chroma_qp_offsets_present_flag ) { |  |
| tile_group_cb_qp_offset | se(v) |
| tile_group_cr_qp_offset | se(v) |
| } |  |
| if( sps_sao_enabled_flag ) { |  |
| tile_group_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| tile_group_sao_chroma_flag | u(1) |
| } |  |
| if( sps_alf_enabled_flag ) { |  |
| tile_group_alf_enabled_flag | u(1) |
| if( tile_group_alf_enabled_flag ) |  |
| alf_data( ) |  |
| } |  |
| if( tile_group_type = = P ¦ ¦ tile_group_type = = B ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( tile_group_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| tile_group_deblocking_filter_disabled_flag | u(1) |
| if( !tile_group_deblocking_filter_disabled_flag ) { |  |
| tile_group_beta_offset_div2 | se(v) |
| tile_group_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( num_tiles_in_tile_group_minus1 > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_tiles_in_tile_group_minus1; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

#### Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( tile_group_type != I && !intra_refresh_cu) { |  |
| if( treeType != DUAL_TREE_CHROMA ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| pred_mode_flag | ae(v) |
| } |  |

…

CuIntraRefresh[ xNbY ][ yNbY ] is derived at frame level according to tile_group_intra_refresh_position. CuIntraRefresh[ xNbY ][ yNbY ] is equal to 1 if the CU belongs to the Intra Refresh area.

….

##### Reference sample availability marking process

Inputs to this process are:

- a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable refIdx specifying the intra prediction reference line index,

- a variable refW specifying the reference samples width,

- a variable refH specifying the reference samples height,

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx for intra sample prediction.

The refW + refH + 1 + ( 2 * refIdx ) neighbouring samples refUnfilt[ x ][ y ] that are constructed samples prior to the in-loop filter process, with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, are derived as follows:

- The neighbouring location ( xNbCmp, yNbCmp ) is specified by:

( xNbCmp, yNbCmp ) = ( xTbCmp + x, yTbCmp + y )	(8-78)

- The current luma location ( xTbY, yTbY ) and the neighbouring luma location ( xNbY, yNbY ) are derived as follows:

( xTbY, yTbY ) = ( cIdx  = =  0 ) ? ( xTbCmp, yTbCmp ) : ( xTbCmp << 1, yTbCmp << 1 )	(8-79)

( xNbY, yNbY ) = ( cIdx  = =  0 ) ? ( xNbCmp, yNbCmp ) : ( xNbCmp << 1, yNbCmp << 1 )	(8-80)

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ) and the neighbouring luma location ( xNbY, yNbY ) as inputs, and the output is assigned to availableN.

–	Each sample refUnfilt[ x ][ y ] is derived as follows:

– If one or more of the following conditions are true, the sample refUnfilt [ x ][ y ] is marked as "not available for intra prediction":

– The variable availableN is equal to FALSE.

– CuPredMode[ xNbY ][ yNbY ] is not equal to MODE_INTRA and CuIntraRefresh[ xCurr ][ yCurr ]  is equal to 1.

- Otherwise, the sample refUnfilt[ x ][ y ] is marked as "available for intra prediction" and the sample at the location ( xNbCmp, yNbCmp ) is assigned to refUnfilt[ x ][ y ].

…

#### Deblocking filter process for one direction

Inputs to this process are:

- the variable treeType specifying whether a single tree (SINGLE_TREE) or a dual tree is used to partition the CTUs and, when a dual tree is used, whether the luma (DUAL_TREE_LUMA) or chroma components (DUAL_TREE_CHROMA) are currently processed,

- when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the reconstructed picture prior to deblocking, i.e., the array recPictureL,

- when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered.

Outputs of this process are the modified reconstructed picture after deblocking, i.e:

- when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the array recPictureL,

- when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr.

For each coding unit with coding block width log2CbW, coding block height log2CbH and location of top-left sample of the coding block ( xCb, yCb ), when edgeType is equal to EDGE_VER and xCb % 8 is equal 0 or when edgeType is equal to EDGE_HOR and yCb % 8 is equal to 0, the edges are filtered by the following ordered steps:

- The coding block width nCbW is set equal to 1  <<  log2CbW and the coding block height nCbH is set equal to 1  <<  log2CbH

- The variable filterEdgeFlag is derived as follows:

- If edgeType is equal to EDGE_VER and one or more of the following conditions are true, filterEdgeFlag is set equal to 0:

- The left boundary of the current coding block is the left boundary of the picture.

- The left boundary of the current coding block is the left boundary of the tile and loop_filter_across_tiles_enabled_flag is equal to 0.

- The left boundary of the current coding block is the left boundary of the tile group and tile_group_loop_filter_across_tile_groups_enabled_flag is equal to 0.

    - The left boundary of the current coding block is the right boundary of the intra refresh area and pps_intra_refresh_mode equal to 0 (NOTE: meaning line-based intra refresh) and tile_group_intra_refresh_direction is equal to 0 (NOTE: meaning from right to left)

    - The left boundary of the current coding block is the left boundary of the intra refresh area and pps_intra_refresh_mode equal to 0 (NOTE: meaning line-based intra refresh) and tile_group_intra_refresh_direction is equal to 1 (NOTE: meaning from left to right)

- Otherwise if edgeType is equal to EDGE_HOR and one or more of the following conditions are true, the variable filterEdgeFlag is set equal to 0:

- The top boundary of the current luma coding block is the top boundary of the picture.

- The top boundary of the current coding block is the top boundary of the tile and loop_filter_across_tiles_enabled_flag is equal to 0.

- The top boundary of the current coding block is the top boundary of the tile group and tile_group_loop_filter_across_tile_groups_enabled_flag is equal to 0.

    - The top boundary of the current coding block is the bottom boundary of the intra refresh area and pps_intra_refresh_mode equal to 1 (NOTE: meaning column-based intra refresh) and tile_group_intra_refresh_direction is equal to 0 (NOTE: meaning from top to bottom)

    - The top boundary of the current coding block is the top boundary of the intra refresh area and pps_intra_refresh_mode equal to 1 (NOTE: meaning line-based intra refresh) and tile_group_intra_refresh_direction is equal to 1 (NOTE: meaning from bottom to top)

- Otherwise, filterEdgeFlag is set equal to 1.