| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0389v1 |
| --- | --- |

| Title: | CE5-related: Minor optimizations for increasing the throughput of CE5.1.5 and CE5.1.6 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Heiner Kirchhoffer, Christian Bartnik, Paul Haase, Tobias Hinz, Stefan Matlage, Benno Stabernack, Jan Stegemann, Detlev Marpe, Heiko Schwarz, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-184 heiner.kirchhoffer@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

Some minor modifications are proposed to the state-based probability estimator of tests CE5.1.5 and CE5.1.6. In the coding interval subdivision and in the probability update of the state-based probability estimator, the order of some operations (abs, negation, right-shift) are exchanged. In this way, the throughput of a software implementation can be increased.

# Proposed modifications to the probability update

The update equations of the state-based probability estimator in CE5.1.5 and CE5.1.6 is according to the following pseudo code:

if( bin )

{

S0 += Next[ 16 + (S0 >> 3) ] >> sh0;

S1 += Next[ 16 + (S1 >> 7) ] >> sh1;

}

else

{

S0 -= Next[ 16 + (-S0 >> 3) ] >> sh0;

S1 -= Next[ 16 + (-S1 >> 7) ] >> sh1;

}

In CE5.1.5, sh0 and sh1 are constant values while they may change from context model to context model in CE5.1.6.

## First proposed modification

It is proposed to change the probability update to:

if( bin )

{

S0 += Next[ 16 + (S0 >> 3) ] >> sh0;

S1 += Next[ 16 + (S1 >> 7) ] >> sh1;

}

else

{

S0 -= Next[ 15 - (S0 >> 3) ] >> sh0;

S1 -= Next[ 15 - (S1 >> 7) ] >> sh1;

}

As can be seen in the else part, the right-shift by 7 (or 3) is done before the subtraction and the 16 is replaced with a 15. When sh0 and sh1 are fixed (like in CE5.1.5), the proposed modification can be implemented without if statement by involving a new lookup-table. This can be advantageous for a throughput-optimized software implementation.

## Second proposed modification

When sh0 and sh1 are not fixed (like in CE5.1.6), the following additional modification is proposed:

if( bin )

{

S0 += Next[ 16 + (S0 >> 3) ] >> sh0;

S1 += Next[ 16 + (S1 >> 7) ] >> sh1;

}

else

{

S0 += (-Next[ 15 - (S0 >> 3) ]) >> sh0;

S1 += (-Next[ 15 - (S1 >> 7) ]) >> sh1;

}

As can be seen in the else part, values from lookup table Next are first negated and then right-shifted by sh0 or sh1. In this way, the second proposed modification can be implemented without if statement by involving a new lookup table.

# Modification to the coding interval subdivision

The coding interval subdivision in CE5.1.5 and CE5.1.6 uses the following lookup table access:

rangeLPS=rangeTabLPSstateIdx range>>5&7

with

stateIdx=abs S0<<4+S1>>7

It is proposed to modify the calculation of stateIdx as follows:

stateIdx=abs S0<<4+S1>>7

The right-shift by 7 is applied before the abs calculation. This is advantageous for a throughput-optimized software implementation because the abs calculation can be integrated into a lookup table that stores the values of the rangeTabLPS.

## Experimental results

The first proposed modification of the probability update was applied to CE5.1.5 and the second proposed modification of the probability update was applied to CE5.1.6. The compression efficiency will be reported in an update of this document as soon as the simulations are finished.

The modification of the coding interval subdivision was applied to test CE5.1.6 using 10 and 14 bit for the state variables (git commit id 94fe3ea8). The compression efficiency is as follows (also see JVET-M0389_RLPS.xlsx):

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE5.1.6_94fe3ea |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,10% | 0,05% | 0,09% | 106% | 103% |
| Class A2 | 0,08% | 0,06% | 0,07% | 104% | 101% |
| Class B | 0,07% | 0,12% | 0,03% | 104% | 104% |
| Class C | 0,06% | 0,08% | 0,07% | 103% | 100% |
| Class E | 0,06% | 0,05% | 0,08% | 104% | 103% |
| Overall | 0,07% | 0,08% | 0,06% | 104% | 102% |
| Class D | 0,04% | 0,06% | 0,15% | 102% | 100% |
| Class F (optional) | 0,11% | 0,12% | 0,04% | 103% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over CE5.1.6_94fe3ea |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,09% | -0,01% | -0,02% | 108% | 102% |
| Class A2 | 0,06% | -0,03% | 0,01% | 106% | 100% |
| Class B | 0,07% | 0,05% | 0,05% | 106% | 106% |
| Class C | 0,10% | 0,13% | 0,14% | 104% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0,08% | 0,04% | 0,05% | 106% | 104% |
| Class D | 0,08% | 0,09% | 0,04% | 104% | 102% |
| Class F (optional) | 0,09% | 0,06% | 0,15% | 105% | 110% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE5.1.6_94fe3ea |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,06% | 0,14% | 0,16% | 108% | 104% |
| Class C | 0,09% | 0,21% | 0,23% | 106% | 107% |
| Class E | -0,12% | 0,66% | -0,03% | 111% | 103% |
| Overall | 0,02% | 0,29% | 0,14% | 108% | 105% |
| Class D | 0,14% | -0,59% | 0,54% | 105% | 104% |
| Class F (optional) | -0,05% | 0,04% | 0,34% | 107% | 105% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over CE5.1.6_94fe3ea |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,09% | 0,09% | -0,23% | 109% | 104% |
| Class C | 0,09% | 0,22% | -0,03% | 107% | 107% |
| Class E | -0,05% | 0,61% | -0,29% | 116% | 108% |
| Overall | 0,05% | 0,27% | -0,18% | 110% | 106% |
| Class D | 0,14% | 0,39% | -0,08% | 106% | 104% |
| Class F (optional) | 0,05% | 0,16% | 0,50% | 110% | 109% |

The implementation is not optimized on throughput. Instead, a throughput analysis will be reported in a future update of this document based the testbed of subtest CE5.2.

Note that the runtimes are not accurate due to varying simulation conditions (CPU type, cluster load, etc.).

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).