| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0390 |
| --- | --- |

| Title: | CE-10 related multi-hypothesis with uni-directional inter prediction restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Poirier E. François K. Naser | Tel: Email: | +33(0)299273198 tangi.poirier@technicolor.com edouard.francois@technicolor.com karam.naser@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution describes a simplification of multi-hypothesis intra-inter prediction by restricting inter-prediction to be uni-directional. Another simplification removes the specific syntax for narrow CUs. The proposed method has been implemented on top of VTM-3.0. The first modification is reported to result in PSNR-Y, U, V BD-rate variations of 0.10%, 0.01%, 0.02% with 98% and 103% encoding and decoding times in RA configuration, 0.06%, 0.16%, 0.20% with 104% and 106% encoding and decoding times in LDB configuration. The second one removes the intra direction restriction for narrow CUs on top of the first restriction. The second modification on top of the first is reported to result in PSNR-Y, U, V BD-rate variations of 0.09%, -0.01%, 0.03% with 94% and 97% encoding and decoding times in RA configuration, 0.06%, 0.07%, 0.07% with 103% and 105% encoding and decoding times in LDB configuration.

# Introduction

In VTM-3.0, multi-hypothesis prediction mode combines inter bi-prediction with intra prediction. 
This contribution proposes to restrict inter prediction to be uni-directional. The uni-directional motion vector predictors derivation from triangle partitions is reused for intra-inter mode.

# Description

In VTM-3.0, multi-hypothesis prediction mode combines inter bi-prediction with intra prediction, samples are clipped after intra and inter prediction individually and after the combination of intra and inter prediction, as shown in Figure 1.

Figure 1: Multi-hypothesis prediction pipeline in VTM-3.0

In the proposed method, inter prediction in kept with high bit-depth precision, intra prediction bitdepth is increased to be the same as inter prediction, the sample clipping is done after the combination of intra and inter prediction, as shown in Figure 2. The uni-directional motion vector predictors derivation from triangle partitions is reused for intra-inter mode.

Figure 2: Proposed multi-hypothesis pipeline

In a second method, intra mode restriction for narrow CU’s is removed on top of the first method.

# Simulation Results

The simulations are conducted using the VTM configuration as described in JVET-L1010 [1]. VTM-3.0 reference software is used with VTM configuration files.

## Uni-directional inter prediction restriction

Table 1: simulation results of the proposed uni-directional restriction of multi-hypothesis in random access

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | -0.13% | 0.02% | 99% | 105% |
| Class A2 | 0.12% | 0.13% | 0.19% | 100% | 106% |
| Class B | 0.11% | -0.06% | -0.12% | 97% | 104% |
| Class C | 0.07% | 0.12% | 0.08% | 97% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | 0.01% | 0.02% | 98% | 103% |
| Class D | 0.04% | -0.23% | 0.05% | 96% | 101% |
| Class F | 0.02% | 0.03% | 0.06% | 97% | 103% |

Table 2: simulation results of the proposed uni-directional restriction of multi-hypothesis in low delay

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.09% | -0.03% | 0.07% | 105% | 102% |
| Class C | 0.04% | 0.39% | 0.43% | 103% | 108% |
| Class E | 0.04% | 0.19% | 0.12% | 102% | 110% |
| Overall | 0.06% | 0.16% | 0.20% | 104% | 106% |
| Class D | -0.01% | 0.20% | -0.16% | 100% | 103% |
| Class F | 0.20% | 0.13% | 0.23% | 103% | 106% |

## Uni-directional inter prediction restriction and narrow CU cases removal

Table 3: simulation results of the intra mode restriction removal on top of the first method in random access

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | -0.08% | 0.08% | 98% | 102% |
| Class A2 | 0.10% | 0.00% | 0.16% | 97% | 100% |
| Class B | 0.11% | -0.06% | -0.13% | 92% | 97% |
| Class C | 0.04% | 0.09% | 0.09% | 90% | 91% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | -0.01% | 0.03% | 94% | 97% |
| Class D | 0.01% | -0.05% | -0.06% | 90% | 90% |
| Class F | -0.01% | -0.03% | 0.01% | 90% | 93% |

Table 4: simulation results of the intra mode restriction removal on top of the first method in low delay

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.11% | -0.09% | 0.17% | 104% | 100% |
| Class C | -0.01% | 0.20% | 0.17% | 103% | 110% |
| Class E | 0.05% | 0.15% | -0.22% | 102% | 109% |
| Overall | 0.06% | 0.07% | 0.07% | 103% | 105% |
| Class D | -0.02% | -0.66% | 0.27% | 101% | 103% |
| Class F | 0.04% | 0.13% | 0.33% | 103% | 105% |

# Conclusion

This contribution proposes two simplifications of the multi-hypothesis prediction. The first one restricts inter prediction to be uni-directional. The first modification is reported to result in PSNR-Y, U, V BD-rate variations of 0.10%, 0.01%, 0.02% with 98% and 103% encoding and decoding times in RA configuration, 0.06%, 0.16%, 0.20% with 104% and 106% encoding and decoding times in LDB configuration. The second one removes the intra direction restriction for narrow CUs on top of the first restriction. The second modification on top of the first is reported to result in PSNR-Y, U, V BD-rate variations of 0.09%, -0.01%, 0.03% with 94% and 97% encoding and decoding times in RA configuration, 0.06%, 0.07%, 0.07% with 103% and 105% encoding and decoding times in LDB configuration.

# References

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).