| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0392-v1 |
| --- | --- |

| Title: | Non-CE3: Extended Mode-Dependent Intra Smoothing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Jianle Chen | Email: | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co. |  |  |

_____________________________

# Abstract

This document proposes to introduce reference sample filtering for angular modes that currently use 4-tap interpolation filter to obtain predicted samples. It is proposed to introduce block size dependent sampling into reference sampling filter, and to replace Gaussian filter with a 2-tap filter that is currently used in chroma interpolation filtering. The following overall results are reported for Test 1 (the one with 4-tap Gaussian filter): -0.09% (Y), -0.07% (U), -0.03% for AI and -0.07% (Y), -0.10% (U), -0.04% for RA; and for Test 2 (the one with 2-tap linear filter): ): -0.04% (Y), -0.02% (U), -0.02% for AI and -0.04% (Y), -0.02% (U), 0.06% for RA. Noticeable coding performance is observed for Class A1 in both tests. The running time is negligibly changed at both encoder and decoder sides for AI and RA configurations as compared with VTM-3.0.

# Introduction

In the JVET-L meeting, 4-tap interpolation filters for intra prediction were adopted into the VTM software and the VVC spec draft as proposed in [1]. Reference sample filter are not applied when Gaussian interpolation filter is used.

# Proposal

In this contribution, we propose to introduce reference sample filter with coefficient [1, 0, 2, 0, 1] when a sum of a block width and height is greater than 32. This could be done by increasing sampling step to 2 when performing reference sample filtering.

In addition, it is proposed to apply Gaussian filter to the results of [1, 2, 1] filter, that is conditionally applied in accordance with MDIS conditions (as it was performed prior to adoption of [1]), but Gaussian filter was deconvolved with [1, 2, 1] filter and, hence, is defined differently (see Table 1).

Table 1. Alternative Gaussian filter used in test 1

| offset | f[0] | f[1] | f[2] | f[3] |
| --- | --- | --- | --- | --- |
| 0 | 12 | 40 | 12 | 0 |
| 1 | 11 | 40 | 13 | 0 |
| 2 | 10 | 40 | 14 | 0 |
| 3 | 9 | 40 | 15 | 0 |
| 4 | 8 | 40 | 16 | 0 |
| 5 | 7 | 40 | 17 | 0 |
| 6 | 7 | 39 | 17 | 1 |
| 7 | 7 | 38 | 18 | 1 |
| 8 | 6 | 37 | 20 | 1 |
| 9 | 5 | 37 | 21 | 1 |
| 10 | 4 | 36 | 23 | 1 |
| 11 | 4 | 34 | 24 | 2 |
| 12 | 4 | 34 | 24 | 2 |
| 13 | 4 | 33 | 25 | 2 |
| 14 | 3 | 32 | 27 | 2 |
| 15 | 3 | 31 | 28 | 2 |
| 16 | 3 | 29 | 29 | 3 |
| 17 | 2 | 28 | 31 | 3 |
| 18 | 2 | 27 | 32 | 3 |
| 19 | 2 | 25 | 33 | 4 |
| 20 | 2 | 24 | 34 | 4 |
| 21 | 2 | 24 | 34 | 4 |
| 22 | 1 | 23 | 36 | 4 |
| 23 | 1 | 21 | 37 | 5 |
| 24 | 1 | 20 | 37 | 6 |
| 25 | 1 | 18 | 38 | 7 |
| 26 | 1 | 17 | 39 | 7 |
| 27 | 0 | 17 | 40 | 7 |
| 28 | 0 | 16 | 40 | 8 |
| 29 | 0 | 15 | 40 | 9 |
| 30 | 0 | 14 | 40 | 10 |
| 31 | 0 | 13 | 40 | 11 |

In Test 2, it is proposed to replace Gaussian filter with 2-tap linear filter (the same used in chroma interpolation process). These 2 configurations are summarized in Table 2.

Table 2. Summary of 2 proposed test

| Test # | Description |
| --- | --- |
| 1 | When MDIS condition are true, apply [1 0 2 0 1] when a sum of a block width and height is greater than 32 and [1 2 1] otherwise. Apply Table 1 Gaussian filter for the filtered reference samples |
| 2 | Test 1 with linear filter (same as for intra chroma interpolation) used instead of Gaussian filter |

# Experiment Results

The results for tests listed in Table 2 obtained using the common test conditions [2] are reportedly presented in Table 3 and Table 4 for AI and RA configurations.

Table 3. Experimental results for Test 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.38% | -0.34% | -0.25% | 100% | 100% |
| Class A2 | -0.07% | 0.03% | 0.00% | 99% | 99% |
| Class B | -0.07% | -0.02% | 0.01% | 99% | 99% |
| Class C | 0.00% | -0.02% | -0.10% | 100% | 100% |
| Class E | 0.06% | -0.07% | 0.16% | 99% | 99% |
| Overall | -0.09% | -0.07% | -0.03% | 99% | 99% |
| Class D | 0.02% | -0.05% | 0.11% | 100% | 100% |
| Class F | 0.04% | 0.00% | 0.06% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.34% | -0.23% | 99% | 99% |
| Class A2 | -0.03% | 0.06% | 0.20% | 99% | 99% |
| Class B | -0.06% | -0.04% | -0.11% | 99% | 100% |
| Class C | 0.00% | -0.11% | 0.00% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.10% | -0.04% | 99% | 100% |
| Class D | -0.04% | -0.24% | -0.18% | 100% | 100% |
| Class F | -0.03% | -0.07% | -0.10% | 99% | 100% |

Table 4. Experimental results for Test 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | -0.21% | -0.09% | 100% | 100% |
| Class A2 | -0.02% | 0.08% | 0.05% | 99% | 100% |
| Class B | -0.02% | -0.01% | 0.02% | 100% | 100% |
| Class C | 0.01% | 0.01% | -0.14% | 100% | 100% |
| Class E | 0.07% | 0.03% | 0.08% | 100% | 99% |
| Overall | -0.04% | -0.02% | -0.02% | 100% | 100% |
| Class D | 0.03% | 0.02% | -0.09% | 100% | 100% |
| Class F | 0.01% | 0.11% | 0.11% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.20% | -0.08% | 99% | 99% |
| Class A2 | -0.02% | 0.18% | 0.09% | 99% | 99% |
| Class B | -0.04% | 0.04% | -0.06% | 99% | 99% |
| Class C | 0.00% | 0.04% | 0.29% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.02% | 0.06% | 99% | 99% |
| Class D | -0.01% | -0.24% | 0.04% | 99% | 100% |
| Class F | -0.01% | 0.04% | -0.04% | 99% | 99% |

# Conclusion

We propose to study this tool in CE3.

# References

- A. Filippov, V. Rufitskiy, J. Chen, G. Van der Auwera, A.K. Ramasubramonian, V. Seregin, T. Hsieh, M. Karczewicz, “CE3: A combination of tests 3.1.2 and 3.1.4 for intra reference sample interpolation filter”, JVET-L0628, 12th JVET Meeting, Macao, China, October 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, 12th JVET Meeting, Macao, China, October 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).