| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0395v1 |
| --- | --- |

| Title: | CE5-related: Alternative implementation of CABAC range sub-interval derivation for CE5.1.5, CE5.1.6 and CE5.1.7 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Paul Haase, Heiner Kirchhoffer, Stefan Matlage, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-409 paul.haase@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes an alternative implementation of the CABAC range sub-interval derivation, for the state-based probability estimator as proposed in JVET-K0430 and further investigated in CE5. Similar to JVET-L0426, the proposed method employs a combination of the range-probability product computation and a table lookup. Additionally, a simple shift operation is applied, which halves the lookup table size. Thus, a (1-dimensional) table lookup, a multiplication and a shift are required. If a computation-based implementation is not desired, a 32x8 lookup table can be used instead.

# Introduction

CABAC range sub-interval derivation in [1] is performed by a lookup in a 2-dimensional table, which outputs the coding interval associated with the LPS (Least Probable Symbol). The tests in CE5 [2] employ lookup tables of size 32x8, which need to be stored. In this proposal, these lookup tables are replaced by a combination of a smaller lookup table of size 16, a multiplication and a bit shift. Alternatively, this approach can be implemented as a 32x8 lookup table, which may be reasonable regarding software implementation aspects. The proposed method does not affect the coding performance.

# Proposed method

The interval subdivision is realized using a one dimensional lookup table probTabLPS of size 16, a multiplication operation and a bit shift, as presented here.

rangeLPS=probTabLPStabIdx∙range>>5>>sh1

stateIdx=abs S0<<4+S1>>7

tabIdx= stateIdx & 15

sh1= stateIdx & 16  ? 6 :3

For the CE tests the resulting rangeLPS values are stored in a 32x8 lookup table, for simplicity. The parameters applied in the CE tests [2] are presented in the following sections.

## Applied parameters for CE5.1.5

The test examines the state-based probability estimator with two states (8 and 12 Bit), a logarithmic state representation and fixed windows sizes.

probTabLPS= 125, 110, 95, 82, 72, 62, 54, 47, 41, 36, 32, 28, 24, 22, 19, 17

In a subtest, using the same configuration as CE5.1.5, but with the two states set to 10 and 14 bit, the following parameters are applied:

probTabLPS= 125, 108, 94, 81, 71, 61, 53, 46, 40, 35, 31, 27, 24, 21, 18, 16

stateIdx=abs S0<<4+S1>>9

## Applied parameters for CE5.1.6

The test examines the state-based probability estimator with two states (8 and 12 Bit), a logarithmic state representation and custom windows sizes (4 Bit).

probTabLPS= 125, 110, 96, 83, 72, 63, 55, 48, 42, 37, 32, 29, 25, 22, 19, 17

In a subtest, using the same configuration as CE5.1.6, but with the two states set to 10 and 14 bit, the following parameters are applied:

probTabLPS= 125, 108, 94, 82, 71, 62, 53, 46, 40, 35, 31, 27, 24, 21, 17, 16,

stateIdx=abs S0<<4+S1>>9

## Applied parameters for CE5.1.7

The test examines the state-based probability estimator with one state (12 Bit), a logarithmic state representation and custom windows sizes.

probTabLPS= 125, 109, 94, 82, 71, 62, 54, 47, 41, 36, 32, 28, 24, 22, 19, 16

# Experimental results

As the proposed method is only an alternative way of implementing the coding interval subdivision for tests CE5.1.5, CE5.1.6, and CE5.1.7, the compression efficiency is the same.

# References

- H. Kirchhoffer, J. Stegemann, D. Marpe, H. Schwarz, T. Wiegand “CE5-related: State-based probability estimator“, JVET-K0430, 11-th Meeting, Ljubljana, SI, July 2018.

- H. Kirchhoffer, A. Said, “Description of Core Experiment 5 (CE5): Arithmetic Coding Engine”, JVET-L1025, 12-th Meeting, Macao, CN, October 2018.

- P. Haase, H. Kirchhoffer, S. Matlage, H. Schwarz, D. Marpe, T. Wiegand, “CE5-related: Alternative implementation of CABAC range sub-interval derivation for test CE 5.1.4“, JVET-L0426, 12-th Meeting, Macao, CN, October 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).