| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-L0398-v1 |
| --- | --- |

| Title: | CE6-related Further simplification of CE6-1.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Pierrick Philippe | Email: | pierrick.philippe@orange.com |
| Source: | b<>com, Orange |  |  |

_____________________________

# Abstract

This contribution studies the impact of retaining only the 16 lower coefficients for MTS when operated in size 32. The results indicate that additional simplification is revealed with respect to the CE6-1.5 and the coding performance impact is measured.

# Outline of the proposal

In CE6-1.5 the DST7 and DCT8 are approximated with an adjustment stage followed by the DCT2. Also, when CE6-1.5 makes usage of size 64, a zero-out of the higher frequency coefficients is performed to limit the complexity impact.

In this contribution, the impact of zeroing out the higher frequency range when size-32 is operated is considered. 16 coefficients are retained in this proposal.

In CE61.5 [2], the DCT8/DST7 transforms are approximated by the combination of a block band matrix and a DCT2 transform, which can be implemented with the partial butterfly algorithm (as for HEVC).

The size M forward DST7 (S7) is approximated as follows:

S7=S⋅C2t⋅F⋅A

- S is a sign matrix, with elements Sk,n=0 if n!=k-1notherwise

- F is a flip matrix, with elements Fk,n=1 if n=M-1-k0 otherwise

- C2t is the transpose of the DCT2 matrix.

- A is a sparse adjustment matrix as specified below.

The size M forward DCT8 (C8) writes:

C8=C2t⋅F⋅A⋅F

Consequently, the inverse DST7 and DCT8 write the following:

iS7=At⋅F⋅C2⋅S and iC8=F⋅At⋅F⋅C2

The flip (F) and sign (S) matrices do not involve any arithmetic operation. Consequently, the complexity of TAF is the sum of the complexity of the adjustment matrix and the DCT2.

When the higher frequency range is zeroed out, the inverse DST7 and DCT8, when they are implemented using adjustment stages, require a partial DCT2, leading to the removal of one butterfly stage. Consequently, some complexity savings are induced, this contribution reports the coding impact of such modification.

# Results

The experimental results are presented under the CTC and under the CTC while MTS is enabled for Inter Coding as well. The reference for the later experiment is VTM3 operating with MTS on for Inter Coding as well.

TAF is experimented for size 32 and 64, the following configurations are tested, in compliance with the CE6 test plan.

The 16 higher frequency coefficients of MTS-32 are set to zero, as the 32 higher frequency coefficients of MTS-64.

## Under the common test conditions

### CE6-1.5a: TAF applied to size 32 with zero-out

### CE6-1.5b: TAF applied to sizes 32 and 64

## With MTS enabled for Inter coding

### CE6-1.5a: TAF applied to size 32

### CE6-1.5b: TAF applied to sizes 32 and 64

# Complexity metrics

For the understanding of complexity, the following information is provided according to CE6.

As zero-out is applied for the transforms, the first butterfly stage of the DCT2 is avoided leading to respectively 32 and 64 less additions compared to a full transform. It also potentially reduces the internal dynamic of the transform.

# Conclusion

It is demonstrated that the removal of the higher frequency coefficients has a minor impact on the coding performance, as the loss in AI is in the range of 0.1 to 0.2%. For other coding scenarios, less impact is measured and some gains are revealed, especially when MTS is on for Inter coding modes.

Consequently, it is proposed to use zero-out when the transform adjustment filtering is applied for size 32 and 64.

# References

- JVET-L0026, Description of Core Experiment 6 (CE6): Transforms and transform signalling

- JVET-M0070 CE6: MTS simplification with TAF (tests 1.5a-d). P. Philippe.

# Patent rights declaration(s)

b<>com, Orange may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).