| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0419 |
| --- | --- |

| Title: | CE8-related: Context modeling on palette mode flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun, Jian Lou | Tel: Email: | +1-858-349-2818 yuchen.s@alibaba-inc.com |
| Source: | Alibaba Cloud Computing Ltd. |  |  |

_____________________________

- Abstract

This document modifies context modeling for palette flag signaling of CE8.2.1. Compared with CE8.2.1, the results of the proposed method show that, under AI configurations, the proposed method provides 0.1% and 0.1% BD-rate gains for luma for 4:2:0 TGM when the method is tested with CPR turned off and on, respectively.

Introduction

In CE8.2.1, the CU palette flag is context coded. There is only one context model for the palette flag. We found that the statistic of palette mode is different between luma CUs and chroma CUs when the separated coding tree is applied. Therefore, this document proposes to add a new context model for the palette flag of the chroma CUs

Description of the tests

In this proposal, we propose to add a new context model for the palette flag of the chroma CUs. The context modeling of CE8.2.1 and this proposal is summarized as the following table:

|  | Separated Coding Tree |  | Shared Coding Tree |
| --- | --- | --- | --- |
|  | Luma CUs | Chroma CUs | Joint CUs |
| CE8.2.1 | ctxIdx = 0 | ctxIdx = 0 | ctxIdx = 0 |
| This proposal | ctxIdx = 0 | ctxIdx = 1 | ctxIdx = 0 |

Simulation results

The proposed method is evaluated according to the CE8 description [1] for the intra-only (AI) and random-access (RA) configurations with the VTM-3.0 software.

The combination is tested with/without CPR. Table 1 shows the results with CPR turned off in the setting. The results are compared with VTM3.0 and CE8.2.1.

Table 1: Results of the CE8.2.2+CE8.2.5 test (CPR is turned off)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.04% | 0.06% | 109% | 100% | 0.00% | -0.01% | -0.02% | 101% | 102% |
| Class A2 | 0.09% | 0.13% | 0.08% | 109% | 104% | 0.00% | 0.00% | 0.00% | 102% | 104% |
| Class B | 0.09% | 0.11% | 0.09% | 107% | 104% | 0.00% | 0.02% | 0.02% | 101% | 104% |
| Class C | 0.10% | 0.13% | -0.04% | 109% | 113% | 0.01% | 0.04% | 0.04% | 101% | 106% |
| Class E | 0.14% | 0.23% | 0.19% | 111% | 109% | 0.02% | 0.06% | -0.01% | 104% | 108% |
| Overall | 0.09% | 0.12% | 0.07% | 109% | 106% | 0.01% | 0.02% | 0.01% | 102% | 105% |
| Class D | 0.13% | 0.11% | 0.03% | 109% | 111% | 0.02% | 0.04% | 0.01% | 102% | 107% |
| Class F | -11.46% | -8.89% | -8.99% | 121% | 97% | -0.04% | 0.05% | 0.05% | 104% | 109% |
| Class SCC 1080p | -33.13% | -27.63% | -27.73% | 135% | 70% | -0.11% | -0.06% | -0.06% | 104% | 109% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.18% | 0.13% | 104% | 101% | 0.00% | 0.01% | 0.01% | 100% | 101% |
| Class A2 | 0.13% | 0.21% | 0.19% | 107% | 102% | 0.02% | -0.02% | -0.04% | 102% | 102% |
| Class B | 0.13% | 0.27% | -0.09% | 108% | 102% | 0.00% | -0.01% | 0.02% | 102% | 101% |
| Class C | 0.18% | 0.14% | 0.27% | 111% | 112% | 0.01% | 0.04% | 0.06% | 102% | 105% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.14% | 0.20% | 0.11% | 108% | 104% | 0.00% | 0.00% | 0.02% | 102% | 102% |
| Class D | 0.22% | 0.02% | 0.33% | 111% | 109% | 0.02% | 0.18% | 0.08% | 102% | 105% |
| Class F | -8.56% | -8.01% | -8.34% | 112% | 105% | -0.05% | 0.01% | -0.04% | 102% | 104% |
| Class SCC 1080p | -15.76% | -14.68% | -14.53% | 108% | 96% | -0.07% | -0.01% | -0.02% | 102% | 103% |

Table 2 shows the results with CPR turned on in the setting. The results of CE8.2.2 are compared with VTM3.0 and CE8.2.1.

Table 2: Results of the CE8.2.2+CE8.2.5 test (CPR is turned on)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.15% | 0.05% | 107% | 102% | 0.00% | -0.02% | -0.03% | 97% | 97% |
| Class A2 | 0.09% | 0.07% | 0.12% | 105% | 101% | 0.00% | 0.00% | 0.01% | 96% | 96% |
| Class B | 0.10% | 0.08% | 0.05% | 105% | 103% | 0.00% | 0.00% | -0.02% | 100% | 101% |
| Class C | 0.09% | 0.15% | 0.08% | 104% | 102% | 0.01% | 0.03% | 0.00% | 98% | 101% |
| Class E | 0.15% | 0.26% | 0.02% | 104% | 100% | 0.01% | -0.03% | 0.00% | 98% | 100% |
| Overall | 0.09% | 0.14% | 0.06% | 105% | 102% | 0.01% | 0.00% | -0.01% | 98% | 99% |
| Class D | 0.11% | 0.23% | -0.01% | 108% | 105% | 0.02% | 0.04% | 0.04% | 100% | 102% |
| Class F | -4.46% | -2.43% | -2.55% | 108% | 96% | 0.00% | 0.03% | -0.03% | 97% | 101% |
| Class SCC 1080p | -12.72% | -7.93% | -7.85% | 112% | 83% | -0.07% | 0.00% | 0.00% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.24% | 0.14% | 106% | 102% | 0.01% | 0.00% | 0.01% | 98% | 98% |
| Class A2 | 0.11% | 0.10% | 0.06% | 107% | 104% | 0.01% | 0.02% | 0.02% | 98% | 98% |
| Class B | 0.11% | 0.04% | 0.13% | 107% | 101% | 0.01% | 0.03% | 0.01% | 99% | 98% |
| Class C | 0.15% | 0.01% | 0.27% | 110% | 105% | -0.01% | -0.04% | -0.03% | 100% | 103% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.11% | 0.08% | 0.16% | 107% | 103% | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class D | 0.27% | 0.25% | -0.20% | 110% | 106% | 0.01% | 0.01% | 0.00% | 101% | 102% |
| Class F | -2.61% | -2.18% | -2.61% | 109% | 103% | 0.00% | 0.17% | -0.01% | 100% | 102% |
| Class SCC 1080p | -4.74% | -3.68% | -3.48% | 103% | 96% | -0.03% | -0.05% | -0.03% | 99% | 100% |

Reference

- X. Xu, Y.-C. Chao, Y.-C. Sun, J. Xu, "Description of Core Experiment 8: Screen Content Coding Tools”, JVET-L1028, 2018.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).