| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0432 |
| --- | --- |

| Title: | CE2-related: Combination of CE2.2.3.d and affine inheritance from motion data line buffer |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, Xiaozhong Xu, Xiang Li, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. Jie Zhao Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, U.S.A | Tel: Email: Tel: Email: | +1-650-798-3300 guichunli@tencent.com 1-201-816-2000 jie.zhao@lge.com |
| Source: | Tencent & LG Electronics |  |  |

_____________________________

# Abstract

This document reports test results of combining CE2.2.3.d (replacing original affine model inheritance with inheritance from affine HMVP) and keeping original affine model inheritance from motion data line buffer. It was observed that the BD-rate performance compared with VTM-3.0 is 0.04% and 0.05% on RA and LDB, respectively. Compared to CE2.2.3.d, the BD-rate performance is -0.03% and -0.04% on RA and LDB, respectively.

# Introduction

In core experiment test CE2.2.3.d, it was proposed to replace existing inherited affine candidates with candidates inherited from affine HMVP. The method reportedly reduced affine related memory storage significantly. Due to the fact that the affine HMVP buffer is reset at the beginning of each CTU row, the affine motion information from blocks above current CTU will not be used in affine HMVP.

# Proposed

To further improve the coding performance without adding local buffer requirements, a combined method is proposed to keep affine inheritance from affine HMVP as in CE2.2.3.d, and the original affine inheritance from motion data line buffer when neighboring candidate is above the current CTU boundary.

In current VTM-3.0, when the current block is at the top of a CTU, and the neighboring block for inheritance is above the CTU boundary, affine model inheritance will load the corresponding regular MV in motion data line buffer and use them as the bottom-left and bottom-right control points to derive control point motion vectors (CPMVs) for current block using 4-parameter affine model. Keeping this model inheritance method does not require additional local buffer for storing CPMVs. The existing line buffer can be utilized.

The proposed combination can keep the benefit of low memory requirement for affine model inheritance from affine HMVP, while utilizing the available affine motion models from above CTU. When the current block is located at the top of a CTU, the number of candidate inherited from affine HMVP is limited to 1.

# Experimental Results

The proposed combined method is implemented into VTM-3.0, and tested following common test condition [5]. In the code, affine HMVP table size is defined as 5. VTM-3.0 and CE2.2.3.d were used as anchors.

Table 3.1 Result compared with VTM-3.0

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.03% | -0.03% | 98% | 99% |  |  |  |  |  |
| Class A2 | 0.03% | 0.14% | 0.13% | 100% | 100% |  |  |  |  |  |
| Class B | 0.04% | 0.09% | 0.12% | 99% | 99% | 0.11% | -0.03% | 0.06% | 99% | 98% |
| Class C | 0.02% | 0.08% | 0.09% | 98% | 97% | 0.09% | 0.25% | 0.05% | 101% | 97% |
| Class E |  |  |  |  |  | -0.03% | 0.24% | -0.62% | 99% | 99% |
| Overall | 0.03% | 0.09% | 0.08% | 99% | 99% | 0.07% | 0.13% | -0.11% | 100% | 98% |
| Class D | 0.12% | 0.12% | 0.06% | 98% | 95% | 0.20% | 0.15% | 0.77% | 100% | 102% |
| Class F | 0.02% | 0.02% | -0.06% | 98% | 98% | 0.03% | -0.20% | 0.32% | 100% | 98% |

Table 3.2 Result compared with CE2.2.3.d

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.05% | -0.05% | 98% | 98% |  |  |  |  |  |
| Class A2 | -0.10% | 0.04% | -0.04% | 99% | 99% |  |  |  |  |  |
| Class B | -0.03% | 0.00% | 0.10% | 98% | 99% | -0.04% | 0.03% | 0.15% | 99% | 98% |
| Class C | -0.03% | -0.03% | 0.05% | 98% | 98% | -0.03% | 0.03% | -0.12% | 100% | 94% |
| Class E |  |  |  |  |  | 0.02% | 0.07% | -0.61% | 98% | 98% |
| Overall | -0.04% | -0.01% | 0.03% | 99% | 98% | -0.02% | 0.04% | -0.13% | 99% | 97% |
| Class D | 0.01% | 0.06% | -0.08% | 99% | 97% | -0.08% | 0.18% | 0.36% | 101% | 101% |
| Class F | 0.00% | -0.03% | -0.05% | 99% | 98% | -0.07% | -0.31% | 0.31% | 100% | 97% |

# Conclusion

The proposed combination of affine model inheritance from affine HMVP and motion data line buffer keeps the low memory requirement while improves coding performance on top of CE2.2.3.d. Compared to CE2.2.3.d, the BD-rate performance is -0.03% and -0.04% on RA and LDB, respectively.

It is proposed to adopt the proposed method into VVC and VTM.

# Reference

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3),” JVT-L1002, Macao, CN, Oct 2018.

- J. Zhao, S. Paluri, S. Kim, G. Li, X. Xu, X. Li, S. Liu, “CE4-related: History Based Affine Merge Candidate,” JVET-L0305, Macao, CN, Oct 2018.

- M. Zhou, B. Heng, “CE4-related: A clean up for affine mode,” JVET-L0047, Macao, CN, Oct 2018.

- Y. He, C.-Y. Chen, C.-C Chen, “Description of Core Experiment 2 (CE2): Sub-block based motion prediction,” JVT-L1024, Macao, CN, Oct 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-L1010 Macao, CN, Oct 2018.

- J. Zhao, S. Kim, G. Li, X. Xu, X. Li, S. Liu, “CE2: History Based Affine Motion Candidate (Test 2.2.3)”, Document of Joint Video Experts Team (JVET), JVET-M0125, Marrakech, MA, Jan 2019.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).