| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0435 |
| --- | --- |

| Title: | CE4-related: MMVD offset table signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, Xiaozhong Xu, Xiang Li, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 guichunli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

An MMVD offset table signaling method is proposed based on the MMVD offset table reordering method in CE4.4.5, to resolve the inter picture dependency and error resilience issue. The proposed method was implemented on top of CE4.4.5 software with signaling in slice header. It is reported 0.05% （RA） and 0.14% （LDB）luma BD-rate increase over CE4.4.5.c. Compared to VTM-3, 0.19% luma BD-rate reduction and 0.01% BD-rate increase are reported for RA and LDB, respectively.

# Introduction

MMVD was adopted in VVC and was included in VTM-3.0. In core experiment CE4.4.5, a method is proposed to reorder the MMVD offset table based on the occurrence of the MMVD offsets used in the reference picture. CE4.4.5 provided additional BD-rate gain on top of current MMVD in VTM-3.0, but it has inter-picture dependency which may lead toerror resilience issue. For example, if a slice is lost for a multi-slice reference picture, the resulting visual artefact should be constrained inside the lost slice. However, with the method in CE4.4.5 the visual artefact may spread to the whole picture as the occurrence statistics at decoder is different from that at encoder due to the loss of one slice.

# Proposed

It is proposed to signal the MMVD table in slice header instead of generating it on the decoder side in order to resolve the inter picture dependency and error resilience issue.

Since there are 8 entries in the distance offset table, each table entry can be represented by 3 bits as shown in Table 1. Combining the binarized entry in the defined order would result in a 24-bit order code.

Table 1 - Binary representation of each offset table entry

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel Distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel |
| Binary Value | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |

A default order code is defined with the default order of the original MMVD offset table. And only the delta between the default order code and the order code of the current picture needs to be signaled in slice header.

# Experimental Results

The proposed method is implemented based on the software of CE4.4.5. The simulations were conducted following JVET common test conditions [2]. CE4.4.5.c and VTM-3.0 were used as anchors.

Table 3.1 - Simulation Result compared with CE4.4.5.c

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.01% | 0.01% | 99% | 99% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 98% |  |  |  |  |  |
| Class B | 0.05% | 0.05% | 0.05% | 100% | 100% | 0.05% | 0.04% | 0.04% | 97% | 95% |
| Class C | 0.12% | 0.11% | 0.11% | 100% | 99% | 0.11% | 0.10% | 0.10% | 101% | 99% |
| Class E |  |  |  |  |  | 0.32% | 0.26% | 0.27% | 99% | 96% |
| Overall | 0.05% | 0.05% | 0.05% | 100% | 99% | 0.14% | 0.11% | 0.12% | 99% | 97% |
| Class D | 0.25% | 0.24% | 0.24% | 100% | 99% | 0.23% | 0.20% | 0.20% | 100% | 101% |
| Class F | 0.09% | 0.08% | 0.08% | 100% | 100% | 0.18% | 0.16% | 0.16% | 98% | 97% |

Table 3.2 - Simulation Result compared with VTM-3.0

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.44% | -0.62% | -0.71% | 97% | 100% |  |  |  |  |  |
| Class A2 | -0.28% | -0.81% | -0.69% | 97% | 99% |  |  |  |  |  |
| Class B | -0.12% | -0.18% | -0.35% | 100% | 99% | -0.08% | -0.47% | -0.04% | 102% | 96% |
| Class C | -0.03% | -0.12% | -0.06% | 103% | 98% | -0.06% | 0.25% | 0.08% | 105% | 99% |
| Class E |  |  |  |  |  | 0.26% | 0.51% | 0.24% | 102% | 97% |
| Overall | -0.19% | -0.38% | -0.41% | 100% | 99% | 0.01% | 0.01% | 0.07% | 103% | 97% |
| Class D | 0.06% | -0.21% | -0.14% | 104% | 99% | 0.06% | -0.41% | 0.02% | 104% | 100% |
| Class F | -0.07% | -0.11% | -0.14% | 105% | 99% | -0.11% | -0.64% | -0.45% | 102% | 98% |

# Conclusion

It was observed that the BD-rate change by the proposed method is 0.05% on RA and 0.14% on LDB compared with CE4.4.5.c. The overall BD-rate compared with VTM-3.0 is -0.19% on RA and 0.01% on LDB, respectively. It’s proposed to further study this method along with MMVD offset table reordering methods.

# Reference

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3),” JVT-L1002, Macao, CN, Oct 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,”, JVET-L1010 Macao, CN, Oct 2018.

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-L1024 Macao, CN, Oct 2018.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).