| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0436 |
| --- | --- |

| Title: | AHG2: Regarding HMVP Table Size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, U.S.A | Tel: Email: | 1-201-816-2000 jie.zhao@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

In VTM3, History-based Motion Vector Prediction (HMVP) allows to store up to 6 HMVP candidates for HMVP table. When updating the HMVP table, redundancy check is applied to prune HMVP candidates in the table. When HMVP candidates are added to merge list, maximum 5 HMVP candidates are allowed. Hence, when there are 6 HMVP candidates, the oldest entry of HMVP table is never used in the list construction. Therefore, it is proposed to change HMVP table size from 6 to 5. Experiments confirmed that with this change, there is no RD performance change. Results are identical.

# Introduction

In Oct. 2018 JVET meeting, History-based Motion Vector Prediction (HMVP) [1] was adopted into VTM3. A table with 6 HMVP candidates is maintained during the encoding/decoding process, and updated when there is an inter-coded non-affine block. When updating the HMVP table, redundancy check is firstly applied to find whether there is an identical HMVP in the table. If a redundant HMVP candidate is found, it is then removed from the table and all the rest HMVP candidates are moved forward, i.e., with indices reduced by 1. Hence, there are no redundant HMVP candidates within the table.

HMVP candidates are used in merge candidate list construction and AMVP candidate list construction process. Pruning is applied on the HMVP candidates relative to the existing spatial or temporal merge candidate in the merge list before an HMVP candidate being added into the merge or AMVP list.

Once the total number of available merge candidates reaches the MaxNumMergeCand minus 1, the merge candidate list construction process from HMVP list is terminated. MaxNumMergeCand is the maximum number of merging MVP candidate, and shall be in the range of 1 to 6, inclusive. In VTM3 common test condition, MaxNumMergeCand is equal to 6, therefore HMVP candidate is added only up to merge candidate 5.

In addition, in the VVC draft text 3, it specifies that the value of MaxNumMergeCand shall be in the range of 1 to 6, inclusive.

# Proposed

Here, it is proposed to change the HMVP table size to 5 because the oldest entry of the size 6 table will never be used in merge list construction. For merge list of size 6, HMVP is added starting from the latest entry and only added up to candidate 5. Because the HMVP table has already been redundancy checked, there are no duplicate entries within the table. That means at most 5 entries from the tables are needed to fill up the merge list.

During HMVP table update, having table size 6 or table size 5 also do not change the latest 5 entries because new entry is always added to the end of the table. Even if there is a duplicate, the older one will be removed, and the new one is added to the end.

Fig. 1. HMVP table of size 6

Therefore by changing the HMVP table size to 5, there should be no impact on RD result. It can reduce the HMVP table storage by 1 entry, and also reduce the number of redundancy check by 1 when updating the HMVP table.

# Experimental Results

VTM3 software is modified by only changing the MAX_NUM_HMVP_CANDS to 5. Tests are done following the CTC [2]. Experiments result confirmed that there is no change of coding performance. RD results are identical.

Table 4.1 Result of Changing HMVP table size to 5

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 97% |

# Conclusion

It is proposed to change HMVP Table size to 5 to eliminate the unnecessary entry of the table. There is no RD performance change.

# Reference

- L. Zhang, K. Zhang, H. Liu, Y. Wang, P. Zhao, D. Dong, “CE4: History-based Motion Vector Prediction (Test 4.4.7),” JVET-L0266, Macao, CN, Oct 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-L1010 Macao, CN, Oct 2018.

- B. Bross, J. Chen, S. Liu Versatile Video Coding (Draft 3), JVET-L1010, Macao, CN, Oct 2018.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).