| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0438 |
| --- | --- |

| Title: | CE10-related: Size constraint on Triangular Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, U.S.A | Tel: Email: | 1-201-816-2000 jie.zhao@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

In this document, it is proposed to disable Triangular prediction if either width or height of a block is 128 to reduce Triangular prediction complexity. Experimental result show that the coding impact is -0.01%, 0.00%, -0.01% for Y, U, V, components respectively in RA case, and 0.01%,0.00%,0.09% for Y, U,V, component respectively in LD case, and there is about 1% run time reduction for both encoder and decoder.

# Introduction

In Oct. 2018 JVET meeting, Triangular prediction [1] was adopted into VTM3. Triangular prediction is used for either skip or merge modes. It splits a CU into two triangular prediction units, in either diagonal or inverse diagonal direction. Each triangular prediction unit in the CU is inter-predicted using its own uni-prediction motion vector and reference frame index which are derived from a uni-prediction candidate list.

In VTM3, Triangular prediction can be applied to all block sizes larger than or equal to 8×8. There is no restriction of the maximum size that allows triangular prediction.

# Proposed

Here, we propose to not allow triangular prediction for blocks with either width or height is 128.

# Experimental Results

Proposed changes are implemented on top of VTM3 software. Tests are done following the CTC [2]. Experiments result showed that there is little performance impact to RD, and there is 1% run time reduction for both encoder and decoder.

Table 4.2 Result of disable Triangular for large block

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.00% | 98% | 99% |  |  |  |  |  |
| Class A2 | -0.02% | -0.01% | 0.06% | 98% | 98% |  |  |  |  |  |
| Class B | -0.01% | -0.01% | -0.06% | 99% | 98% | 0.01% | -0.13% | 0.14% | 99% | 102% |
| Class C | 0.00% | 0.03% | 0.00% | 100% | 102% | -0.01% | 0.16% | 0.15% | 100% | 97% |
| Class E |  |  |  |  |  | 0.01% | 0.02% | -0.06% | 98% | 96% |
| Overall | -0.01% | 0.00% | -0.01% | 99% | 99% | 0.01% | 0.00% | 0.09% | 99% | 99% |
| Class D | 0.00% | 0.00% | -0.01% | 101% | 103% | -0.03% | 0.42% | -0.04% | 100% | 100% |
| Class F | -0.02% | -0.04% | 0.01% | 98% | 99% | 0.02% | 0.04% | 0.03% | 98% | 99% |

# Conclusion

This document proposes to disable trianglular prediction for block with width or height equals to 128 to reduce complexity.  Experiments show that there is no loss. It is proposed to adopt this size constraint into VVC draft and VTM.

.

# Reference

- R.-L. Liao, C. S. Lim, “CE10.3.1.b: Triangular prediction unit mode,” JVET-L0124, Macao, CN, Oct 2018.

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVT-L1024, Macao, CN, Oct 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-L1010 Macao, CN, Oct 2018.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).