# Syntax

#### 7.3.4.6 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_enabled_flag && cbWidth >= 16 && cbHeight >= 16 ) { |  |
| inter_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_type_flag && inter_affine_flag[ x0 ][ y0 ] ) |  |
| cu_affine_type_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] == PRED_BI && refIdxSymL0 > -1 && refIdxSymL1 > -1 && inter_affine_flag[ x0 ][ y0 ] == 0 ) |  |
| sym_mvd_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 && !sym_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 0, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 0, 2 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 && !sym_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| … |  |
| } else { |  |
| if( !sym_mvd_flag[ x0 ][ y0 ] ) { |  |
| mvd_coding( x0, y0, 1, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 1, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 1, 2 ) |  |
| } |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| … |  |
| } |  |
| } |  |
| … |  |
| } |  |

# Semantics

#### 7.4.5.6 Coding unit semantics

sym_mvd_flag[ x0 ][ y0 ] specifies that the syntax elements ref_idx_l0[ x0 ][ y0 ] and ref_idx_l1[ x0 ][ y0 ], and the mvd_coding() syntax structure are not presented.

# Decoding process

#### 8.2.2 Derivation process for refIdxSymL0 and refIdxSymL1

[Proponent: The process requires reference picture list construction process being defined. The reference picture list construction is not defined yet in the current specification]

Input to this process are:

- RefPicListX, for X being replaced by 0 and 1

Output of this process is variables refIdxSymLX, for X being replaced by 0 and 1

The variables refIdxSymL0 and refIdxSymL1 are derived as follows:

- For X being replaced by 0 and 1, refIdxSymLX is set to −1

- The variable currPic specifies the current picture.

- For each index i with i = 0..( num_ref_idx_l0_active_minus1 + 1 ),

- When all of the following conditions are true, refIdxSymL0 is set to i

- DiffPicOrderCnt( currPic, RefPicList0[ i ] ) > 0,

- refIdxSymL0 is equal to -1 or DiffPicOrderCnt( currPic, RefPicList0[ i ] ) < DiffPicOrderCnt( currPic, RefPicList0[ refIdxSymL0 ] )

- For each index i with i = 0..( num_ref_idx_l1_active_minus1 + 1 ),

- When all of the following conditions are true, refIdxSymL1 is set to i

- DiffPicOrderCnt( currPic, RefPicList1[ i ] ) < 0, and

- RefIdxSymL1 is equal to -1 or DiffPicOrderCnt( currPic, RefPicList1[ i ] ) > DiffPicOrderCnt( currPic, RefPicList1[ refIdxSymL1 ] )

- If refIdxSymL0 is equal to −1 or refIdxSymL1 is equal to −1,

- For each index i with i = 0..( num_ref_idx_l0_active_minus1 + 1 ),

- When all of the following conditions are true, refIdxSymL0 is set to i

- DiffPicOrderCnt( currPic, RefPicList0[ i ] ) < 0,

- refIdxSymL0 is equal to -1 or DiffPicOrderCnt( currPic, RefPicList0[ i ] ) > DiffPicOrderCnt( currPic, RefPicList0[ refIdxSymL0 ] )

- For each index i with i = 0..( num_ref_idx_l1_active_minus1 + 1 ),

- When all of the following conditions are true, refIdxSymL1 is set to i

- DiffPicOrderCnt( currPic, RefPicList1[ i ] ) > 0,

- RefIdxSymL1 is equal to -1 or DiffPicOrderCnt( currPic, RefPicList1[ i ] ) < DiffPicOrderCnt( currPic, RefPicList1[ refIdxSymL1 ] )

### 8.4.2 Derivation process for motion vector components and reference indices

#### 8.4.2.1 General

…

- If merge_flag[ xCb ][ yCb ] is equal to 1, the derivation process for luma motion vectors for merge mode as specified in clause 8.4.2.2 is invoked with the luma location ( xCb, yCb ), the variables cbWidth and cbHeight inputs, and the output being the luma motion vectors mvL0[ 0 ][ 0 ], mvL1[ 0 ][ 0 ], the reference indices refIdxL0, refIdxL1, the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], and the bi-prediction weight index gbiIdx.

- the following applies:

- For X being replaced by either 0 or 1 in the variables predFlagLX[ 0 ][0 ], mvLX[ 0 ][0 ] and refIdxLX, in PRED_LX, and in the syntax elements ref_idx_lX and MvdLX, the following ordered steps apply:

- The variables refIdxLX and predFlagLX[ 0 ][0 ] are derived as follows:

- If sym_mvd_flag[ xCb ][ yCb ] is equal to 1,

refIdxLX = refIdxSymLX

predFlagLX[ 0 ][0 ] = 1

- Otherwise, if inter_pred_idc[ xCb ][ yCb ] is equal to PRED_LX or PRED_BI,

refIdxLX = ref_idx_lX[ xCb ][ yCb ]	(8-205)

predFlagLX[ 0 ][0 ] = 1		(8-206)

- Otherwise, the variables refIdxLX and predFlagLX[ 0 ][0 ] are specified by:

refIdxLX = −1		(8-207)

predFlagLX[ 0 ][0 ] = 0		(8-208)

- The variable mvdLX is derived as follows:

- If sym_mvd_flag[ xCb ][ yCb ] is equal to 1,

mvdL0[ 0 ] = MvdL0[ xCb ][ yCb ][ 0 ]

mvdL0[ 1 ] = MvdL0[ xCb ][ yCb ][ 0 ]

mvdL1[ 0 ] = −MvdL0[ xCb ][ yCb ][ 0 ]

mvdL1[ 1 ] = −MvdL0[ xCb ][ yCb ][ 0 ]

- Otherwise, the variables mvdLX is derived as follows:

mvdLX[ 0 ] = MvdLX[ xCb ][ yCb ][ 0 ]	(8-209)

mvdLX[ 1 ] = MvdLX[ xCb ][ yCb ][ 1 ]	(8-210)