| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0446-v2 |
| --- | --- |

| Title: | CE1: Rectangular virtual pipeline data unit (test 1.1.1) and supplementary results |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Xu, Xiang Li, Shan Liu | Tel: Email: | {mengxxu, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

VTM-3.0 adopts the CU split restriction regarding to the concept of square-shaped virtual pipeline data unit (VPDU). In this contribution, relaxed restrictions based on rectangular VPDU were proposed to enable the capabilities for two additional partitions, i.e., 128x32 and 32x128 CUs (and sub-partitions) for non-boundary blocks. It is reported to have 0.00%, 0.25%, 0.20%, and 0.17% average BD rate gain for AI, RA, LB, and LP configurations, with encoder complexity increased by 0%, 10%, 5%, 5%, respectively. It is reported that the proposed method benefits more for high-resolution (i.e., 4K and above) contents.

# Introduction

The concept of VPDU is introduced in JVET-K0556 as non-overlapping MxM-luma(L)/NxN-chroma(C) units in a picture. In certain decoders, successive VPDUs are processed by multiple pipeline stages at the same time; different stages process different VPDUs simultaneously. The VPDU size is roughly proportional to the buffer size in most pipeline stages.

VTM-3.0 adopts the CU split restriction regarding to certain requirements to reduce the pipeline buffer used in the VPDU for certain codecs. To be more specific, the partitions illustrated in Figure 1 are not allowed in VTM-3.0. However, certain block sizes are allowed at the picture boundary, including 128x32, 128x16, etc.

Figure 1 Partitions allowed in VTM-2.0 but not in VTM-3.0

# Proposed method

There are two issues in the VPDU design in VTM-3.0.

- Allowing rectangular VPDU would increase the RD performance.

- The sizes for the boundary and non-boundary blocks are not unified. For certain implementations (e.g., the encoder), certain buffer (e.g., 128x32) is allocated to handle the blocks only at the picture boundary, which is not efficient.

To address these issues, non-square VPDU are proposed in JVET-L0313, where each VPDU has the same size but may have various shapes, including rectangle, L-shape, or more complex shapes. In CE 1 test 1.1.1, the rectangular VPDU is tested. To avoid complicated address calculation, only two additional partitions over VTM-3.0, i.e., 128x32 and 32x128 (and sub-partitions), are introduced.

In the proposed method, the constraints on the CU partition based on square VPDU are relaxed so that more partitions are allowed. To be more specific, TT is allowed for 128x128 block, and BT in both directions are allowed in 128x64 and 64x128 blocks. However, horizontal or vertical TT is not allowed for 128x64 and 64x128 blocks. As an example, the partitions shown in the top row of Figure 1 are allowed, and those in the bottom row are not allowed.

An encoder-only fast method is also implemented at CTU level to adaptively enable or disable the partitions yield form rectangular VPDU.

# Simulation results

Reference: VTM-3.0

Test: CE1 Test 1.1.1

Compared with VTM-3.0, it shows 0.00%, 0.25%, 0.20%, and 0.17% gain on average for luma in AI, RA, LB, and LP configurations, respectively. It is also reported with up to 10% encoding complexity increase. It is observed that major gain comes from high-resolution sequences. For class A1 and A2, average 0.48% coding gain is achieved.

# Supplementary results

In addition to the CE test, two additional tests are provided. In the first test, the fast method is disabled for the CE test. In the second test, the VPDU constraints are applied on the boundary CUs so that the allowed block sizes are unified for both boundary and non-boundary blocks.

## Test with fast method disabled

Reference: VTM-3.0

Test 1: CE1 Test 1.1.1 without fast method

Compared with VTM-3.0, it shows 0.30%, 0.21%, and 0.20% gain on average for luma in RA, LB, and LP configurations, respectively. It is also reported with up to 10% encoding complexity increase.

## Test with enforcing square VPDU constraints at picture boundary

Reference: VTM-3.0

Test 2: VTM-3.0 with square VPDU constraints on boundary CUs, i.e., disallowing 128x32 and 32x128 partitions (and some sub-partitions that do not satisfy the square VPDU condition) at picture boundary.

Compared with VTM-3.0, it shows 0.04%, 0.07%, and 0.01% loss on average for luma in RA, LB, and LP configurations, respectively. It is also reported up to 2% encoding complexity savings.

# Conclusion

In this contribution, square VPDU is reported to have 0.25%, 0.20%, and 0.17% gain on average for luma in RA, LB, and LP configurations, respectively, with up to 10% encoding complexity increasing. Without the encoder-only fast method, it shows 0.30%, 0.21%, and 0.20% gain on average for luma in RA, LB, and LP configurations, respectively. More important, 128x32 and 32x128 partitions have been supported in current VTM and VVC draft. It is proposed to support 128x32 and 32x128 partitions for non-boundary blocks.

# References

- “JVET common test conditions and software reference configurations for SDR video,” F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-L1010.

- “Description of Core Experiment 1 (CE 1): Partitioning,” J. Ma, F. Le Léannec, M. W. Park, JVET-L1021.

- “CE1-related: CE1-related: Constraint for binary and ternary partitions,” C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, JVET-K0556.

- “CE1-related: Non-square virtual pipeline data unit,” M. Xu, X. Li, S. Liu, JVET-L0313.

- “Versatile Video Coding (Draft 3)”, B. Bross, J. Chen, S. Liu, JVET-L1001.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).