| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0450-v2 |
| --- | --- |

| Title: | CE10-related: LIC inheritance restrictions and interaction with GBI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Xu, Xiang Li, Xiaozhong Xu, Shan Liu | Tel: Email: | {mengxxu, xlxiangli, xiaozhongxu, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

The LIC tool tested in CE10.5.2 is explicitly signaled or implicitly derived. When it is derived from a spatial or temporal neighboring block, the LIC flags needs to be stored in temporal and spatial buffers. This contribution describes three methods to restrict the LIC flag derivation, so that the storage for LIC flag in certain buffers is no longer required. In method 1, LIC flag is not allowed to inherit from a temporal neighbor, with 0.00%, -0.02%, and 0.01% BD-rate difference on average compared with CE10.5.2 for RA, LB, and LP, respectively. In method 2, LIC flag is not allowed to inherit from a different CTU row, with 0.07%, 0.03%, and 0.07% BD-rate loss on average compared with CE10.5.2 for RA, LB, and LP, respectively. In method 3, LIC flag is not allowed to inherit from a different CTU, with 0.09%, 0.04%, and 0.09% BD-rate loss on average compared with CE10.5.2 for RA, LB, and LP, respectively. In addition, it is proposed to disable GBI when LIC is on. It shows 0.03% and 0.00% BD-rate loss on average for RA and LB, respectively.

# Introduction

The LIC tool in JEM is simplified and tested in CE10.5.2. In AMVP, the LIC flag is explicitly signaled. In skip/merge, it is implicitly derived, as inherited from a temporal or spatial neighboring block. Therefore, the LIC flag needs to be stored in the temporal and spatial buffer (e.g., frame buffer, line buffer, etc.). Moreover, GBI is allowed when LIC is on, which looks not efficient.

# Proposed method

Three methods are proposed to disallow certain LIC flag inheritance so that the LIC flag does not need to be stored in the temporal or spatial neighboring blocks.

In method 1, the LIC flag is not allowed to be inherited from a temporal reference block. Thus the LIC flag does not need to be stored in the temporal buffer.

In method 2, the LIC flag is not allowed to be inherited from a block in a different CTU row. Thus the LIC flag does not need to be stored in the line buffer.

In method 3, the LIC flag is not allowed to be inherited from a block in a different CTU. Thus the LIC flag does not need to be stored in the spatial buffer other than the current CTU buffer.

In addition, it is proposed to disable GBI when LIC is on.

# Simulation results

The proposed three methods are implemented on top of CE10.5.2 software.

## Method 1, LIC inheritance not allowed from temporal reference

1st reference: VTM-3.0

2nd reference: CE10.5.2

Test: method 1, LIC inheritance not allowed from temporal reference

With LIC flag is not allowed to be inherited from a temporal reference block, it is reported to have 0.00%, -0.02%, and 0.01% BD-rate difference on average compared with CE10.5.2 for RA, LB, and LP, respectively.

## Method 2, LIC inheritance not allowed from a different CTU row

1st reference: VTM-3.0

2nd reference: CE10.5.2

Test: method2, LIC inheritance not allowed from a different CTU row

With LIC flag is not allowed to be inherited from a different CTU row, it is reported to have 0.07%, 0.03%, and 0.07% BD-rate loss on average compared with CE10.5.2 for RA, LB, and LP, respectively.

## Method 3, LIC inheritance not allowed from a different CTU

1st reference: VTM-3.0

2nd reference: CE10.5.2

Test: method3, LIC inheritance not allowed from a different CTU

With LIC flag is not allowed to be inherited from a different CTU, it is reported to have 0.09%, 0.04%, and 0.09% BD-rate loss on average compared with CE10.5.2 for RA, LB, and LP, respectively.

## Disabling GBI when LIC is on

1st reference: VTM-3.0

2nd reference: CE10.5.2

Test: disabling GBI when LIC is used

In this test, GBI is disabled when LIC is on. It is reported to have 0.03% and 0.00% BD-rate loss on average for RA and LB, respectively.

# Conclusion

The storage for LIC flag in the temporal or spatial buffer may be removed by the proposed methods. In method 1, LIC flag is not allowed to inherit from a temporal neighbor, with 0.00%, -0.02%, and 0.01% BD-rate difference on average compared with CE10.5.2 for RA, LB, and LP, respectively. In method 2, LIC flag is not allowed to inherit from a different CTU row, with 0.07%, 0.03%, and 0.07% BD-rate loss on average compared with CE10.5.2 for RA, LB, and LP, respectively. In method 3, LIC flag is not allowed to inherit from a different CTU, with 0.09%, 0.04%, and 0.09% BD-rate loss on average compared with CE10.5.2 for RA, LB, and LP, respectively. In addition, it is proposed to disable GBI when LIC is on, with 0.03% and 0.00% BD-rate loss on average for RA and LB, respectively.

# References

- “JVET common test conditions and software reference configurations for SDR video,” F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-L1010.

- “Description of Core Experiment 10 (CE10): Combined and multi-hypothesis prediction,” C.-W. Hsu, M. Winken, JVET-L1030.

- “Versatile Video Coding (Draft 3)”, B. Bross, J. Chen, S. Liu, JVET-L1001.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).