| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0453-v2 |
| --- | --- |

| Title: | CE5 on arithmetic coding: experiments 5.1.1, 5.1.2, 5.1.3, 5.1.4, 5.1.5, 5.1.6, 5.1.7, 5.1.8, 5.1.10, 5.1.11, 5.1.12, 5.1.13, 5.2, and more |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen 335 Britannia Rd E Mississauga ON L4Z 1W9 Canada | Tel: Email: | +1 647 745 1691 frank@bossentech.com |
| Source: | Sharp Electronics of Canada Ltd |  |  |

# Abstract

This report provides results for the following CE5 experiments: 5.1.1, 5.1.2, 5.1.3, 5.1.4, 5.1.5, 5.1.6, 5.1.7, 5.1.8, 5.1.10, 5.1.11, 5.1.12, 5.1.13 and 5.2. Additional variants of these experiments are considered where constants, such as initial state values and shift amounts, are modified.

Version 2 of the document provides additional results for experiment 5.2 on throughput, as well as BD-rate results for a configuration that yields no encoder run time increase.

# Summary of results

The table below summarizes the performance of various arithmetic coding engines, classifying them in three groups:

- Two states, fixed window sizes

- Two states, variable (context-adaptive) window sizes

- Single state, variable (context-adaptive) window sizes

BD rate numbers are reported as combined YUV were BD rate numbers for Y, U and V are weighted as 90%/5%/5% to obtain a single number. This is done to account for the fact that results for individual color components can sometimes diverge.

Also provided for each engine are encoder/decoder run times relative to VTM3, as well as SW throughput results from experiment 5.2.

Within each group and each test condition the best BD rate and throughput results are highlighted in green. The worst results are highlighted in red.

Experiment numbers with a star indicate a variant, where some constants are modified as described.

More detailed results are provided in sections 2 and 3.

| Engine | AI | RA | LB | LP | AI | RA | LB | Thruput |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 states, fixed window size |  |  |  |  |  |  |  |  |
| 5.1.1 | -0.85% | -0.74% | -0.56% | -0.56% | 106/103 | 103/102 | 101/100 | 120.17 |
| 5.1.5 | -0.83% | -0.64% | -0.62% | -0.60% | 109/107 | 105/103 | 103/103 | 113.46 |
| 5.1.8 | -0.91% | -0.69% | -0.48% | -0.50% | 106/102 | 103/101 | 100/98 | 118.80 |
| 5.1.10 | -0.83% | -0.70% | -0.55% | -0.61% | 108/104 | 104/102 | 103/101 | 125.05 |
| 5.1.10* + new init from 5.1.2 | -0.84% | -0.73% | -0.51% | -0.53% | 109/107 | 105/104 | 105/104 | 125.05 |
| 5.1.10* 5x6 mult | -0.85% | -0.73% | -0.58% | -0.64% | 109/107 | 105/104 | 104/104 | 127.86 |
| 2 states, variable window size |  |  |  |  |  |  |  |  |
| 5.1.2 | -1.00% | -0.97% | -0.82% | -0.76% | 109/105 | 104/103 | 101/101 | 115.00 |
| 5.1.3 clz + 8x8 table | -1.03% | -0.96% | -0.74% | -0.83% | 107/108 | 104/104 | 104/104 | 106.63 |
| 5.1.3 4x5 mult | -0.99% | -0.91% | -0.71% | -0.83% | 109/105 | 104/102 | 103/102 | 127.28 |
| 5.1.3* 5x6 mult | -1.07% | -0.99% | -0.79% | -0.91% | 108/104 | 104/103 | 104/103 | 127.28 |
| 5.1.6 8+12 bit state | -0.90% | -0.76% | -0.72% | -0.69% | 111/105 | 105/103 | 105/102 | 110.75 |
| 5.1.6 10+14 bit state | -0.97% | -0.82% | -0.67% | -0.73% | 112/105 | 105/102 | 105/102 | 110.75 |
| 5.1.11 | -0.97% | -0.93% | -0.75% | -0.76% | 108/103 | 103/101 | 102/101 | 121.79 |
| 5.1.11* + new init from 5.1.2 | -1.00% | -0.98% | -0.77% | -0.75% | 110/105 | 104/102 | 104/102 | 121.79 |
| 5.1.13 | -0.95% | -0.91% | -0.73% | -0.74% | 109/103 | 104/101 | 101/98 | 127.28 |
| 5.1.13* +new init from 5.1.2 | -0.98% | -0.96% | -0.75% | -0.73% | 110/105 | 104/103 | 106/103 | 127.28 |
| 5.1.13* 5x6 mult | -1.03% | -0.98% | -0.75% | -0.75% | 109/105 | 104/102 | 104/104 | 127.28 |
| 1 state, variable window size |  |  |  |  |  |  |  |  |
| 5.1.4 clz + 8x8 table | -0.92% | -0.65% | -0.41% | -0.44% | 108/107 | 103/104 | 104/103 | 109.09 |
| 5.1.4 4x5 mult | -0.83% | -0.58% | -0.37% | -0.41% | 108/105 | 104/102 | 102/102 | 132.95 |
| 5.1.4* 5x6 mult | -0.93% | -0.67% | -0.46% | -0.49% | 107/105 | 104/102 | 104/103 | 132.95 |
| 5.1.7 | -0.70% | -0.57% | -0.53% | -0.52% | 107/107 | 104/103 | 103/104 | 119.00 |
| 5.1.12 | -0.65% | -0.44% | -0.33% | -0.25% | 108/103 | 102/100 | 101/99 | 128.95 |

Several alternatives may be considered for inclusion into the VVC working draft:

- Stick with the current engine, which dates from 2002. While its compression performance doesn’t match the state of the art, it achieves high throughput and people are familiar with how to implement it efficiently. Furthermore run times would remain unchanged from VTM 3.

- Use an engine with 1 state and variable window sizes. The compression benefit of doing so is about 0.5% and the throughput (in SW at least) can match the AVC/HEVC engine. The 5.1.4 variant with a small multiplier (e.g., 5.1.4* 5x6 mult in the table above) should be preferred given its throughput and coding efficiency.

- Use an engine with 2 states and variable window sizes. This provides the maximum compression efficiency benefit of about 1%, while the SW throughput is somewhat lower (by about 4%). The 5.1.3 variant with a small multiplier (e.g., 5.1.3* 5x6 mult in the table above) should be preferred given its throughput and coding efficiency.

- Having an engine with 2 states and fixed window sizes does not appear to provide a very good tradeoff in terms of compression efficiency and throughput when compared to the other solutions. It may thus be the least desirable option based on the data available herein.

The increase in encoder run time with the new engines may be undesirable. To avoid such an increase it is possible to use the existing CABAC engine to make all RDO decisions and use only the new engine for the final encoding. Such a strategy yields the following results:

| Engine | AI | RA | LB | LP | AI | RA | LB | Thruput |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.1.3* 5x6 mult, fast encode | -0.36% | -0.46% | -0.32% | -0.45% | 97/99 | 98/99 | 99/98 | 127.28 |

# Coding efficiency experiments

## CE 5.1.1: linear state, dual estimator, fixed window size

Features may be summarized as follows:

| State representation | 10 + 14 bit linear, full range |
| --- | --- |
| rLPS computation | 32x8x8 bit table |
| Initialization | 128x15 bit to map HEVC-like state to linear representation, retrained init values |
| Rate estimation | 256x2x19 bit table |
| Window size | Fixed a=4 b=7 |
| Other | rMPS >= 128 is guaranteed |

Results are summarized in the tables below.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.81% | -1.36% | -1.22% | 109% | 103% |
| Class A2 | -0.84% | -1.31% | -1.27% | 102% | 103% |
| Class B | -0.79% | -0.96% | -1.09% | 107% | 101% |
| Class C | -0.82% | -0.99% | -1.19% | 105% | 106% |
| Class E | -0.79% | -1.26% | -1.30% | 109% | 105% |
| Overall | -0.81% | -1.15% | -1.20% | 106% | 103% |
| Class D | -0.78% | -0.99% | -1.01% | 103% | 104% |
| Class F | -0.68% | -0.83% | -0.71% | 104% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.62% | -1.37% | -1.23% | 103% | 102% |
| Class A2 | -0.71% | -0.94% | -0.76% | 101% | 100% |
| Class B | -0.72% | -0.92% | -1.04% | 104% | 101% |
| Class C | -0.75% | -1.08% | -0.86% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.70% | -1.06% | -0.97% | 103% | 102% |
| Class D | -0.75% | -1.59% | -1.22% | 101% | 101% |
| Class F | -0.44% | -0.49% | -0.48% | 103% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.63% | -0.35% | -0.59% | 101% | 103% |
| Class C | -0.63% | -0.35% | -0.38% | 102% | 98% |
| Class E | -0.54% | 0.16% | 0.76% | 100% | 97% |
| Overall | -0.60% | -0.22% | -0.18% | 101% | 100% |
| Class D | -0.51% | -1.04% | -0.54% | 105% | 103% |
| Class F | -0.19% | -0.36% | -0.20% | 104% | 100% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.61% | 0.04% | -0.33% | 102% | 101% |
| Class C | -0.57% | -0.58% | -0.58% | 102% | 99% |
| Class E | -0.64% | 0.25% | 0.56% | 104% | 100% |
| Overall | -0.60% | -0.11% | -0.19% | 103% | 100% |
| Class D | -0.53% | -1.28% | -0.27% | 102% | 103% |
| Class F | -0.05% | -0.67% | 0.24% | 102% | 99% |

## CE 5.1.2: linear state, dual estimator, variable window size

Features may be summarized as follows:

| State representation | 10 + 14 bit linear, full range |
| --- | --- |
| rLPS computation | 32x8x8 bit table |
| Initialization | 128x15 bit to map HEVC-like state to linear representation, retrained init values |
| Rate estimation | 256x2x19 bit table |
| Window size | Defined per context a=2..5 b=a+3..6 |
| Other | rMPS >= 128 is guaranteed |

Results are summarized in the tables below.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.00% | -1.38% | -1.09% | 112% | 106% |
| Class A2 | -1.03% | -1.36% | -1.28% | 108% | 105% |
| Class B | -0.95% | -0.95% | -1.21% | 108% | 103% |
| Class C | -0.93% | -1.04% | -1.26% | 107% | 105% |
| Class E | -0.96% | -1.39% | -1.57% | 111% | 105% |
| Overall | -0.97% | -1.18% | -1.27% | 109% | 105% |
| Class D | -0.91% | -1.10% | -1.07% | 106% | 105% |
| Class F | -0.80% | -0.99% | -0.89% | 106% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.93% | -1.34% | -1.11% | 104% | 104% |
| Class A2 | -0.95% | -0.85% | -0.80% | 101% | 102% |
| Class B | -1.01% | -0.76% | -1.01% | 105% | 103% |
| Class C | -0.95% | -1.13% | -0.79% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.97% | -0.99% | -0.93% | 104% | 103% |
| Class D | -0.95% | -1.56% | -0.99% | 104% | 103% |
| Class F | -0.69% | -0.82% | -0.82% | 104% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.86% | -0.05% | 0.37% | 103% | 106% |
| Class C | -0.85% | -0.40% | -0.54% | 99% | 97% |
| Class E | -1.00% | 0.28% | -0.90% | 101% | 98% |
| Overall | -0.89% | -0.08% | -0.25% | 101% | 101% |
| Class D | -0.81% | -0.12% | -0.06% | 107% | 104% |
| Class F | -0.48% | -0.04% | -0.44% | 100% | 98% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.83% | 0.42% | 0.59% | 104% | 103% |
| Class C | -0.80% | -0.46% | -0.49% | 103% | 98% |
| Class E | -0.88% | 0.04% | -0.95% | 103% | 101% |
| Overall | -0.83% | 0.03% | -0.15% | 103% | 101% |
| Class D | -0.75% | 0.20% | -0.28% | 106% | 105% |
| Class F | -0.46% | -0.59% | -0.09% | 103% | 99% |

## CE 5.1.3: linear state, dual estimator, variable window size

Features may be summarized as follows:

| State representation | 10 + 14 bit linear, full range |
| --- | --- |
| rLPS computation | Count leading zeros + 8x8x8 bit table + variable shift |
| Initialization | 128x15 bit to map HEVC-like state to linear representation, retrained init values |
| Rate estimation | 256x2x19 bit table |
| Window size | Defined per context a=2..5 b=a+1..4 |
| Other | rMPS may be less than 128 |

The rLPS computation method is described in more details in the CE 5.1.4 section.

Results are summarized in the tables below.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.05% | -1.65% | -1.50% | 110% | 110% |
| Class A2 | -1.02% | -1.59% | -1.48% | 108% | 109% |
| Class B | -0.97% | -1.27% | -1.47% | 109% | 107% |
| Class C | -0.94% | -1.24% | -1.49% | 104% | 109% |
| Class E | -0.98% | -1.55% | -1.77% | 105% | 108% |
| Overall | -0.99% | -1.42% | -1.53% | 107% | 108% |
| Class D | -0.81% | -1.15% | -1.09% | 104% | 106% |
| Class F | -0.72% | -1.14% | -1.08% | 107% | 108% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.98% | -1.43% | -1.41% | 106% | 105% |
| Class A2 | -0.92% | -0.85% | -1.02% | 104% | 102% |
| Class B | -0.98% | -0.98% | -1.02% | 105% | 104% |
| Class C | -0.90% | -1.09% | -0.78% | 103% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.95% | -1.07% | -1.04% | 104% | 104% |
| Class D | -0.73% | -1.26% | -0.74% | 102% | 102% |
| Class F | -0.56% | -0.73% | -0.74% | 104% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.86% | -0.36% | 0.30% | 104% | 107% |
| Class C | -0.79% | -0.20% | -0.17% | 104% | 103% |
| Class E | -0.78% | 0.69% | -0.86% | 105% | 101% |
| Overall | -0.82% | -0.04% | -0.15% | 104% | 104% |
| Class D | -0.48% | -1.05% | 0.10% | 106% | 107% |
| Class F | -0.39% | 0.14% | -0.19% | 103% | 103% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.90% | 0.09% | 0.69% | 106% | 105% |
| Class C | -0.80% | -0.42% | -0.55% | 104% | 102% |
| Class E | -1.07% | 0.17% | -1.38% | 105% | 102% |
| Overall | -0.91% | -0.06% | -0.24% | 105% | 103% |
| Class D | -0.64% | -0.64% | -0.03% | 106% | 107% |
| Class F | -0.26% | -0.44% | -0.80% | 106% | 103% |

Alternatively the engine can be configured to use a small multiplier as in JVET-L0335 (macro JVET_L0335 is set to 1). Features may be summarized as follows:

| State representation | 10 + 14 bit linear, reduced range |
| --- | --- |
| rLPS computation | 4x5 bit multiplier |
| Initialization | 128x15 bit to map HEVC-like state to linear representation, retrained init values |
| Rate estimation | 256x2x19 bit table |
| Window size | Defined per context a=2..5 b=a+1..4 |
| Other | rMPS >= 128 is guaranteed |

Results are summarized in the tables below.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.00% | -1.56% | -1.38% | 111% | 105% |
| Class A2 | -0.98% | -1.57% | -1.48% | 108% | 104% |
| Class B | -0.94% | -1.15% | -1.40% | 108% | 103% |
| Class C | -0.89% | -1.12% | -1.33% | 107% | 107% |
| Class E | -0.95% | -1.54% | -1.65% | 111% | 106% |
| Overall | -0.95% | -1.35% | -1.44% | 109% | 105% |
| Class D | -0.78% | -1.23% | -1.03% | 104% | 104% |
| Class F | -0.71% | -1.02% | -1.03% | 107% | 105% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.93% | -1.34% | -1.34% | 106% | 103% |
| Class A2 | -0.87% | -0.92% | -1.00% | 103% | 101% |
| Class B | -0.93% | -1.03% | -0.95% | 104% | 102% |
| Class C | -0.87% | -1.00% | -0.67% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.90% | -1.06% | -0.97% | 104% | 102% |
| Class D | -0.67% | -1.21% | -0.89% | 101% | 101% |
| Class F | -0.54% | -0.63% | -0.76% | 104% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.82% | -0.05% | 0.37% | 103% | 104% |
| Class C | -0.74% | -0.25% | -0.16% | 104% | 102% |
| Class E | -0.74% | 0.39% | -1.33% | 103% | 100% |
| Overall | -0.77% | -0.01% | -0.23% | 103% | 102% |
| Class D | -0.48% | -0.61% | -0.30% | 102% | 104% |
| Class F | -0.18% | 0.06% | 0.04% | 105% | 103% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.84% | 0.17% | 0.66% | 104% | 104% |
| Class C | -0.71% | -0.45% | -0.39% | 103% | 101% |
| Class E | -1.22% | -0.16% | -1.96% | 105% | 102% |
| Overall | -0.89% | -0.12% | -0.35% | 104% | 103% |
| Class D | -0.53% | -1.15% | -0.21% | 104% | 106% |
| Class F | -0.35% | -0.13% | -0.26% | 105% | 101% |

## CE 5.1.4: linear state, single estimator, variable window size

Features may be summarized as follows:

| State representation | 14 bit linear, full range |
| --- | --- |
| rLPS computation | Count leading zeros + 8x8x8 bit table + variable shift |
| Initialization | 128x15 bit to map HEVC-like state to linear representation, retrained init values |
| Rate estimation | 256x2x19 bit table |
| Window size | Defined per context a=2..9 |
| Other | rMPS may be less than 128 |

This engine uses a single 14-bit estimator. The window size is separately configured for each context using a 3-bit value indicating a shift parameter between 2 and 9.

Given a range R and a probability estimate p (a 15-bit value, obtained by doubling the 14-bit value from the estimator), the range for the least probable symbol is computed as:

plps = p & 0x4000 ? 0x7fff ^ p : p
nlz = clz(plps) & 15
a = plps << nlz >> 12
b = R >> 4
rlps = ((a | 1) * (b | 1) >> nlz) | 2

where clz() is a count-leading-zeros function and the multiplication is 5 by 5 bits wide.

It should be noted that for R = 256 and plps = 0x3fff, the value of rlps is 131 and the value of rmps is this 125. This implies that renormalization after processing an MPS may require shifts by more than 1 bit. This may be disadvantageous for certain implementations.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.97% | -1.21% | -0.98% | 110% | 108% |
| Class A2 | -1.01% | -1.26% | -1.11% | 106% | 107% |
| Class B | -0.88% | -1.18% | -1.23% | 108% | 106% |
| Class C | -0.81% | -1.17% | -1.24% | 107% | 109% |
| Class E | -0.80% | -1.26% | -1.29% | 111% | 108% |
| Overall | -0.89% | -1.21% | -1.18% | 108% | 107% |
| Class D | -0.76% | -0.87% | -1.08% | 103% | 105% |
| Class F | -0.51% | -0.75% | -0.69% | 108% | 106% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.74% | -0.34% | -0.34% | 103% | 104% |
| Class A2 | -0.64% | 0.21% | 0.09% | 103% | 103% |
| Class B | -0.73% | 0.01% | 0.16% | 104% | 103% |
| Class C | -0.70% | -0.88% | -0.22% | 103% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.71% | -0.26% | -0.05% | 103% | 104% |
| Class D | -0.60% | -0.63% | -0.60% | 102% | 102% |
| Class F | -0.24% | -0.27% | 0.03% | 104% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.61% | 1.52% | 2.70% | 105% | 106% |
| Class C | -0.66% | 0.36% | 0.92% | 104% | 102% |
| Class E | -0.53% | 1.07% | 1.34% | 101% | 101% |
| Overall | -0.61% | 1.02% | 1.76% | 104% | 103% |
| Class D | -0.45% | 1.16% | 0.89% | 105% | 106% |
| Class F | 0.02% | 0.32% | 0.69% | 104% | 102% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.60% | 1.83% | 3.01% | 106% | 104% |
| Class C | -0.61% | 0.39% | 0.75% | 103% | 103% |
| Class E | -0.83% | 1.50% | 1.07% | 103% | 102% |
| Overall | -0.66% | 1.27% | 1.77% | 104% | 103% |
| Class D | -0.51% | 0.67% | 1.05% | 104% | 106% |
| Class F | 0.14% | 0.23% | 0.81% | 100% | 103% |

Alternatively a subrange computation rule based on JVET-L0335 is tested as:

plps = p & 0x4000 ? 0x7fff ^ p : p
a = plps >> 9
b = R >> 5
rlps = (a * b >> 1) + 4

where the multiplication is 5 by 4 bits wide.

| State representation | 14 bit linear, reduced range |
| --- | --- |
| rLPS computation | 5x4 bit multiplier |
| Initialization | 128x15 bit to map HEVC-like state to linear representation, retrained init values |
| Rate estimation | 256x2x19 bit table |
| Window size | Defined per context a=2..9 |
| Other | rMPS >= 128 is guaranteed |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.86% | -1.15% | -0.88% | 110% | 103% |
| Class A2 | -0.91% | -1.16% | -1.04% | 106% | 105% |
| Class B | -0.80% | -1.06% | -1.15% | 108% | 103% |
| Class C | -0.73% | -1.06% | -1.21% | 107% | 107% |
| Class E | -0.73% | -1.21% | -1.22% | 110% | 105% |
| Overall | -0.80% | -1.12% | -1.11% | 108% | 105% |
| Class D | -0.70% | -0.90% | -1.09% | 105% | 103% |
| Class F | -0.45% | -0.70% | -0.63% | 105% | 106% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.65% | -0.20% | -0.24% | 105% | 103% |
| Class A2 | -0.57% | 0.31% | 0.12% | 102% | 101% |
| Class B | -0.67% | 0.02% | 0.06% | 105% | 102% |
| Class C | -0.64% | -0.53% | -0.01% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.64% | -0.11% | -0.01% | 104% | 102% |
| Class D | -0.52% | -0.67% | -0.46% | 101% | 101% |
| Class F | -0.25% | -0.32% | 0.06% | 103% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.55% | 1.31% | 2.47% | 101% | 104% |
| Class C | -0.60% | 0.52% | 0.98% | 103% | 101% |
| Class E | -0.52% | 1.16% | 0.64% | 104% | 100% |
| Overall | -0.56% | 1.01% | 1.51% | 102% | 102% |
| Class D | -0.34% | 1.09% | 1.17% | 104% | 105% |
| Class F | 0.00% | 1.40% | 0.84% | 105% | 103% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.54% | 1.94% | 2.99% | 102% | 104% |
| Class C | -0.56% | 0.38% | 0.84% | 103% | 101% |
| Class E | -0.80% | 1.53% | 0.35% | 106% | 104% |
| Overall | -0.61% | 1.32% | 1.62% | 104% | 103% |
| Class D | -0.44% | 1.03% | 1.08% | 102% | 104% |
| Class F | 0.17% | 0.75% | 0.43% | 105% | 102% |

The difference between the 2 rules shows that there is an added benefit from a RD perspective from the more precise multiplication.

Additional tests not described in the CE were conducted where the JVET-L0335 method is used but with higher precision, requiring a 5x6 bit multiplier instead of a 4x5 one:

plps = p & 0x4000 ? 0x7fff ^ p : p
a = plps >> 8
b = R >> 4
rlps = (a * b >> 3) + 2

The results below show that the gap can be completely closed.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.98% | -1.26% | -0.99% | 108% | 104% |
| Class A2 | -1.02% | -1.27% | -1.15% | 106% | 105% |
| Class B | -0.90% | -1.15% | -1.24% | 107% | 103% |
| Class C | -0.83% | -1.15% | -1.30% | 105% | 106% |
| Class E | -0.81% | -1.29% | -1.30% | 110% | 106% |
| Overall | -0.90% | -1.21% | -1.21% | 107% | 105% |
| Class D | -0.78% | -0.99% | -1.18% | 105% | 105% |
| Class F | -0.52% | -0.77% | -0.70% | 108% | 106% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.75% | -0.30% | -0.34% | 106% | 103% |
| Class A2 | -0.66% | 0.22% | 0.03% | 101% | 101% |
| Class B | -0.76% | -0.07% | -0.03% | 104% | 102% |
| Class C | -0.72% | -0.60% | -0.09% | 104% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.73% | -0.20% | -0.10% | 104% | 102% |
| Class D | -0.59% | -0.74% | -0.53% | 101% | 101% |
| Class F | -0.31% | -0.38% | 0.00% | 103% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.65% | 1.20% | 2.36% | 104% | 105% |
| Class C | -0.67% | 0.44% | 0.90% | 103% | 102% |
| Class E | -0.59% | 1.08% | 0.56% | 105% | 100% |
| Overall | -0.64% | 0.92% | 1.42% | 104% | 103% |
| Class D | -0.42% | 1.01% | 1.09% | 106% | 106% |
| Class F | -0.07% | 1.33% | 0.78% | 105% | 103% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.64% | 1.83% | 2.88% | 105% | 103% |
| Class C | -0.64% | 0.30% | 0.76% | 104% | 101% |
| Class E | -0.88% | 1.45% | 0.28% | 107% | 103% |
| Overall | -0.70% | 1.23% | 1.52% | 105% | 102% |
| Class D | -0.52% | 0.95% | 1.00% | 103% | 105% |
| Class F | 0.11% | 0.68% | 0.36% | 107% | 104% |

## 5.1.5: log state,  dual estimator, fixed window size

Features may be summarized as follows:

| State representation | 8 + 12 bit logarithmic |
| --- | --- |
| rLPS computation | 32x8x8 bit table |
| Initialization | 128x16 bit to map HEVC-like state to linear representation, retrained init values |
| Rate estimation | 1024x2x18 bit table |
| Window size | Fixed a=4 b=7 |
| Other | rMPS >= 128 is guaranteed |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.85% | -1.36% | -1.34% | 111% | 109% |
| Class A2 | -0.81% | -1.60% | -1.44% | 106% | 107% |
| Class B | -0.76% | -1.44% | -1.41% | 109% | 105% |
| Class C | -0.73% | -1.38% | -1.40% | 107% | 108% |
| Class E | -0.69% | -1.60% | -1.45% | 111% | 107% |
| Overall | -0.76% | -1.47% | -1.41% | 109% | 107% |
| Class D | -0.70% | -1.30% | -1.09% | 105% | 105% |
| Class F | -0.60% | -0.96% | -0.89% | 109% | 107% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.65% | -0.83% | -0.90% | 107% | 104% |
| Class A2 | -0.60% | -0.41% | -0.57% | 103% | 102% |
| Class B | -0.68% | -0.80% | -0.75% | 105% | 103% |
| Class C | -0.58% | -1.03% | -0.57% | 104% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.63% | -0.79% | -0.69% | 105% | 103% |
| Class D | -0.49% | -1.15% | -0.48% | 102% | 102% |
| Class F | -0.40% | -0.45% | -0.47% | 103% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.69% | -0.20% | 0.33% | 103% | 106% |
| Class C | -0.64% | -0.58% | -0.11% | 105% | 102% |
| Class E | -0.73% | -0.06% | -0.09% | 102% | 101% |
| Overall | -0.68% | -0.29% | 0.08% | 103% | 103% |
| Class D | -0.59% | -0.31% | 0.23% | 106% | 105% |
| Class F | -0.22% | -0.46% | 0.01% | 104% | 104% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.69% | 0.31% | 0.65% | 106% | 104% |
| Class C | -0.56% | -0.60% | -0.22% | 104% | 102% |
| Class E | -0.77% | 0.07% | -0.53% | 106% | 103% |
| Overall | -0.67% | -0.05% | 0.06% | 106% | 103% |
| Class D | -0.51% | -0.64% | -0.66% | 103% | 104% |
| Class F | -0.28% | -0.51% | -0.41% | 107% | 103% |

## 5.1.6: log state, dual estimator, variable window size

There are two versions of this engine. The first uses an 8+12 bit representation and the second a 10+14 bit representation.

8+12 bits:

Features may be summarized as follows:

| State representation | 8 + 12 bit logarithmic |
| --- | --- |
| rLPS computation | 32x8x8 bit table |
| Initialization | 128x16 bit to map HEVC-like state to linear representation, retrained init values |
| Rate estimation | 1024x2x18 bit table |
| Window size | Variable |
| Other | rMPS >= 128 is guaranteed |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.90% | -1.33% | -1.41% | 112% | 104% |
| Class A2 | -0.86% | -1.59% | -1.46% | 110% | 104% |
| Class B | -0.82% | -1.51% | -1.46% | 111% | 104% |
| Class C | -0.82% | -1.49% | -1.61% | 109% | 107% |
| Class E | -0.77% | -1.63% | -1.71% | 112% | 106% |
| Overall | -0.83% | -1.51% | -1.53% | 111% | 105% |
| Class D | -0.81% | -1.24% | -1.12% | 107% | 105% |
| Class F | -0.60% | -1.12% | -1.10% | 110% | 107% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.78% | -0.76% | -0.94% | 107% | 103% |
| Class A2 | -0.71% | -0.23% | -0.61% | 103% | 101% |
| Class B | -0.81% | -0.80% | -0.75% | 105% | 103% |
| Class C | -0.73% | -1.16% | -0.72% | 105% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.76% | -0.77% | -0.75% | 105% | 103% |
| Class D | -0.76% | -1.25% | -0.82% | 102% | 101% |
| Class F | -0.46% | -0.75% | -0.69% | 104% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.81% | -0.41% | 0.19% | 105% | 103% |
| Class C | -0.71% | -0.72% | -0.19% | 106% | 103% |
| Class E | -0.75% | -0.13% | -0.66% | 102% | 101% |
| Overall | -0.76% | -0.44% | -0.15% | 105% | 102% |
| Class D | -0.69% | -0.13% | -0.12% | 105% | 106% |
| Class F | -0.47% | -0.36% | -0.26% | 107% | 104% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.80% | 0.13% | 0.34% | 104% | 103% |
| Class C | -0.71% | -0.60% | -0.07% | 106% | 102% |
| Class E | -0.76% | 0.10% | -0.53% | 106% | 103% |
| Overall | -0.76% | -0.12% | -0.01% | 105% | 103% |
| Class D | -0.67% | -0.42% | 0.34% | 106% | 106% |
| Class F | -0.32% | -0.70% | 0.56% | 106% | 102% |

10+14 bits:

Features may be summarized as follows:

| State representation | 10 + 14 bit logarithmic |
| --- | --- |
| rLPS computation | 32x8x8 bit table |
| Initialization | 128x16 bit to map HEVC-like state to linear representation, retrained init values |
| Rate estimation | 1024x2x18 bit table |
| Window size | Variable |
| Other | rMPS >= 128 is guaranteed |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.01% | -1.49% | -1.39% | 114% | 104% |
| Class A2 | -0.97% | -1.64% | -1.48% | 110% | 104% |
| Class B | -0.91% | -1.40% | -1.40% | 112% | 103% |
| Class C | -0.86% | -1.26% | -1.41% | 109% | 108% |
| Class E | -0.84% | -1.49% | -1.59% | 113% | 106% |
| Overall | -0.91% | -1.44% | -1.44% | 112% | 105% |
| Class D | -0.81% | -1.05% | -1.05% | 107% | 105% |
| Class F | -0.73% | -0.95% | -0.86% | 110% | 105% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.82% | -0.99% | -1.09% | 105% | 103% |
| Class A2 | -0.76% | -0.54% | -0.71% | 104% | 101% |
| Class B | -0.87% | -0.97% | -0.91% | 106% | 102% |
| Class C | -0.77% | -1.07% | -0.70% | 104% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.81% | -0.92% | -0.85% | 105% | 102% |
| Class D | -0.66% | -0.93% | -0.89% | 103% | 101% |
| Class F | -0.54% | -0.65% | -0.64% | 104% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.79% | -0.28% | 0.18% | 105% | 103% |
| Class C | -0.68% | -0.67% | -0.33% | 105% | 102% |
| Class E | -0.66% | -0.32% | -0.05% | 104% | 101% |
| Overall | -0.72% | -0.42% | -0.05% | 105% | 102% |
| Class D | -0.73% | -0.05% | -0.31% | 105% | 105% |
| Class F | -0.44% | -0.58% | -0.58% | 104% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.80% | 0.02% | 0.58% | 105% | 101% |
| Class C | -0.69% | -0.78% | -0.34% | 105% | 102% |
| Class E | -0.93% | -0.39% | -0.40% | 105% | 103% |
| Overall | -0.79% | -0.35% | 0.03% | 105% | 102% |
| Class D | -0.70% | -0.72% | -0.06% | 106% | 107% |
| Class F | -0.42% | -0.54% | -0.79% | 102% | 103% |

## 5.1.7: log state, single estimator, variable window size

Features may be summarized as follows:

| State representation | 12 bit logarithmic |
| --- | --- |
| rLPS computation | 32x8x8 bit table |
| Initialization | 128x16 bit to map HEVC-like state to linear representation, retrained init values |
| Rate estimation | 1024x2x18 bit table |
| Window size | Variable |
| Other | rMPS >= 128 is guaranteed |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.72% | -1.24% | -1.06% | 110% | 110% |
| Class A2 | -0.71% | -1.34% | -1.21% | 107% | 107% |
| Class B | -0.65% | -1.07% | -1.06% | 108% | 106% |
| Class C | -0.61% | -0.94% | -1.02% | 103% | 106% |
| Class E | -0.60% | -1.13% | -1.23% | 109% | 107% |
| Overall | -0.66% | -1.12% | -1.10% | 107% | 107% |
| Class D | -0.62% | -0.98% | -0.89% | 104% | 105% |
| Class F | -0.47% | -0.70% | -0.60% | 105% | 105% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.52% | -0.90% | -1.06% | 107% | 104% |
| Class A2 | -0.47% | -0.81% | -0.85% | 103% | 102% |
| Class B | -0.58% | -0.94% | -1.10% | 104% | 103% |
| Class C | -0.52% | -1.16% | -0.76% | 103% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.53% | -0.97% | -0.95% | 104% | 103% |
| Class D | -0.53% | -1.37% | -1.03% | 102% | 102% |
| Class F | -0.30% | -0.54% | -0.47% | 103% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.53% | -0.74% | -0.40% | 103% | 107% |
| Class C | -0.48% | -0.81% | -0.36% | 104% | 102% |
| Class E | -0.56% | -0.31% | -0.95% | 102% | 102% |
| Overall | -0.52% | -0.66% | -0.52% | 103% | 104% |
| Class D | -0.51% | -0.85% | -1.12% | 107% | 105% |
| Class F | -0.31% | -0.52% | -0.46% | 103% | 104% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.52% | -0.43% | 0.24% | 108% | 104% |
| Class C | -0.48% | -0.92% | -0.64% | 104% | 102% |
| Class E | -0.60% | -0.28% | -1.06% | 103% | 104% |
| Overall | -0.53% | -0.56% | -0.38% | 105% | 103% |
| Class D | -0.53% | -1.29% | -0.41% | 107% | 107% |
| Class F | -0.08% | -0.49% | -0.82% | 104% | 101% |

## 5.1.8: linear state, dual estimator, fixed window size, counter

Features may be summarized as follows:

| State representation | 15 + 15 bit linear, full range |
| --- | --- |
| rLPS computation | 512x64x9 bit table, computes r0 not rLPS |
| Initialization | 126x16 bit to map HEVC-like state to linear representation, unchanged init values |
| Rate estimation | 256x2x19 bit table |
| Window size | Fixed, 5-bit counter used to select between 2 window sizes |
| Other |  |

The provided implementation differs from the other implementations in CE5 in several ways (see above). It is thus difficult to assess the real impact of the added counter.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.97% | -1.37% | -1.41% | 106% | 101% |
| Class A2 | -0.91% | -1.48% | -1.50% | 105% | 101% |
| Class B | -0.87% | -1.41% | -1.37% | 106% | 100% |
| Class C | -0.81% | -1.30% | -1.40% | 107% | 106% |
| Class E | -0.76% | -1.24% | -1.15% | 109% | 104% |
| Overall | -0.86% | -1.36% | -1.37% | 106% | 102% |
| Class D | -0.69% | -1.08% | -1.12% | 105% | 105% |
| Class F | -0.69% | -0.78% | -0.71% | 102% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.74% | -0.88% | -1.08% | 103% | 100% |
| Class A2 | -0.65% | -0.59% | -0.77% | 101% | 98% |
| Class B | -0.71% | -0.88% | -0.65% | 104% | 102% |
| Class C | -0.60% | -1.01% | -0.47% | 103% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.68% | -0.86% | -0.71% | 103% | 101% |
| Class D | -0.50% | -0.95% | -0.11% | 102% | 102% |
| Class F | -0.51% | -0.37% | -0.45% | 102% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.68% | -0.92% | 1.17% | 100% | 99% |
| Class C | -0.60% | -0.57% | 0.00% | 101% | 96% |
| Class E | -0.20% | -0.10% | 0.08% | 100% | 99% |
| Overall | -0.53% | -0.60% | 0.51% | 100% | 98% |
| Class D | -0.47% | 0.30% | 1.19% | 104% | 104% |
| Class F | -0.56% | 0.10% | -0.84% | 102% | 99% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.65% | -0.29% | 1.82% | 101% | 98% |
| Class C | -0.55% | -0.91% | -0.28% | 103% | 98% |
| Class E | -0.40% | -0.90% | 0.00% | 103% | 99% |
| Overall | -0.55% | -0.65% | 0.66% | 102% | 98% |
| Class D | -0.45% | 0.13% | 1.77% | 105% | 106% |
| Class F | -0.52% | -0.79% | -0.37% | 99% | 100% |

## 5.1.10: linear state, dual estimator, fixed window size

| State representation | 10 + 14 bit linear, reduced range |
| --- | --- |
| rLPS computation | 32x8x8 bit table, same as 5.1.1/2 |
| Initialization | 128x16 bit to map HEVC-like state to linear representation, retrained init values |
| Rate estimation | 256x2x19 bit table |
| Window size | Fixed |
| Other | rMPS >= 128 is guaranteed |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.79% | -1.29% | -1.13% | 108% | 104% |
| Class A2 | -0.83% | -1.32% | -1.28% | 105% | 104% |
| Class B | -0.78% | -0.94% | -1.11% | 109% | 102% |
| Class C | -0.80% | -0.96% | -1.10% | 108% | 106% |
| Class E | -0.78% | -1.17% | -1.25% | 108% | 105% |
| Overall | -0.79% | -1.10% | -1.16% | 108% | 104% |
| Class D | -0.78% | -1.00% | -0.86% | 105% | 105% |
| Class F | -0.67% | -0.73% | -0.65% | 108% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.58% | -1.27% | -1.29% | 104% | 103% |
| Class A2 | -0.70% | -0.93% | -0.93% | 102% | 99% |
| Class B | -0.68% | -0.90% | -1.08% | 104% | 102% |
| Class C | -0.69% | -1.20% | -0.82% | 105% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.67% | -1.06% | -1.02% | 104% | 102% |
| Class D | -0.65% | -1.47% | -0.99% | 104% | 103% |
| Class F | -0.43% | -0.72% | -0.66% | 105% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.56% | -0.73% | -0.86% | 102% | 102% |
| Class C | -0.54% | -0.58% | -0.33% | 104% | 100% |
| Class E | -0.52% | 0.47% | -1.26% | 105% | 101% |
| Overall | -0.54% | -0.38% | -0.78% | 103% | 101% |
| Class D | -0.55% | -0.34% | -0.05% | 104% | 103% |
| Class F | 0.01% | 0.33% | 0.03% | 101% | 102% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.58% | -0.36% | -0.68% | 105% | 99% |
| Class C | -0.60% | -0.65% | -0.70% | 102% | 100% |
| Class E | -0.66% | 0.25% | -1.46% | 103% | 101% |
| Overall | -0.61% | -0.30% | -0.88% | 103% | 100% |
| Class D | -0.55% | -0.80% | -1.06% | 106% | 104% |
| Class F | -0.09% | -0.67% | -0.45% | 104% | 101% |

## 5.1.11: linear state, dual estimator, variable window size

| State representation | 10 + 14 bit linear, reduced range |
| --- | --- |
| rLPS computation | 32x8x8 bit table, same as 5.1.1/2 |
| Initialization | 128x16 bit to map HEVC-like state to linear representation, retrained init values |
| Rate estimation | 256x2x19 bit table |
| Window size | Variable, same as 5.1.2 |
| Other | rMPS >= 128 is guaranteed |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.96% | -1.37% | -1.13% | 111% | 102% |
| Class A2 | -0.99% | -1.35% | -1.29% | 107% | 102% |
| Class B | -0.93% | -0.98% | -1.18% | 108% | 100% |
| Class C | -0.92% | -0.97% | -1.19% | 106% | 106% |
| Class E | -0.94% | -1.22% | -1.38% | 109% | 104% |
| Overall | -0.94% | -1.14% | -1.23% | 108% | 103% |
| Class D | -0.87% | -1.09% | -1.08% | 105% | 102% |
| Class F | -0.78% | -0.84% | -0.83% | 107% | 105% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.89% | -1.10% | -1.23% | 102% | 101% |
| Class A2 | -0.92% | -0.76% | -0.84% | 103% | 99% |
| Class B | -0.96% | -0.87% | -1.09% | 104% | 101% |
| Class C | -0.92% | -1.09% | -1.02% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.93% | -0.95% | -1.05% | 103% | 101% |
| Class D | -0.86% | -1.24% | -1.06% | 102% | 101% |
| Class F | -0.61% | -0.66% | -0.86% | 104% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.80% | -0.20% | 0.06% | 100% | 102% |
| Class C | -0.78% | -0.21% | -0.30% | 104% | 100% |
| Class E | -0.84% | -0.45% | -1.20% | 101% | 99% |
| Overall | -0.80% | -0.27% | -0.38% | 102% | 101% |
| Class D | -0.77% | -0.49% | -0.22% | 107% | 107% |
| Class F | -0.30% | 0.07% | -0.63% | 104% | 102% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.81% | 0.12% | 0.36% | 105% | 101% |
| Class C | -0.78% | -0.32% | -0.46% | 105% | 101% |
| Class E | -0.90% | -0.27% | -1.34% | 106% | 103% |
| Overall | -0.82% | -0.12% | -0.34% | 105% | 102% |
| Class D | -0.74% | -0.24% | 0.11% | 105% | 105% |
| Class F | -0.25% | -0.35% | -0.12% | 105% | 101% |

## 5.1.12: linear state, single estimator, variable window size

| State representation | 14 bit linear, reduced range |
| --- | --- |
| rLPS computation | 32x8x8 bit table, same as 5.1.1/2 |
| Initialization | 128x16 bit to map HEVC-like state to linear representation, retrained init values |
| Rate estimation | 256x2x19 bit table |
| Window size | Variable 0..15 |
| Other | rMPS >= 128 is guaranteed |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.72% | -0.52% | -0.31% | 109% | 102% |
| Class A2 | -0.71% | -0.63% | -0.44% | 107% | 104% |
| Class B | -0.66% | -0.79% | -0.49% | 107% | 102% |
| Class C | -0.64% | -0.90% | -0.70% | 108% | 106% |
| Class E | -0.57% | -0.80% | -0.49% | 109% | 103% |
| Overall | -0.66% | -0.74% | -0.50% | 108% | 103% |
| Class D | -0.63% | -0.87% | -0.66% | 105% | 103% |
| Class F | -0.44% | -0.37% | -0.21% | 107% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.56% | 0.22% | 0.61% | 104% | 100% |
| Class A2 | -0.45% | 0.75% | 1.18% | 99% | 99% |
| Class B | -0.55% | 0.27% | 1.20% | 102% | 99% |
| Class C | -0.61% | -0.40% | 0.69% | 104% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.55% | 0.18% | 0.94% | 102% | 100% |
| Class D | -0.66% | -0.98% | -0.01% | 103% | 101% |
| Class F | -0.35% | -0.06% | 0.24% | 103% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.56% | 1.02% | 3.50% | 102% | 102% |
| Class C | -0.59% | -0.01% | 1.76% | 101% | 97% |
| Class E | -0.45% | 1.54% | 1.14% | 101% | 98% |
| Overall | -0.54% | 0.81% | 2.33% | 101% | 99% |
| Class D | -0.70% | 0.98% | 2.13% | 103% | 101% |
| Class F | -0.41% | 0.04% | 0.05% | 101% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.48% | 1.67% | 4.37% | 104% | 101% |
| Class C | -0.55% | 0.10% | 1.66% | 102% | 98% |
| Class E | -0.41% | 1.81% | 1.12% | 103% | 100% |
| Overall | -0.49% | 1.18% | 2.66% | 103% | 100% |
| Class D | -0.58% | 0.81% | 1.87% | 105% | 104% |
| Class F | -0.34% | -0.22% | 0.77% | 104% | 100% |

## 5.1.13: linear state, dual estimator, variable window size

| State representation | 10+14 bit linear, reduced range |
| --- | --- |
| rLPS computation | 5x4 bit multiplier |
| Initialization | 128x16 bit to map HEVC-like state to linear representation, retrained init values |
| Rate estimation | 256x2x19 bit table |
| Window size | Variable, same as 5.1.2 |
| Other | rMPS >= 128 is guaranteed |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.94% | -1.35% | -1.11% | 109% | 103% |
| Class A2 | -0.97% | -1.33% | -1.27% | 108% | 103% |
| Class B | -0.92% | -0.97% | -1.16% | 108% | 100% |
| Class C | -0.90% | -0.95% | -1.17% | 108% | 107% |
| Class E | -0.93% | -1.21% | -1.37% | 110% | 104% |
| Overall | -0.93% | -1.13% | -1.21% | 109% | 103% |
| Class D | -0.85% | -1.08% | -1.07% | 107% | 106% |
| Class F | -0.76% | -0.82% | -0.81% | 108% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.87% | -1.09% | -1.22% | 106% | 100% |
| Class A2 | -0.90% | -0.74% | -0.82% | 101% | 97% |
| Class B | -0.94% | -0.85% | -1.07% | 104% | 101% |
| Class C | -0.89% | -1.07% | -0.99% | 104% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.91% | -0.93% | -1.03% | 104% | 101% |
| Class D | -0.84% | -1.22% | -1.04% | 103% | 102% |
| Class F | -0.59% | -0.65% | -0.84% | 104% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.78% | -0.18% | 0.07% | 101% | 102% |
| Class C | -0.75% | -0.18% | -0.27% | 103% | 97% |
| Class E | -0.82% | -0.42% | -1.18% | 99% | 95% |
| Overall | -0.78% | -0.24% | -0.35% | 101% | 98% |
| Class D | -0.75% | -0.47% | -0.20% | 105% | 104% |
| Class F | -0.29% | 0.08% | -0.61% | 103% | 98% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.79% | 0.14% | 0.38% | 104% | 100% |
| Class C | -0.75% | -0.29% | -0.43% | 101% | 95% |
| Class E | -0.88% | -0.24% | -1.32% | 105% | 98% |
| Overall | -0.80% | -0.10% | -0.31% | 103% | 98% |
| Class D | -0.72% | -0.22% | 0.13% | 106% | 106% |
| Class F | -0.24% | -0.33% | -0.11% | 103% | 98% |

## Additional variants

Additional variants were tested, including where the 4x5 multiplication-based rLPS computation (5.1.13) is replaced by a higher-precision 5x6 multiplier.

# SW throughput experiments

The following table reports the decoder throughput from an actual bin stream (first 10 million context-coded bins from ParkRunning, RA configuration, QP22). Three compilers have been tested: clang 10.0 (Apple LLVM version 10.0.0), gcc 7.4 (Homebrew GCC 7.4.0) and gcc 8.2 (Homebrew GCC 8.2.0).  The test bed was configured with the macro NOBRANCH_MPS either on (left half of table) or off (right half of table). Results were obtained on an Intel® Xeon® W-2140B CPU @ 3.20GHz.

In almost all cases the combination of clang and NOBRANCH_MPS enabled yielded the highest throughput. This configuration is thus considered for comparing the various engines.

The rightmost column in the table below reports the decoder throughput for a different bin stream (first 10 million context-coded bins from BQTerrace, RA configuration, QP22, clang, NOBRANCH_MPS enabled).

| Engine | clang | gcc 7 | gcc 8 | clang | gcc 7 | gcc 8 | clang stream #2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AVC/HEVC CABAC | 132.03 | 134.92 | 128.64 | 127.27 | 121.13 | 124.98 | 137.88 |
| CE5.1.1 | 120.17 | 117.48 | 118.58 | 113.54 | 108.52 | 109.26 | 121.51 |
| CE5.1.2 | 115.00 | 110.93 | 113.28 | 104.01 | 108.47 | 102.20 | 114.67 |
| CE5.1.3 | 106.63 | 104.98 | 104.60 | 95.89 | 96.84 | 96.37 | 102.79 |
| CE5.1.4 | 109.09 | 106.68 | 107.28 | 104.77 | 104.45 | 104.77 | 110.55 |
| CE5.1.4 mult | 134.85 | 133.09 | 131.12 | 120.09 | 116.32 | 117.72 | 135.13 |
| CE5.1.5 | 113.46 | 106.75 | 107.56 | 111.67 | 102.44 | 104.75 | 113.02 |
| CE5.1.6 (8+12 bit) | 110.75 | 105.07 | 103.62 | 103.64 | 98.89 | 100.07 | 108.50 |
| CE5.1.7 | 119.00 | 110.98 | 112.45 | 118.51 | 110.15 | 107.84 | 120.23 |
| CE5.1.8 | 118.80 | 114.27 | 107.99 | 107.71 | 103.96 | 100.47 | 119.39 |
| CE5.1.9 / CE 5.1.10 | 125.05 | 122.68 | 120.02 | 120.47 | 113.33 | 115.96 | 127.29 |
| CE5.1.11 | 121.79 | 117.33 | 117.14 | 107.25 | 113.08 | 111.22 | 123.35 |
| CE5.1.12 | 128.95 | 123.80 | 127.78 | 118.60 | 119.66 | 120.22 | 129.01 |
| CE5.1.13 / CE5.1.3 mult | 127.28 | 121.00 | 122.57 | 109.90 | 109.25 | 110.67 | 127.32 |

Engines with a log state representation appear to have lower throughput than those with a linear state representation. Consider for instance 5.1.5 (113), 5.1.6 (111) and 5.1.7 (119) versus 5.1.10 (125), 5.1.11 (122) and 5.1.12 (129).

Using a restricted range for the state representation appears to increase throughput. Consider for instance 5.1.1 (120) and 5.1.2 (115) that use a full range versus 5.1.10 (125) and 5.1.11 (122) that use a restricted range.

Using a multiplier instead of a lookup table appears to increase throughput. Consider for instance 5.1.11 (122) that uses a lookup table versus 5.1.13 (127) that uses a multiplier.

## Macro settings

Macros in the test bed were set as follows for each experiment.

| Engine | ENGINE |  | FULL_RANGE |  | USE_RLPS_TABLE |  | LOG_MULT |  |  | SINGLE |  | COUNTER |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AVC / HEVC | 0 |  |  |  |  |  |  |  |  |  |  |  |  |
| CE5.1.1 | 4 |  | 1 |  | 1 |  |  |  |  |  |  |  |  |
| CE5.1.2 | 2 |  | 1 |  | 1 |  |  |  |  |  |  |  |  |
| CE5.1.3 | 2 |  | 1 |  |  |  | 1 |  |  |  |  |  |  |
| CE5.1.4 | 2 |  | 1 |  |  |  | 1 |  |  | 1 |  |  |  |
| CE5.1.4 mult | 2 | 0 | 0 |  | 1 |  |
| CE5.1.5 | 3 |  |  |  |  |  |  |  |  |  |  |  |  |
| CE5.1.6 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| CE5.1.7 | 1 |  |  |  |  |  |  |  |  | 1 |  |  |  |
| CE5.1.8 | 4 |  | 1 |  | 1 |  |  |  |  |  |  | 1 |  |
| CE5.1.9 / CE5.1.10 | 4 | 0 | 1 |  |  |  |
| CE5.1.11 | 2 | 0 | 1 |  |  |  |
| CE5.1.12 | 2 | 0 | 1 |  | 1 |  |
| CE5.1.13 / CE5.1.3 mult | 2 | 0 | 0 |  |  |  |

## Fast MPS path

The NOBRANCH_MPS macro controls the use of one of two implementations:

- When disabled, a fast MPS path is present where renormalization can be skipped in some cases

- When enabled, a single code path is used to process MPS and LPS symbols

As shown in the figure below, having a fast MPS path yields much higher throughputs for highly skewed sources (i.e., the Bernoulli parameter is close to 0 or 1). However the throughput for sequences with Bernoulli parameters close to 0.5 is lower.

Having a single path yields throughputs that are less dependent on the Bernoulli parameter. When considering real bin streams, the single path method tends to yield higher throughput numbers, which is why it is generally preferred.

## Multiple runs

Experiments were run a second time, where each test is done 7 times and the highest throughput value is recorded. While the numbers are slightly higher, the same trends persist.

| Engine | clang | gcc 7 | gcc 8 | clang | gcc 7 | gcc 8 | clang stream #2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AVC/HEVC CABAC | 137.27 | 137.83 | 131.01 | 131.69 | 124.68 | 127.93 | 141.37 |
| CE5.1.1 | 124.17 | 121.64 | 120.26 | 117.41 | 112.53 | 110.42 | 124.48 |
| CE5.1.2 | 116.69 | 115.36 | 116.85 | 106.22 | 110.39 | 109.24 | 115.93 |
| CE5.1.3 | 107.96 | 106.13 | 105.99 | 96.69 | 98.59 | 97.68 | 108.09 |
| CE5.1.4 | 111.83 | 109.14 | 109.93 | 106.80 | 106.21 | 106.51 | 112.09 |
| CE5.1.4 mult | 136.30 | 133.96 | 132.36 | 120.85 | 116.32 | 121.01 | 137.27 |
| CE5.1.5 | 115.84 | 109.11 | 108.83 | 114.71 | 105.29 | 105.21 | 115.73 |
| CE5.1.6 (8+12 bit) | 112.69 | 106.88 | 103.99 | 105.00 | 102.27 | 102.21 | 112.68 |
| CE5.1.7 | 120.75 | 112.94 | 113.24 | 119.12 | 106.58 | 114.97 | 120.89 |
| CE5.1.8 | 122.01 | 116.04 | 108.10 | 110.09 | 107.50 | 102.54 | 125.28 |
| CE5.1.9 / CE 5.1.10 | 128.67 | 124.20 | 121.22 | 122.22 | 119.20 | 117.55 | 129.44 |
| CE5.1.11 | 124.55 | 118.73 | 119.37 | 114.53 | 114.48 | 112.39 | 125.28 |
| CE5.1.12 | 130.26 | 128.17 | 130.77 | 121.13 | 121.84 | 120.45 | 131.45 |
| CE5.1.13 / CE5.1.3 mult | 128.51 | 122.63 | 126.85 | 112.10 | 111.23 | 112.96 | 128.87 |

## CPU dependency

The same experiments were run on an older and less powerful Intel® Core™ i7-4870HQ CPU @ 2.50GHz. Higher variations are observed between various compilers. However the general ranking of engines appears to be consistent with previous results.

| Engine | clang | gcc 7 | gcc 8 | clang | gcc 7 | gcc 8 | clang stream #2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AVC/HEVC CABAC | 123.60 | 118.56 | 112.78 | 115.07 | 112.83 | 109.77 | 126.06 |
| CE5.1.1 | 103.17 | 103.28 | 102.67 | 103.01 | 104.22 | 101.28 | 103.90 |
| CE5.1.2 | 101.29 | 103.49 | 104.73 | 96.19 | 96.87 | 95.47 | 102.08 |
| CE5.1.3 | 98.81 | 97.10 | 93.43 | 87.09 | 90.29 | 88.87 | 99.27 |
| CE5.1.4 | 101.28 | 97.68 | 97.92 | 96.47 | 98.29 | 99.19 | 101.93 |
| CE5.1.4 mult | 120.07 | 118.56 | 117.99 | 112.33 | 113.09 | 108.35 | 121.06 |
| CE5.1.5 | 86.50 | 96.59 | 94.41 | 98.46 | 94.72 | 93.51 | 87.25 |
| CE5.1.6 (8+12 bit) | 83.31 | 97.79 | 94.63 | 90.42 | 91.35 | 90.75 | 83.62 |
| CE5.1.7 | 99.32 | 98.97 | 99.48 | 103.73 | 100.73 | 101.38 | 100.48 |
| CE5.1.8 | 93.76 | 103.73 | 96.21 | 98.35 | 95.57 | 90.27 | 95.14 |
| CE5.1.9 / CE 5.1.10 | 113.36 | 105.09 | 105.78 | 108.90 | 108.86 | 107.02 | 114.89 |
| CE5.1.11 | 106.43 | 104.52 | 101.70 | 102.67 | 103.25 | 101.43 | 107.21 |
| CE5.1.12 | 110.81 | 111.62 | 112.23 | 109.65 | 113.12 | 106.99 | 111.83 |
| CE5.1.13 / CE5.1.3 mult | 115.09 | 114.80 | 111.37 | 101.96 | 103.01 | 102.23 | 115.87 |

## Entropy

In addition to reporting throughput, the test bed also reports compression efficiency in terms of ratio to the entropy. For tests that use a Bernoulli source (parameters 0.1 to 0.9) the ratio is always larger than 1.0 for obvious theoretical reasons. For an actual stream the ratio is lower than 1.0, because the entropy computation doesn’t account for context modeling and nonergodicity of the source.

One noticeable feature in the results below is that the engines considered in the CE do much better than the AVC / HEVC, in particular for highly skewed Bernoulli sources (e.g., 0.1 and 0.9).

| Engine | 0.1 | 0.2 | 0.3 | 0.4 | 0.5 | 0.6 | 0.7 | 0.8 | 0.9 | actual |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AVC / HEVC | 1.046 | 1.029 | 1.025 | 1.021 | 1.016 | 1.021 | 1.025 | 1.029 | 1.046 | 0.615 |
| CE5.1.1 | 1.018 | 1.013 | 1.010 | 1.010 | 1.014 | 1.010 | 1.010 | 1.012 | 1.018 | 0.610 |
| CE5.1.2 | 1.018 | 1.013 | 1.010 | 1.010 | 1.014 | 1.010 | 1.010 | 1.012 | 1.018 | 0.610 |
| CE5.1.3 | 1.020 | 1.013 | 1.010 | 1.010 | 1.008 | 1.010 | 1.011 | 1.013 | 1.020 | 0.610 |
| CE5.1.4 | 1.009 | 1.005 | 1.004 | 1.004 | 1.002 | 1.004 | 1.004 | 1.005 | 1.009 | 0.616 |
| CE5.1.4 mult | 1.008 | 1.005 | 1.004 | 1.004 | 1.007 | 1.004 | 1.004 | 1.005 | 1.008 | 0.616 |
| CE5.1.5 | 1.024 | 1.014 | 1.012 | 1.021 | 1.026 | 1.012 | 1.013 | 1.015 | 1.024 | 0.612 |
| CE5.1.6 | 1.024 | 1.014 | 1.012 | 1.021 | 1.026 | 1.012 | 1.013 | 1.015 | 1.024 | 0.612 |
| CE5.1.7 | 1.016 | 1.006 | 1.005 | 1.013 | 1.015 | 1.003 | 1.007 | 1.007 | 1.011 | 0.619 |
| CE5.1.8 | 1.018 | 1.012 | 1.010 | 1.010 | 1.014 | 1.010 | 1.010 | 1.012 | 1.018 | 0.610 |
| CE5.1.9 / CE5.1.10 | 1.018 | 1.012 | 1.010 | 1.010 | 1.014 | 1.010 | 1.010 | 1.012 | 1.018 | 0.610 |
| CE5.1.11 | 1.018 | 1.012 | 1.010 | 1.010 | 1.014 | 1.010 | 1.010 | 1.012 | 1.018 | 0.610 |
| CE5.1.12 | 1.008 | 1.005 | 1.004 | 1.004 | 1.007 | 1.004 | 1.004 | 1.005 | 1.008 | 0.616 |
| CE5.1.13 / CE5.1.3 mult | 1.018 | 1.012 | 1.010 | 1.010 | 1.014 | 1.010 | 1.010 | 1.012 | 1.018 | 0.610 |

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).