The following is based on text from JCTVC-L1003-v34. It includes changes relevant to test 5.1.13.

## CABAC parsing process for slice segment data

### General

This process is invoked when parsing syntax elements with descriptor ae(v) in subclauses 7.3.8.1 through 7.3.8.11.

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

The initialization process of the CABAC parsing process as specified in subclause 9.3.2 is invoked when starting the parsing of one or more of the following:

    - the slice segment data syntax specified in subclause 7.3.8.1

    - the coding tree unit syntax specified in subclause 7.3.8.2 and the coding tree unit is the first coding tree unit in a tile

    - the coding tree unit syntax specified in subclause 7.3.8.2, entropy_coding_sync_enabled_flag is equal to 1, and the associated luma coding tree block is the first luma coding tree block in a coding tree unit row

The parsing of syntax elements proceeds as follows:

For each requested value of a syntax element a binarization is derived as specified in subclause 9.3.3.

The binarization for the syntax element and the sequence of parsed bins determines the decoding process flow as described in subclause 9.3.4.

In case the request for a value of a syntax element is processed for the syntax element pcm_flag and the decoded value of pcm_flag is equal to 1, the decoding engine is initialized after the decoding of any pcm_alignment_zero_bit and all pcm_sample_luma and pcm_sample_chroma data as specified in subclause 9.3.2.5.

The storage process for context variables is applied as follows:

  - When ending the parsing of the coding tree unit syntax in subclause 7.3.8.2, entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrInRs % PicWidthInCtbsY is equal to 1, the storage process for context variables as specified in subclause 9.3.2.3 is invoked with TableStateIdxWpp and TableMpsValWpp as outputs.

  - When ending the parsing of the general slice segment data syntax in subclause 7.3.8.1, dependent_slice_segments_enabled_flag is equal to 1 and end_of_slice_segment_flag is equal to 1, the storage process for context variables as specified in subclause 9.3.2.3 is invoked with TableStateIdxDs and TableMpsValDs as outputs.

The whole CABAC parsing process for a syntax element synEl is illustrated in Figure 9-1.

Figure 9-1 – Illustration of CABAC parsing process for a syntax element synEl (informative)

### Initialization process

#### General

Outputs of this process are initialized CABAC internal variables.

Figure 9-2 – Spatial neighbour T that is used to invoke the coding tree block availability derivation process relative to the current coding tree block (informative)

The context variables of the arithmetic decoding engine are initialized as follows:

  - If the coding tree unit is the first coding tree unit in a tile, the initialization process for context variables is invoked as specified in subclause 9.3.2.2.

  - Otherwise, if entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrInRs % PicWidthInCtbsY is equal to 0, the following applies:

  - The location ( xNbT, yNbT ) of the top-left luma sample of the spatial neighbouring block T (Figure 9-2) is derived using the location ( x0, y0 ) of the top-left luma sample of the current coding tree block as follows:

( xNbT, yNbT ) = ( x0 + CtbSizeY, y0 − CtbSizeY )	(9-3)

  - The availability derivation process for a block in z-scan order as specified in subclause 6.4.1 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbT, yNbT ) as inputs, and the output is assigned to availableFlagT.

  - The synchronization process for context variables is invoked as follows:

  - If availableFlagT is equal to 1, the synchronization process for context variables as specified in subclause 9.3.2.4 is invoked with TableStateIdxWpp and TableMpsValWpp as inputs.

  - Otherwise, the initialization process for context variables is invoked as specified in subclause 9.3.2.2.

  - Otherwise, if CtbAddrInRs is equal to slice_segment_address and dependent_slice_segment_flag is equal to 1, the synchronization process for context variables as specified in subclause 9.3.2.4 is invoked with TableStateIdxDs and TableMpsValDs as inputs.

  - Otherwise, the initialization process for context variables is invoked as specified in subclause 9.3.2.2.

The initialization process for the arithmetic decoding engine is invoked as specified in subclause 9.3.2.5.

The whole initialization process for a syntax element synEl is illustrated in the flowchart of Figure 9-3.

Figure 9-3 – Illustration of CABAC initialization process (informative)

#### Initialization process for context variables

Outputs of this process are the initialized CABAC context variables indexed by ctxTable and ctxIdx.

Table 9-5 to Table 9-31 contain the values of the 8 bit variable initValue used in the initialization of context variables that are assigned to all syntax elements in subclauses 7.3.8.1 through 7.3.8.11, except end_of_slice_segment_flag, end_of_sub_stream_one_bit, and pcm_flag.

For each context variable, the two variables pStateIdx and valMps are initialized.

NOTE 1 – The variable pStateIdx corresponds to a probability state index and the variable valMps corresponds to the value of the most probable symbol as further described in subclause 9.3.4.3.

From the 8 bit table entry initValue, the two 4 bit variables slopeIdx and offsetIdx are derived as follows:

slopeIdx = initValue  >>  4
offsetIdx = initValue & 15	(9-4)

The variables m and n, used in the initialization of context variables, are derived from slopeIdx and offsetIdx as follows:

m = slopeIdx * 5 − 45
n = ( offsetIdx  <<  3 ) − 16	(9-5)

The two values assigned to pStateIdx0 and pStateIdx1 for the initialization are derived from SliceQpY, which is derived in Equation 7-40. Given the variables m and n, the initialization is specified as follows:

preCtxState = Clip3( 0, 127, ( ( m * Clip3( 0, 51, SliceQpY ) )  >>  4 ) + n )
pStateIdx0 = initStateIdxToState[preCtxState] >> 4
pStateIdx1 = initStateIdxToState[preCtxState]	(9-6)

The values of initStateIdxToState are defined in Table x-xx.

Table x-xx – Initial state mapping table

| preCtxState | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pStateIdx1 | 307 | 323 | 340 | 359 | 378 | 398 | 419 | 442 | 466 | 491 | 517 | 544 | 574 | 604 | 637 | 671 |
| preCtxState | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| pStateIdx1 | 707 | 745 | 784 | 826 | 871 | 917 | 966 | 1018 | 1073 | 1130 | 1191 | 1254 | 1321 | 1392 | 1467 | 1545 |
| preCtxState | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| pStateIdx1 | 1628 | 1715 | 1807 | 1903 | 2005 | 2112 | 2226 | 2345 | 2470 | 2602 | 2741 | 2888 | 3043 | 3206 | 3377 | 3558 |
| preCtxState | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| pStateIdx1 | 3748 | 3949 | 4160 | 4383 | 4617 | 4864 | 5124 | 5399 | 5687 | 5992 | 6312 | 6650 | 7006 | 7381 | 7775 | 8192 |
| preCtxState | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| pStateIdx1 | 8192 | 8608 | 9002 | 9377 | 9733 | 10071 | 10391 | 10696 | 10984 | 11259 | 11519 | 11766 | 12000 | 12223 | 12434 | 12635 |
| preCtxState | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 |
| pStateIdx1 | 12825 | 13006 | 13177 | 13340 | 13495 | 13642 | 13781 | 13913 | 14038 | 14157 | 14271 | 14378 | 14480 | 14576 | 14668 | 14755 |
| preCtxState | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 |
| pStateIdx1 | 14838 | 14916 | 14991 | 15062 | 15129 | 15192 | 15253 | 15310 | 15365 | 15417 | 15466 | 15512 | 15557 | 15599 | 15638 | 15676 |
| preCtxState | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 |
| pStateIdx1 | 15712 | 15746 | 15779 | 15809 | 15839 | 15866 | 15892 | 15917 | 15941 | 15964 | 15985 | 16005 | 16024 | 16043 | 16060 | 16076 |

In Table 9-4, the ctxIdx for which initialization is needed for each of the three initialization types, specified by the variable initType, are listed. Also listed is the table number that includes the values of initValue needed for the initialization. For P and B slice types, the derivation of initType depends on the value of the cabac_init_flag syntax element. The variable initType is derived as follows:

if( slice_type  = =  I )
	initType = 0
else if( slice_type  = =  P )
	initType = cabac_init_flag ? 2 : 1	(9-7)
else
	initType = cabac_init_flag ? 1 : 2

#### Initialization process for the arithmetic decoding engine

Outputs of this process are the initialized decoding engine registers ivlCurrRange and ivlOffset both in 16 bit register precision.

The status of the arithmetic decoding engine is represented by the variables ivlCurrRange and ivlOffset. In the initialization procedure of the arithmetic decoding process, ivlCurrRange is set equal to 510 and ivlOffset is set equal to the value returned from read_bits( 9 ) interpreted as a 9 bit binary representation of an unsigned integer with the most significant bit written first.

The bitstream shall not contain data that result in a value of ivlOffset being equal to 510 or 511.

NOTE – The description of the arithmetic decoding engine in this Specification utilizes 16 bit register precision. However, a minimum register precision of 9 bits is required for storing the values of the variables ivlCurrRange and ivlOffset after invocation of the arithmetic decoding process (DecodeBin) as specified in subclause 9.3.4.3. The arithmetic decoding process for a binary decision (DecodeDecision) as specified in subclause 9.3.4.3.2 and the decoding process for a binary decision before termination (DecodeTerminate) as specified in subclause 9.3.4.3.5 require a minimum register precision of 9 bits for the variables ivlCurrRange and ivlOffset. The bypass decoding process for binary decisions (DecodeBypass) as specified in subclause 9.3.4.3.4 requires a minimum register precision of 10 bits for the variable ivlOffset and a minimum register precision of 9 bits for the variable ivlCurrRange.

#### Arithmetic decoding process

##### General

Inputs to this process are ctxTable, ctxIdx, and bypassFlag, as derived in subclause 9.3.4.2, and the state variables ivlCurrRange and ivlOffset of the arithmetic decoding engine.

Output of this process is the value of the bin.

Figure 9-5 illustrates the whole arithmetic decoding process for a single bin. For decoding the value of a bin, the context index table ctxTable and the ctxIdx are passed to the arithmetic decoding process DecodeBin( ctxTable, ctxIdx ), which is specified as follows:

–	If bypassFlag is equal to 1, DecodeBypass( ) as specified in subclause 9.3.4.3.4 is invoked.

–	Otherwise, if bypassFlag is equal to 0, ctxTable is equal to 0, and ctxIdx is equal to 0, DecodeTerminate( ) as specified in subclause 9.3.4.3.5 is invoked.

–	Otherwise (bypassFlag is equal to 0 and ctxTable is not equal to 0), DecodeDecision( ) as specified in subclause 9.3.4.3.2 is invoked.

Figure 9-5 – Overview of the arithmetic decoding process for a single bin (informative)

NOTE – Arithmetic coding is based on the principle of recursive interval subdivision. Given a probability estimation p( 0 ) and p( 1 ) = 1  p( 0 ) of a binary decision ( 0, 1 ), an initially given code sub-interval with the range ivlCurrRange will be subdivided into two sub-intervals having range p( 0 ) * ivlCurrRange and ivlCurrRange  p( 0 ) * ivlCurrRange, respectively. Depending on the decision, which has been observed, the corresponding sub-interval will be chosen as the new code interval, and a binary code string pointing into that interval will represent the sequence of observed binary decisions. It is useful to distinguish between the most probable symbol (MPS) and the least probable symbol (LPS), so that binary decisions have to be identified as either MPS or LPS, rather than 0 or 1. Given this terminology, each context is specified by the probability pLPS of the LPS and the value of MPS (valMps), which is either 0 or 1. The arithmetic core engine in this Specification has three distinct properties:

–	The probability estimation is performed by means of a finite-state machine with a table-based transition process between 64 different representative probability states { pLPS( pStateIdx ) | 0  <=  pStateIdx < 64 } for the LPS probability pLPS. The numbering of the states is arranged in such a way that the probability state with index pStateIdx = 0 corresponds to an LPS probability value of 0.5, with decreasing LPS probability towards higher state indices.

–	The range ivlCurrRange representing the state of the coding engine is quantized to a small set {Q1,…,Q4} of pre-set quantization values prior to the calculation of the new interval range. Storing a table containing all 64x4 pre-computed product values of Qi * pLPS( pStateIdx ) allows a multiplication-free approximation of the product ivlCurrRange * pLPS( pStateIdx ).

–	For syntax elements or parts thereof for which an approximately uniform probability distribution is assumed to be given a separate simplified encoding and decoding bypass process is used.

##### Arithmetic decoding process for a binary decision

###### General

Inputs to this process are the variables ctxTable, ctxIdx, ivlCurrRange, and ivlOffset.

Outputs of this process are the decoded value binVal, and the updated variables ivlCurrRange and ivlOffset.

Figure 9-6 shows the flowchart for decoding a single decision (DecodeDecision):

- The value of the variable ivlLpsRange is derived as follows:

–	Given the current value of ivlCurrRange, the variable qRangeIdx is derived as follows:

qRangeIdx = ivlCurrRange  >>  5		(9-45)

–	Given qRangeIdx, pStateIdx0 and pStateIdx1 associated with ctxTable and ctxIdx, valMps and -ivlLpsRange are derived as follows:

pState = pState1 + 16 * pState0
valMps = pState >> 14
ivlLpsRange = ( qRangeIdx * ( (valMps ?  32767 - pState : pState ) >> 9 ) >> 1 ) + 4	(9-46)

- The variable ivlCurrRange is set equal to ivlCurrRange  ivlLpsRange and the following applies:

–	If ivlOffset is greater than or equal to ivlCurrRange, the variable binVal is set equal to 1 − valMps, ivlOffset is decremented by ivlCurrRange, and ivlCurrRange is set equal to ivlLpsRange.

–	Otherwise, the variable binVal is set equal to valMps.

Given the value of binVal, the state transition is performed as specified in subclause 9.3.4.3.2.2. Depending on the current value of ivlCurrRange, renormalization is performed as specified in subclause 9.3.4.3.3.

###### State transition process

Inputs to this process are the current pStateIdx0 and pStateIdx1, and the decoded value binVal.

Outputs of this process are the updated pStateIdx0 and pStateIdx1 of the context variable associated with ctxIdx.

The variables shift0 and shift1 are derived from the shiftIdx value associated with ctxTable and ctxInc.

shift0 = (shiftIdx >> 2) + 2 
shift1 = (shiftIdx & 3) + 3 + shift0		(x-xx)

Tables x-xx to x-xx define the values of shiftIdx.

Depending on the decoded value binVal, the update of the two variables pStateIdx0 and pStateIdx1 associated with ctxIdx is derived as follows:

pStateIdx0 = pStateIdx0  (pStateIdx0 >> shift0) + (1023 * binVal >> shift0)
pStateIdx1 = pStateIdx1  (pStateIdx1 >> shift1) + (16383 * binVal >> shift1)	(x-xx)
	
				-

-

Note: needs to be updated

Figure 9-6 – Flowchart for decoding a decision

-

-

##### Renormalization process in the arithmetic decoding engine

Inputs to this process are bits from slice segment data and the variables ivlCurrRange and ivlOffset.

Outputs of this process are the updated variables ivlCurrRange and ivlOffset.

A flowchart of the renormalization is shown in Figure 9-7. The current value of ivlCurrRange is first compared to 256 and further steps are specified as follows:

–	If ivlCurrRange is greater than or equal to 256, no renormalization is needed and the RenormD process is finished;

–	Otherwise (ivlCurrRange is less than 256), the renormalization loop is entered. Within this loop, the value of ivlCurrRange is doubled, i.e. left-shifted by 1 and a single bit is shifted into ivlOffset by using read_bits( 1 ).

The bitstream shall not contain data that result in a value of ivlOffset being greater than or equal to ivlCurrRange upon completion of this process.

Figure 9-7 – Flowchart of renormalization

##### Bypass decoding process for binary decisions

Inputs to this process are bits from slice segment data and the variables ivlCurrRange and ivlOffset.

Outputs of this process are the updated variable ivlOffset and the decoded value binVal.

The bypass decoding process is invoked when bypassFlag is equal to 1. Figure 9-8 shows a flowchart of the corresponding process.

First, the value of ivlOffset is doubled, i.e. left-shifted by 1 and a single bit is shifted into ivlOffset by using read_bits( 1 ). Then, the value of ivlOffset is compared to the value of ivlCurrRange and further steps are specified as follows:

–	If ivlOffset is greater than or equal to ivlCurrRange, the variable binVal is set equal to 1 and ivlOffset is decremented by ivlCurrRange.

–	Otherwise (ivlOffset is less than ivlCurrRange), the variable binVal is set equal to 0.

The bitstream shall not contain data that result in a value of ivlOffset being greater than or equal to ivlCurrRange upon completion of this process.

Figure 9-8 – Flowchart of bypass decoding process

##### Decoding process for binary decisions before termination

Inputs to this process are bits from slice segment data and the variables ivlCurrRange and ivlOffset.

Outputs of this process are the updated variables ivlCurrRange and ivlOffset, and the decoded value binVal.

This decoding process applies to decoding of end_of_slice_segment_flag, end_of_sub_stream_one_bit and pcm_flag corresponding to ctxTable equal to 0 and ctxIdx equal to 0. Figure 9-9 shows the flowchart of the corresponding decoding process, which is specified as follows:

First, the value of ivlCurrRange is decremented by 2. Then, the value of ivlOffset is compared to the value of ivlCurrRange and further steps are specified as follows:

–	If ivlOffset is greater than or equal to ivlCurrRange, the variable binVal is set equal to 1, no renormalization is carried out, and CABAC decoding is terminated. The last bit inserted in register ivlOffset is equal to 1. When decoding end_of_slice_segment_flag, this last bit inserted in register ivlOffset is interpreted as rbsp_stop_one_bit. When decoding end_of_sub_stream_one_bit, this last bit inserted in register ivlOffset is interpreted as alignment_bit_equal_to_one.

–	Otherwise (ivlOffset is less than ivlCurrRange), the variable binVal is set equal to 0 and renormalization is performed as specified in subclause 9.3.4.3.3.

NOTE – This procedure may also be implemented using DecodeDecision( ctxTable, ctxIdx, bypassFlag ) with ctxTable = 0, ctxIdx = 0 and bypassFlag = 0. In the case where the decoded value is equal to 1, seven more bits would be read by DecodeDecision( ctxTable, ctxIdx, bypassFlag ) and a decoding process would have to adjust its bitstream pointer accordingly to properly decode following syntax elements.

Figure 9-9 – Flowchart of decoding a decision before termination