| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0456-v1 |
| --- | --- |

| Title: | CE8: palette mode when dual-tree is enabled (Test 8.2.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jing Ye, Xiaozhong Xu, Xiang Li, Shan Liu, and | Tel: Email: | jingye@tencent.com xiaozhongxu@tencent.com xlxiangli@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the test results of CE8.2.4. In this test, the modification on palette mode when luma block and chroma block have separate tree structure is proposed. The proposed method reports -11.50%, -8.30%, -8.35% luma and chroma BD rate changing for AI configuration on class F over VTM 3.0. And reports -8.56%, -7.60%, -7.78% luma and chroma BD rate changing for RA configuration on class F over VTM 3.0.

# Proposed CE test

In CE 8.2.1 [1], when it is I slice, the luma and chroma have separate tree structure. In the proposed method, when coding chroma plane, if the co-located luma block are in all palette mode, the chroma block has the flexibility to use palette mode. A flag is signaled to indicate whether the chroma block uses palette mode or not. If the chroma block uses palette mode, the corresponding palette mode syntax will be signaled. Otherwise, the chroma block will not use palette mode.

# Simulation results

The simulation is performed using VTM 3.0, under VTM configuration. The software is implemented on top of CE8.2.1. The mandatory common test conditions (CTC) as specified in the JVET-L1010 [2] are followed. Additional screen content testing materials as specified in CE8 [1] are also evaluated. Results are reported in the following table:

Table 1: simulation results of CE8.2.4.

# Conclusion

In this CE test, an alternative way of palette mode modification when dual tree enabled is proposed. The proposed method reports -11.50%, -8.30%, -8.35% luma and chroma BD rate changing for AI configuration on class F over VTM 3.0. And reports -8.56%, -7.60%, -7.78% luma and chroma BD rate changing for RA configuration on class F over VTM 3.0. It is recommended to adopt this method.

# References

[1] X. Xu, Y.-C. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-L1028, Ljubljana, SL, July 2018

[2]	F.Bossen, J. Boyce, X. Li, V. Sergin, and K. Suhring, ”JVET common test conditions and software reference configurations", JVET-K1010, 11th JVET meeting, July 2018.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).