| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0458-v2 |
| --- | --- |

| Title: | Non-CE3: Combined-Hypothesis Intra-Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gosala Kulupana Andre Seixas Dias Saverio Blasi 56 Wood Lane, Centre House, Block D W12 7SB, London, UK | Tel: Email: | gosala.kulupana@bbc.co.uk andre.seixasdias@bbc.co.uk saverio.blasi@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

This contribution presents a new intra-prediction technique for VTM 3.0. Two intra-prediction hypotheses are computed for a block, one obtained by means of Planar prediction and the other obtained using one of the Angular intra-predictions. The two hypotheses are averaged on a sample-by-sample basis to obtain a combined-hypothesis intra-prediction. Additional syntax elements are signalled to indicate whether a block is predicted using the new technique. In particular, when the intra-prediction mode for a given block is Angular, a flag is signalled in the bitstream to indicate that the block is predicted using combined-hypothesis intra-prediction. Average BD-rates of -0.16% (Y), -0.09% (U) and -0.09% (V) with particularly significant gains in Class A1 (-0.47% (Y), -0.28% (U) and -0.25% ) are reported for AI with 105% average encoding time and 100% decoding time with respect to VTM 3.0.

# Introduction

In VTM 3.0, a given sample can be predicted using one of the intra-prediction modes which consist of Planar, DC and 65 Angular modes. In this contribution, a set of 65 additional combined hypothesis intra-prediction modes are considered. Each additional combined hypothesis mode is obtained by averaging an intra-prediction hypothesis obtained by predicting the block using a given Angular mode, with another intra-prediction hypothesis obtained by predicting the same block using Planar intra-prediction.

|  |  | A | B |  |
| --- | --- | --- | --- | --- |
|  |  | X |  |  |
|  |  | (a) |  |  |

|  |  | X |  |  |
| --- | --- | --- | --- | --- |
|  |  | (b) |  |  |

|  |  | A |  |  |  |
| --- | --- | --- | --- | --- | --- |
| C |  | X |  |  | E |
|  |  | D | (c) |  |  |

# Proposed method

Fig. 1: (a) Conventional Angular      (c) Proposed combined-hypothesis intra        (b) Conventional Planar

As explained in Fig. 1, in this contribution, a combined-hypothesis intra-prediction for a given sample is computed by combining the corresponding Angular prediction (PAng) with the Planar prediction (PPlanar) of the same sample. The combined prediction PComb for a sample (i,j) can therefore be expressed as,

Pcombi,j=(PAngi,j+PPlanari,j+ 1)≫1

An additional binary flag (combined_hypothesis_intra) is sent for each luma block to indicate whether to use the combined hypothesis intra prediction mode or not. The additional modes are applied only on luma samples. Additional restrictions are applied to limit usage of the new modes only on specific block sizes. In particular, the additional modes are only allowed if both the height and width of the block are larger than 8, and at least one between height or width is larger than or equal to 32. Moreover, the method is not used in combination with the multi-line reference samples [1] mode.

Regarding signalling, a single flag is signalled for each intra-predicted block which satisfies the size restriction (cbWidth > 8 && cbHeight > 8 && (cbWidth >= 32 || cbHeight >= 32)). The flag is only signalled in case the intra-prediction mode for the current block is an Angular mode. In case the block is predicted using DC or Planar modes, no additional flag is parsed, and the combined-hypothesis intra-prediction is not used. In case a block is predicted using a MRL index different than zero, the additional flag is not parsed, and combined-hypothesis intra-prediction is not used. The method is disabled for chroma blocks, which are predicted using conventional intra-prediction modes as in VTM 3.0.

The proposed changes to the relevant part of the draft specification text are highlighted below.

|  | Descriptor |
| --- | --- |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( pcm_enabled_flag && cbWidth >= MinIpcmCbSizeY && cbWidth <= MaxIpcmCbSizeY && cbHeight >= MinIpcmCbSizeY && cbHeight <= MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( y0 % CtbSizeY ) > 0 ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if (intra_luma_ref_idx[ x0 ][ y0 ] = = 0) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( IntraPredModeY[x0 ][ y0 ] ! = INTRA_PLANAR && IntraPredModeY[x0 ][ y0 ] ! = INTRA_DC && intra_luma_ref_idx[ x0 ][ y0] = = 0 && (cbWidth > 8 && cbHeight > 8) && (cbWidth >= 32 ¦¦ cbHeight >= 32) ) { |  |
| combined_hypothesis_intra [ x0 ][ y0 ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { /* MODE_INTER */ |  |

# Performance results

Two variants of the proposed method are implemented in the VTM3.0 reference encoder software for AI configuration. TEST1: Test first round of EMT for the combined-hypothesis intra-prediction modes if the corresponding Angular modes are in the SAD candidate list, and test the remaining EMT rounds as per the conventional VTM 3.0. TEST2: Test the first round of EMT for the combined-hypothesis intra-prediction modes if the corresponding Angular modes are in the SAD candidate list, and if an Angular mode is tested during the remaining EMT rounds, test that mode both with and without the combined-hypothesis intra-prediction. The performance of the proposed contribution is summarised in Table 1 and Table 2 respectively. The tests were performed using the JVET common test conditions [2] with parallel encoding/decoding used for RA. The software modifications were made on top of VTM 3.0, compiled with gcc 4.8.2. All experiments were run using AVX.

Table 1: Performance of the proposed combined-hypothesis intra (TEST1).

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.46% | -0.30% | -0.18% | 108% | 101% | -0.18% | -0.37% | -0.31% | 103% | 101% |
| Class A2 | -0.13% | -0.07% | -0.05% | 106% | 101% | -0.07% | -0.04% | 0.03% | 101% | 102% |
| Class B | -0.10% | -0.10% | -0.07% | 103% | 100% | -0.07% | 0.14% | -0.15% | 101% | 101% |
| Class C | -0.03% | -0.01% | -0.06% | 102% | 98% | 0.00% | -0.10% | -0.09% | 101% | 101% |
| Class E | -0.11% | -0.07% | -0.01% | 103% | 98% |  |  |  |  |  |
| Overall | -0.15% | -0.10% | -0.07% | 104% | 99% | -0.07% | -0.06% | -0.13% | 101% | 101% |
| Class D | -0.02% | -0.08% | -0.08% | 103% | 104% | -0.01% | -0.01% | -0.23% | 100% | 102% |
| Class F (optional) | -0.01% | 0.08% | 0.08% | 104% | 97% | 0.00% | 0.06% | 0.16% | 101% | 101% |

Table 2: Performance of the proposed combined-hypothesis intra (TEST2).

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.47% | -0.28% | -0.25% | 108% | 98% | -0.19% | -0.40% | -0.32% | 102% | 100% |
| Class A2 | -0.14% | -0.05% | -0.05% | 105% | 99% | -0.06% | -0.13% | -0.03% | 101% | 101% |
| Class B | -0.11% | -0.06% | -0.05% | 104% | 99% | -0.06% | 0.15% | -0.11% | 101% | 101% |
| Class C | -0.03% | 0.00% | -0.07% | 104% | 101% | -0.01% | -0.18% | -0.04% | 101% | 100% |
| Class E | -0.12% | -0.08% | -0.04% | 105% | 102% |  |  |  |  |  |
| Overall | -0.16% | -0.09% | -0.09% | 105% | 100% | -0.07% | -0.10% | -0.12% | 102% | 100% |
| Class D | -0.02% | -0.08% | -0.06% | 106% | 102% | -0.01% | -0.08% | -0.06% | 101% | 101% |
| Class F (optional) | 0.00% | 0.03% | 0.10% | 105% | 99% | 0.01% | 0.02% | 0.08% | 101% | 99% |

# Conclusions

In this contribution, a method to predict a block by averaging two intra-prediction hypotheses obtained by predicting the same block using Planar prediction and one of the Angular prediction modes is proposed. The method is signalled as an additional syntax element in case a block is predicted using one of the available Angular modes in VTM 3.0. The performance of the proposed combined hypothesis intra-prediction method when implemented in VTM 3.0, demonstrates an average BD-rates of -0.16% (Y), -0.09% (U) and -0.09% (V) with particularly significant gains in Class A1 (-0.47% (Y), -0.28% (U) and -0.25% ) for an average 105% encoder complexity and  no decoder complexity.

# References

- Benjamin Bross, Paul Keydel, Heiko Schwarz, Detlev Marpe, Thomas Wiegand, Liang Zhao, Xin Zhao, Xiang Li, Shan Liu,Yao-Jen Chang, Hui-Yu Jiang,Po-Han Lin, Chi-Chang Kuo Ching-Chieh Lin, Chun-Lung Lin “CE3: Multiple reference line intra prediction (Test 1.1.1, 1.1.2, 1.1.3 and 1.1.4)”, in JVET-L0283, October 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring “JVET common test conditions and software reference configurations for SDR video”, in JVET-L1010, October 2018.

# Patent rights declaration(s)

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).