| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0462-v2 |
| --- | --- |

| Title: | CE2-related: 4x4 chroma affine motion compensation and motion vector rounding unification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van Wei-Jung Chien Han Huang Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 | Tel: Email: | +1-858-658-5664 lphamvan@qti.qualcomm.com |
| Source: | Qualcomm Technologies Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two simplifications for motion vector processing. In the first aspect, the motion vector of a 4x4 affine chroma block is obtained by averaging motion vectors of two diagonal corresponding luma blocks instead of four. This solution leads to an average (Y, U, V) BD-rate change of (0.00%, 0.06%, 0.01%) and (0.03%, 0.01%, 0.12%) under RA and LDB, respectively. The second simplification unifies the motion vector rounding using the TMVP rounding format. This unification results in an average (Y, U, V) BD-rate change of (0.00%, 0.07%, 0.05%) and (-0.01%, -0.08%, 0.12%) under RA and LDB, respectively. The combination of these two approaches leads to an average (Y, U, V) BD-rate change of (0.00%, 0.08%, 0.06%) and (-0.01%, 0.02%, 0.01%) under RA and LDB, respectively.

# Introduction

In the current VTM3, the minimum sub-block size of chroma-components of is set to be 4x4 in affine prediction. The motion vector (MV) of an 4x4 chroma sub-block is defined as the average of the motion vectors of the four corresponding 4x4 luma sub-blocks.

In addition, the rounding pattern of AMVR and affine motion vector is different from that of TMVP motion vector. This difference leads to an inefficient implementation for the decoder.

The affine and AMVR use the following rounding pattern:

offset = 1 << (shift - 1);

rMV.hor = MV.hor >= 0 ? (MV.hor + offset) >> shift: -((-MV.hor + offset) >> shift);

rMV.ver = MV.hor >= 0 ? (MV.ver + offset) >> shift: -((-MV.ver + offset) >> shift);

where rMV is the rounded representation of motion vector MV.

On the other hand, the rounding of the TMVP motion vector uses the following equation:

rMV.hor = (iScale * MV.hor + 127 + (iScale * MV.hor < 0)) >> 8 ;

rMV.ver = (iScale * MV.ver + 127 + (iScale * MV.ver < 0)) >> 8 ;

# Proposed Method

- 4x4 affine chroma sub-block

In this contribution, the motion vector of an 4x4 affine chroma sub-block (MVChroma) is simply calculated by averaging two MVs of two diagonal luma corresponding sub-blocks, i.e., A and D as presented in Figure 1. First, the sum of these two MVs are calculated. Then, this sum motion vector (sMV) is right shifted by 1 with the affine motion vector rounding pattern to form the MV of the 4x4 sub-chroma block as follows:

sMV = MVA + MVD

MVChroma.hor = sMV.hor >= 0 ? (sMV.hor + offset) >> shift : -((-sMV.hor + offset) >> shift)

MVChroma.ver = sMV.ver >= 0 ? (sMV.ver + offset) >> shift : -((-sMV.ver + offset) >> shift)

where offset and shift are set equal to 1.

Figure 1 An 4x4 chroma sub-block and its corresponding 4x4 luma sub-blocks.

- Motion vector rounding unification

In this unification, the motion vector rounding for affine blocks and AMVR is performed with the use of TMVP rounding format as follow:

rMV.hor = (iScale * MV.hor + offset + (iScale * MV.hor < 0)) >> shift)

rMV.ver = (iScale * MV.ver + offset + (iScale * MV.ver < 0)) >> shift)

with offset = (1 << (shift - 1))  - 1. rMV is the rounded representation of the motion vector MV.

For the rounding of the TMVP motion vector, shift is set equal to 8.

For the rounding of the affine and AMVR motion vectors, iScale is set equal to 1.

# Experimental Results

The proposed techniques have been implemented on the top of VTM3. Three following tests have been evaluated:

Test 1: 4x4 affine chroma sub-block using two diagonal motion vectors of the luma component.

Test 2: Motion vector rounding unification with the use of the TMVP rounding pattern.

Test 3: Combination of Test 1 and Test 2. The motion vector of an affine chroma sub-block is also derived using the TMVP rounding pattern.

The performance of these tests is summarized in Table 1, Table 2, and Table, respectively.

As can be seen from these tables, Test 1 leads to an average (Y, U, V) BD-rate change of (0.00%, 0.06%, 0.01%) and (0.03%, 0.01%, 0.12%) under RA and LDB, respectively. Test 2 results in an average (Y, U, V) BD-rate change of (0.00%, 0.07%, 0.05%) and (-0.01%, -0.08%, 0.12%) under RA and LDB, respectively. Test 3 leads to an average (Y, U, V) BD-rate change of (0.00%, 0.08%, 0.06%) and (-0.01%, 0.02%, 0.01%) under RA and LDB, respectively.

Table 1 4x4 affine chroma sub-block using two diagonal motion vectors

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.00% | 0.00% | 98% | 101% |
| Class A2 | 0.00% | 0.10% | 0.10% | 99% | 101% |
| Class B | -0.01% | 0.07% | -0.01% | 99% | 98% |
| Class C | 0.01% | 0.07% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.06% | 0.01% | 99% | 100% |
| Class D | 0.00% | -0.09% | -0.01% | 101% | 101% |
| Class F | 0.00% | -0.01% | 0.01% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.22% | -0.04% | 101% | 101% |
| Class C | 0.06% | 0.24% | 0.36% | 100% | 100% |
| Class E | 0.00% | 0.10% | 0.08% | 100% | 101% |
| Overall | 0.03% | 0.01% | 0.12% | 100% | 101% |
| Class D | -0.01% | -0.18% | 0.78% | 101% | 102% |
| Class F | -0.04% | -0.11% | -0.43% | 101% | 102% |

Table 2 . Motion vector rounding unification

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.00% | 97% | 100% |
| Class A2 | 0.02% | 0.11% | 0.14% | 99% | 98% |
| Class B | 0.00% | 0.08% | 0.00% | 98% | 98% |
| Class C | -0.01% | 0.09% | 0.09% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.07% | 0.05% | 98% | 98% |
| Class D | -0.07% | 0.06% | 0.04% | 100% | 99% |
| Class F | 0.06% | 0.06% | 0.13% | 98% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.06% | 0.20% | 99% | 102% |
| Class C | 0.00% | -0.10% | -0.09% | 102% | 104% |
| Class E | -0.05% | -0.07% | 0.27% | 99% | 98% |
| Overall | -0.01% | -0.08% | 0.12% | 100% | 102% |
| Class D | -0.13% | -0.17% | 0.06% | 96% | 94% |
| Class F | 0.03% | 0.38% | 0.02% | 98% | 96% |

Table 3 Combination of Test 1 and 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.04% | 0.04% | 97% | 100% |
| Class A2 | 0.02% | 0.16% | 0.18% | 99% | 98% |
| Class B | 0.00% | 0.13% | -0.01% | 98% | 97% |
| Class C | -0.02% | 0.03% | 0.08% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.08% | 0.06% | 98% | 98% |
| Class D | -0.07% | 0.08% | -0.01% | 100% | 99% |
| Class F | 0.04% | 0.07% | 0.16% | 98% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.08% | -0.06% | 99% | 100% |
| Class C | -0.01% | 0.12% | 0.13% | 102% | 103% |
| Class E | -0.06% | 0.08% | -0.04% | 98% | 97% |
| Overall | -0.01% | 0.02% | 0.01% | 100% | 100% |
| Class D | -0.13% | -0.12% | 0.08% | 99% | 100% |
| Class F | -0.02% | 0.23% | -0.16% | 98% | 101% |

# Conclusions

This contribution proposes two simplifications for motion vector processing. In the first aspect, the motion vector of a 4x4 affine chroma block is obtained by averaging motion vectors of two diagonal corresponding luma blocks instead of four. In the second aspect, the motion vector rounding is unified to be the TMVP rounding format. A combination of these two approaches has also been tested.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).