| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0471 |
| --- | --- |

| Title: | CE11.1.6, CE11.1.7 and CE11.1.8: Joint proposals for long deblocking from Sony, Qualcomm, Sharp, Ericsson |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | M. Ikeda, T. Suzuki (Sony) D. Rusanovskyy, M.Karczewicz (Qualcomm) W. Zhu, K. Misra, P. Cowan, A. Segall (Sharp) K. Andersson, J. Enhorn, Z. Zhang, R. Sjöberg (Ericsson) | Tel: Email: | +1-360-834-8054 masaru.ikeda@sony.com dmytror@qti.qualcomm.com misrak@sharplabs.com kenneth.r.andersson@ericsson.com |
| Source: | Sony Corp, Qualcomm, Sharp Labs of America, Ericsson |  |  |

_____________________________

# Abstract

This contribution describes a parallel friendly long filter design for deblocking of large blocks common for CE11.1.6, CE11.1.7 and CE11.1.8. It further describes two variants of CTU line buffer reduction in CE11.1.7 and CE11.1.8 respectively. The CE tests are based on technologies that scored highly in subjective assessment carried out for CE11 before Macao meeting. It is asserted that all three tests give better subjective quality than the VTM-3.0 anchor.

The BD-Rate number for CE11.1.6 is Y/U/V reportedly shows 0.0%/-0.5%/-0.5% at AI, 0.0%/-0.8%/-0.8% at RA, 0.1%/-1.3% /-1.6% at LDB and 0.0%/-1.5%/-1.9% at LDP over VTM-3.0 with ALF switched on, respectively. When ALF is switched off, the BD-Rate results for Y/U/V reportedly shows 0.0%/-0.7%/-0.6 % at AI, 0.0%/-0.9%/ -0.9% at RA, 0.0%/-1.1%/-1.7% at LDB and 0.0%/-1.8%/-1.9% at LDP, respectively.

The BD-Rate number for CE11.1.7 is Y/U/V reportedly shows 0.0%/-0.5%/-0.5% at AI, 0.0%/-0.8%/-0.8% at RA, 0.1%/-1.4% /-1.5% at LDB and 0.0%/-1.5%/-1.9% at LDP over VTM-3.0 with ALF switched on, respectively. When ALF is switched off, the BD-Rate results for Y/U/V reportedly shows 0.0%/-0.7%/-0.6 % at AI, 0.0%/-0.9%/ -0.9% at RA, 0.0%/-1.3%/-1.6% at LDB and -0.1%/-1.9%/-2.0% at LDP, respectively.

The BD-Rate number for CE11.1.8 is Y/U/V reportedly shows 0.0%/-0.4%/-0.4% at AI, 0.0%/-0.7%/-0.7% at RA, 0.1%/-1.2% /-1.4% at LDB and 0.0%/-1.2%/-1.6% at LDP over VTM-3.0 with ALF switched on, respectively. When ALF is switched off, the BD-Rate results for Y/U/V reportedly shows 0.0%/-0.6%/-0.5 % at AI, 0.0%/-0.8%/ -0.8% at RA, 0.1%/-1.1%/-1.5% at LDB and 0.0%/-1.6%/-1.7% at LDP, respectively.

# Introduction

Tests CE11.1.6, CE11.1.7 and CE11.1.8 [1] are based on a combination of high-scoring technologies [4][5][6][7] tested in CE11 subjective assessment [8]. The modifications common to all: long filter deblocking of luma, long filter deblocking of chroma, position dependent clipping and sub-block deblocking adjustment. CE11.1.7 and CE11.1.8 also includes modifications to address CTU line buffer restriction. Details of the design is described in section 2. The experimental results are shown in section 3 and the conclusion is provided at the end.

# Proposed Scheme

We first describe CE11.1.6 in sections 2.1-2-6 which also is the basis for CE11.1.7 and CE11.1.8. CE11.1.7 and CE11.1.8 apply additional modifications related to CTU line buffer reduction and those are described in subsections 2.7 and 2.8 respectively.

## Stronger deblocking filter for luma

The proposal uses a bilinear filter when samples at either one side of a boundary belong to a large block. A sample belonging to a large block is defined as when the width >= 32 for a vertical edge, and when height>=32 for a horizontal edge.

The bilinear filter is listed below.

Block boundary samples pi for i=0 to Sp-1 and qi for j=0 to Sq-1 are then replaced by linear interpolation as follows:

- pi´=(fi*Middles,t+64-fi*Ps+32)≫6), clipped to pi±tcPDi

- qj´=(gj*Middles,t+64-gj*Qs+32)≫6), clipped to qj±tcPDj

where tcPDi and tcPDj term is a position dependent clipping described in Section 2.5 and gj, fi, Middles,t, Ps and Qs are given below:

| Sp, Sq 7, 7 (p side: 7, q side: 7) | fi=59-i*9, can also be described as f=59,50,41,32,23,14,5 gj=59-j*9, can also be described as g=59,50,41,32,23,14,5 Middle7,7=(2*po+qo+p1+q1+p2+q2+p3+q3+p4+q4+p5+q5+p6+q6+8)≫4 P7=(p6+p7+1)≫1, Q7=(q6+q7+1)≫1 |
| --- | --- |
| 7, 3 (p side: 7 q side: 3) | fi=59-i*9, can also be described as f=59,50,41,32,23,14,5 gj=53-j*21, can also be described as g=53,32,11 Middle7,3=(2*po+qo+q0+2*(q1+q2)+p1+q1+p2+p3+p4+p5+p6+8)≫4 P7=(p6+p7+1)≫1, Q3=(q2+q3+1)≫1 |
| 3, 7 (p side: 3 q side: 7) | gj=59-j*9, can also be described as g=59,50,41,32,23,14,5 fi=53-i*21, can also be described as f=53,32,11 Middle3.7=(2*qo+po+p0+2*(p1+p2)+q1+p1+q2+q3+q4+q5+q6+8)≫4 Q7=(q6+q7+1)≫1, P3=(p2+p3+1)≫1 |
| 7, 5 (p side: 7 q side: 5) | gj=58-j*13, can also be described as g=58,45,32,19,6 fi=59-i*9, can also be described as f=59,50,41,32,23,14,5 Middle7,5=(2*(po+qo+p1+q1)+q2+p2+q3+p3+q4+p4+q5+p5+8)≫4 Q5=(q4+q5+1)≫1, P7=(p6+p7+1)≫1 |
| 5, 7 (p side: 5 q side: 7) | gj=59-j*9, can also be described as g=59,50,41,32,23,14,5 fi=58-i*13, can also be described as f=58,45,32,19,6 Middle5,7=(2*(qo+po+p1+q1)+q2+p2+q3+p3+q4+p4+q5+p5+8)≫4 Q7=(q6+q7+1)≫1, P5=(p4+p5+1)≫1 |
| 5, 5 (p side: 5 q side: 5) | gj=58-j*13, can also be described as g=58,45,32,19,6 fi=58-i*13, can also be described as f=58,45,32,19,6 Middle5,5=(2*(qo+po+p1+q1+q2+p2)+q3+p3+q4+p4+8)≫4 Q5=(q4+q5+1)≫1, P5=(p4+p5+1)≫1 |
| 5, 3 (p side: 5 q side: 3) | gj=53-j*21, can also be described as g=53,32,11 fi=58-i*13, can also be described as f=58,45,32,19,6 Middle5,3=(qo+po+p1+q1+q2+p2+q3+p3+4)≫3 Q3=(q2+q3+1)≫1, P5=(p4+p5+1)≫1 |
| 3, 5 (p side: 3 q side: 5) | gj=58-j*13, can also be described as g=58,45,32,19,6 fi=53-i*21, can also be described as f=53,32,11 Middle3,5=(qo+po+p1+q1+q2+p2+q3+p3+4)≫3 Q5=(q4+q5+1)≫1, P3=(p2+p3+1)≫1 |

## Deblocking control for luma

The deblocking decision process is described in this sub-section.

Wider-stronger luma filter is filters are used only if all of the Condition1, Condition2 and Condition 3 are TRUE.

The condition 1 is the “large block condition”. This condition detects whether the samples at P-side and Q-side belong to large blocks, which are represented by the variable bSidePisLargeBlk and bSideQisLargeBlk respectively. The bSidePisLargeBlk and bSideQisLargeBlk are defined as follows.

bSidePisLargeBlk = ((edge type is vertical and p0 belongs to CU with width >= 32) | | (edge type is horizontal and p0 belongs to CU with height >= 32))? TRUE: FALSE

bSideQisLargeBlk = ((edge type is vertical and q0 belongs to CU with width >= 32) | | (edge type is horizontal and q0 belongs to CU with height >= 32))? TRUE: FALSE

Based on bSidePisLargeBlk and bSideQisLargeBlk, the condition 1 is defined as follows.

Condition1 = (bSidePisLargeBlk || bSidePisLargeBlk) ? TRUE: FALSE

Next, if Condition 1 is true, the condition 2 will be further checked. First, the following variables are derived:

dp0, dp3, dq0, dq3 are first derived as in HEVC

if (p side is greater than or equal to 32)

dp0 = ( dp0 + Abs( p5,0 − 2 * p4,0 + p3,0 ) + 1 ) >> 1

dp3 = ( dp3 + Abs( p5,3 − 2 * p4,3 + p3,3 ) + 1 ) >> 1

if (q side is greater than or equal to 32)

dq0 = ( dq0 + Abs( q5,0 − 2 * q4,0 + q3,0 ) + 1 ) >> 1

dq3 = ( dq3 + Abs( q5,3 − 2 * q4,3 + q3,3 ) + 1 ) >> 1

dpq0, dpq3, dp, dq, d are then derived as in HEVC.

Then the condition 2 is defined as follows.

Condition2 = (d < β) ? TRUE: FALSE

If Condition1 and Condition2 are valid it is checked if any of the blocks uses sub-blocks:

If(bSidePisLargeBlk)

If(mode block P == SUBBLOCKMODE)

Sp =5

else

Sp =7

else

Sp = 3

If(bSideQisLargeBlk)

If(mode block Q == SUBBLOCKMODE)

Sq =5

else

Sq =7

else

Sq = 3

Finally, if both the Condition 1 and Condition 2 are valid, the proposed deblocking method will check the condition 3 (the large block Strong filter condition), which is defined as follows.

In the Condition3 StrongFilterCondition, the following variables are derived:

dpq is derived as in HEVC.

sp3 = Abs( p3 − p0 ), derived as in HEVC

if (p side is greater than or equal to 32)

if(Sp==5)

sp3 = ( sp3 + Abs( p5 − p3 ) + 1) >> 1

else

sp3 = ( sp3 + Abs( p7 − p3 ) + 1) >> 1

sq3 = Abs( q0 − q3 ), derived as in HEVC

if (q side is greater than or equal to 32)

If(Sq==5)

sq3 = ( sq3 + Abs( q5 − q3 ) + 1) >> 1

else

sq3 = ( sq3 + Abs( q7 − q3 ) + 1) >> 1

As in HEVC derive, StrongFilterCondition = (dpq is less than ( β  >>  2 ), sp3 + sq3 is less than ( 3*β  >>  5 ), and Abs( p0 − q0 ) is less than ( 5 * tC + 1 )  >>  1) ? TRUE : FALSE

## Strong deblocking filter for chroma

The following strong deblocking filter for chroma is defined:

p2′= (3*p3+2*p2+p1+p0+q0+4) >> 3

p1′= (2*p3+p2+2*p1+p0+q0+q1+4) >> 3

p0′= (p3+p2+p1+2*p0+q0+q1+q2+4) >> 3

The proposed chroma filter performs deblocking on a 4x4 chroma sample grid.

## Deblocking control for chroma

The chroma strong filters are used on both sides of the block boundary.  Here, the chroma filter is selected when both sides of the chroma edge are greater than or equal to 8 (chroma position), and the following decision with three conditions are satisfied: The first one is for decision of boundary strength as well as large block. The proposed filter can be applied when the block width or height which orthogonally crosses the block edge is equal to or larger than 8 in chroma sample domain. The second and third one are basically the same as for HEVC luma decison, which are on/off decision and strong filter decision, respectively.

In the first decision, boundary strength (bS) is modified for chorma filtering as shown in Table 1. The condition in Table 1 are checked sequentially. If a condition is satisfied then the remaining conditions with lower priorities are skipped.

Table 1. The modified boundary strength

| Priority | Conditions | Y | U | V |
| --- | --- | --- | --- | --- |
| 5 | At least one of the adjacent blocks is intra | 2 | 2 | 2 |
| 4 | At least one of the adjacent blocks has non-zero transform coefficients | 1 | 1 | 1 |
| 3 | Absolute difference between the motion vectors that belong to the adjacent blocks is greater than or equal to one integer luma sample | 1 | N/A | N/A |
| 2 | Motion prediction in the adjacent blocks refers to vectors is different | 1 | N/A | N/A |
| 1 | Otherwise | 0 | 0 | 0 |

Chroma deblocking is performing when bS is equal to 2, or bS is equal to 1 when a large block boundary is detected.

The second and third condition is basically the same as HEVC luma strong filter decision as follows.

In the second condition :

d is then derived as in HEVC luma deblocking.

The second condition will be TRUE when d is less than β.

In the third condition StrongFilterCondition is derived as follows:

dpq is derived as in HEVC.

sp3 = Abs( p3 − p0 ), derived as in HEVC

sq3 = Abs( q0 − q3 ), derived as in HEVC

As in HEVC derive, StrongFilterCondition = (dpq is less than ( β  >>  2 ), sp3 + sq3 is less than ( β  >>  3 ), and Abs( p0 − q0 ) is less than ( 5 * tC + 1 )  >>  1)

## Position dependent clipping

The proposal also introduces a position dependent clipping tcPD which is applied to the output samples of the luma filtering process involving strong and long filters that are modifying 7, 5 and 3 samples at the boundary. Assuming quantization error distribution, it is proposed to increase clipping value for samples which are expected to have higher quantization noise, thus expected to have higher deviation of the reconstructed sample value from the true sample value.

For each P or Q boundary filtered with proposed asymmetrical filter, depending on the result of decision making process described in Section 2.2, position dependent threshold table is selected from Tc7 and Tc3 tables that are provided to decoder as a side information:

Tc7 = { 6, 5, 4, 3, 2, 1, 1};
     Tc3 = { 6, 4, 2 };

tcPD = (SP == 3) ? Tc3 : Tc7;

tcQD = (SQ == 3) ? Tc3 : Tc7;

For the P or Q boundaries being filtered with a short symmetrical filter, position dependent threshold of lower magnitude is applied:

Tc3 = { 3, 2, 1 };

Following defining the threshold, filtered p’i and q’i sample values are clipped according to tcP and tcQ clipping values:

p’’i = clip3(p’i + tcPi, p’i – tcPi, p’i );

q’’j = clip3(q’j + tcQj, q’j – tcQ j, q’j );

where p’i  and q’i are filtered sample values, p’’i and q’’j are output sample value after the clipping and tcPi  tcPi  are clipping thresholds that are derived from the VVC tc parameter and tcPD and tcQD. Term clip3 is a clipping function as it is specified in VVC.

## Sub-block deblocking adjustment

To enable parallel friendly deblocking using both long filters and sub-block deblocking the long filters is restricted to modify at most 5 samples on a side that uses sub-block deblocking (AFFINE or ATMVP) as shown in the luma control for long filters. Additionally the sub-block deblocking is adjusted such that that sub-block boundaries on an 8x8 grid that are close to a CU or an implicit TU boundary is restricted to modify at most two samples on each side.

Following applies to sub-block boundaries that not are aligned with the CU boundary.

If(mode block Q == SUBBLOCKMODE && edge!=0){

if (!(implicitTU && (edge == (64 / 4))))

if (edge == 2 || edge == (orthogonalLength - 2) || edge == (56 / 4) || edge == (72 / 4))

Sp = Sq = 2;

else

Sp = Sq = 3;

else

Sp = Sq = bSideQisLargeBlk ? 5:3

}

Where edge equal to 0 corresponds to CU boundary, edge equal to 2 or equal to orthogonalLength-2 corresponds to sub-block boundary 8 samples from a CU boundary etc. Where implicit TU is true if implicit split of TU is used.

## CE11.1.7 Restriction to 6 CTU/3CTU line buffers for luma/chroma

Filtering of horizontal boundary is limiting Sp=5 for luma, Sp=3 for chroma when the horizontal boundary is aligned with the CTU boundary.

In chroma, the reference sample for p side is limited up to three samples away from the boundary in case of the CTU boundary to reduce CTU line buffers. Accordingly, the StrongFilterCondition and chroma filter are modified by padding as follows.

The sp3 derivation is modified by padding of p2 sample instead of p3:

sp3 = Abs( p2 − p0 )

The chroma filter for p2 is modified by padding of p2 sample instead of p3:

p2′= (5*p2+p1+p0+q0+4) >> 3

## CE11.1.8 Restriction to 4CTU/2CTU line buffers for luma/chroma

Filtering of horisontal boundary is limiting Sp= 3 for luma, Sp=1 and Sq=1 for chroma, when the horisontal boundary is aligned with the CTU boundary.

# Experimental Results

The following two conditions are evaluated: 1) VTM-3.0 with ALF switched on 2) VTM-3.0 with ALF switched off. LDP results are provided in addition to CTC .[2]

## CE11.1.6

Table 1 Coding Performance

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 (ALF OFF) |  |  |  |  | Over VTM-3.0 (ALF ON) |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V |  | EncT | DecT |
| Class A1 | -0.08% | -2.17% | -1.50% | 96% | 98% | -0.02% | -1.32% | -1.01% |  |  | 99% |
| Class A2 | -0.01% | -0.21% | -0.19% | 93% | 102% | -0.01% | -0.18% | -0.20% |  |  | 99% |
| Class B | -0.01% | -0.47% | -0.40% | 87% | 103% | -0.01% | -0.44% | -0.37% |  |  | 101% |
| Class C | -0.01% | -0.21% | -0.28% | 97% | 103% | 0.00% | -0.22% | -0.29% |  |  | 100% |
| Class E | -0.08% | -0.80% | -0.96% | 90% | 100% | -0.07% | -0.66% | -0.75% |  |  | 97% |
| Overall | -0.03% | -0.71% | -0.61% | 92% | 101% | -0.02% | -0.53% | -0.49% |  |  | 99% |
| Class D | 0.00% | -0.13% | -0.18% | 95% | 103% | 0.00% | -0.14% | -0.18% |  |  | 103% |
| Class F | -0.01% | -0.18% | -0.29% | 93% | 103% | -0.01% | -0.18% | -0.30% |  |  | 101% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 (ALF OFF) |  |  |  |  | Over VTM-3.0 (ALF ON) |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT |  | DecT |
| Class A1 | -0.20% | -2.20% | -2.15% | 84% | 102% | -0.06% | -1.79% | -1.88% |  |  | 100% |
| Class A2 | -0.01% | -0.44% | -0.40% | 85% | 102% | -0.02% | -0.58% | -0.45% |  |  | 103% |
| Class B | 0.06% | -0.77% | -0.84% | 85% | 103% | 0.04% | -0.74% | -0.84% |  |  | 103% |
| Class C | 0.00% | -0.35% | -0.36% | 84% | 103% | 0.01% | -0.32% | -0.29% |  |  | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.02% | -0.88% | -0.89% | 84% | 103% | 0.00% | -0.80% | -0.82% |  |  | 102% |
| Class D | 0.04% | -0.21% | -0.33% | 85% | 101% | -0.02% | -0.28% | -0.32% |  |  | 102% |
| Class F | 0.00% | -0.22% | -0.29% | 90% | 101% | 0.00% | -0.16% | -0.17% |  |  | 102% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 (ALF OFF) |  |  |  |  | Over VTM-3.0 (ALF ON) |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.11% | -1.65% | -2.11% | 93% | 103% | 0.14% | -1.89% | -2.08% |  |  | 102% |
| Class C | -0.02% | -0.23% | -0.77% | 90% | 104% | 0.02% | -0.25% | -0.33% |  |  | 101% |
| Class E | -0.02% | -1.41% | -2.42% | 81% | 101% | 0.26% | -1.80% | -2.40% |  |  | 101% |
| Overall | 0.04% | -1.12% | -1.74% | 89% | 103% | 0.13% | -1.32% | -1.58% |  |  | 102% |
| Class D | 0.02% | -0.67% | -0.91% | 95% | 102% | -0.01% | -0.48% | -0.68% |  |  | 103% |
| Class F | -0.04% | 0.68% | -0.36% | 85% | 105% | 0.01% | 0.55% | 0.54% |  |  | 101% |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 (ALF OFF) |  |  |  |  | Over VTM-3.0 (ALF ON) |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.04% | -2.30% | -2.63% | 80% | 103% | 0.00% | -1.86% | -1.92% |  |  | 101% |
| Class C | 0.01% | -0.38% | -0.71% | 78% | 101% | -0.03% | -0.25% | -0.84% |  |  | 99% |
| Class E | -0.06% | -2.72% | -2.42% | 88% | 102% | -0.08% | -2.71% | -3.21% |  |  | 106% |
| Overall | -0.02% | -1.77% | -1.94% | 81% | 102% | -0.03% | -1.54% | -1.88% |  |  | 102% |
| Class D | -0.05% | -0.72% | -0.58% | 80% | 105% | -0.02% | -0.56% | -0.66% |  |  | 102% |
| Class F | 0.01% | -0.01% | -0.22% | 84% | 105% | 0.02% | 0.06% | -0.36% |  |  | 103% |

## CE11.1.7

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 (ALF OFF) |  |  |  |  | Over VTM-3.0 (ALF ON) |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V |  | EncT | DecT |
| Class A1 | -0.08% | -2.18% | -1.51% | 105% | 98% | -0.02% | -1.34% | -1.02% |  | 98% | 98% |
| Class A2 | -0.01% | -0.22% | -0.20% | 104% | 98% | -0.01% | -0.19% | -0.20% |  | 99% | 99% |
| Class B | -0.01% | -0.47% | -0.41% | 108% | 98% | -0.01% | -0.45% | -0.38% |  | 101% | 99% |
| Class C | -0.01% | -0.21% | -0.28% | 110% | 100% | 0.00% | -0.23% | -0.30% |  | 101% | 99% |
| Class E | -0.08% | -0.81% | -0.97% | 111% | 100% | -0.07% | -0.67% | -0.76% |  | 104% | 100% |
| Overall | -0.03% | -0.71% | -0.62% | 108% | 99% | -0.02% | -0.54% | -0.50% |  | 101% | 99% |
| Class D | 0.00% | -0.13% | -0.17% | 111% | 100% | 0.00% | -0.14% | -0.17% |  | 103% | 99% |
| Class F | -0.01% | -0.19% | -0.29% | 107% | 100% | -0.01% | -0.19% | -0.31% |  | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 (ALF OFF) |  |  |  |  | Over VTM-3.0 (ALF ON) |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT |  | DecT |
| Class A1 | -0.20% | -2.22% | -2.16% | 102% | 102% | -0.07% | -1.88% | -1.90% | 99% |  | 101% |
| Class A2 | -0.02% | -0.37% | -0.45% | 100% | 102% | -0.03% | -0.48% | -0.40% | 100% |  | 102% |
| Class B | 0.04% | -0.71% | -0.79% | 100% | 102% | 0.04% | -0.82% | -0.91% | 103% |  | 102% |
| Class C | -0.02% | -0.38% | -0.27% | 100% | 102% | 0.00% | -0.27% | -0.31% | 101% |  | 103% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.04% | -0.86% | -0.86% | 100% | 102% | -0.01% | -0.82% | -0.85% | 101% |  | 102% |
| Class D | 0.00% | -0.26% | -0.29% | 101% | 102% | 0.00% | -0.21% | -0.36% | 101% |  | 102% |
| Class F | 0.00% | -0.21% | -0.30% | 101% | 103% | -0.02% | -0.26% | -0.26% | 102% |  | 103% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 (ALF OFF) |  |  |  |  | Over VTM-3.0 (ALF ON) |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.10% | -1.69% | -2.05% | 107% | 104% | 0.12% | -2.03% | -1.69% | 103% |  | 102% |
| Class C | -0.02% | -0.25% | -0.58% | 100% | 105% | 0.03% | 0.01% | -0.48% | 101% |  | 104% |
| Class E | 0.03% | -2.12% | -2.20% | 107% | 106% | 0.21% | -2.14% | -2.47% | 100% |  | 105% |
| Overall | 0.04% | -1.32% | -1.60% | 105% | 105% | 0.12% | -1.38% | -1.48% | 102% |  | 103% |
| Class D | -0.01% | -0.89% | -1.02% | 104% | 102% | -0.01% | -0.41% | -1.25% | 101% |  | 103% |
| Class F | 0.07% | 0.29% | 0.00% | 103% | 103% | 0.14% | 0.59% | 0.34% | 101% |  | 103% |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 (ALF OFF) |  |  |  |  | Over VTM-3.0 (ALF ON) |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.02% | -2.27% | -2.69% | 95% | 103% | -0.01% | -1.74% | -1.85% | 102% |  | 102% |
| Class C | -0.04% | -0.48% | -0.93% | 92% | 105% | 0.02% | -0.21% | -0.68% | 101% |  | 101% |
| Class E | -0.16% | -2.98% | -2.31% | 99% | 103% | -0.08% | -2.79% | -3.45% | 100% |  | 102% |
| Overall | -0.06% | -1.85% | -2.01% | 95% | 104% | -0.02% | -1.49% | -1.86% | 101% |  | 102% |
| Class D | -0.10% | -0.68% | -0.71% | 99% | 103% | -0.02% | -0.85% | -0.44% | 101% |  | 101% |
| Class F | 0.06% | 0.18% | -0.11% | 100% | 101% | 0.03% | -0.27% | 0.04% | 101% |  | 101% |

## CE11.1.8

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 (ALF OFF) |  |  |  |  | Over VTM-3.0 (ALF ON) |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V |  | EncT | DecT |
| Class A1 | -0.07% | -1.80% | -1.22% | 106% | 107% | -0.02% | -1.08% | -0.80% |  | 102% | 102% |
| Class A2 | -0.01% | -0.16% | -0.15% | 105% | 105% | 0.00% | -0.14% | -0.15% |  | 99% | 101% |
| Class B | -0.01% | -0.38% | -0.33% | 102% | 105% | -0.01% | -0.36% | -0.30% |  | 99% | 101% |
| Class C | 0.00% | -0.17% | -0.23% | 102% | 103% | 0.00% | -0.19% | -0.25% |  | 102% | 105% |
| Class E | -0.07% | -0.64% | -0.78% | 105% | 105% | -0.07% | -0.49% | -0.58% |  | 104% | 105% |
| Overall | -0.03% | -0.58% | -0.50% | 104% | 105% | -0.02% | -0.43% | -0.39% |  | 101% | 103% |
| Class D | 0.00% | -0.12% | -0.16% | 100% | 102% | 0.00% | -0.12% | -0.16% |  | 101% | 104% |
| Class F | -0.01% | -0.14% | -0.23% | 104% | 103% | -0.01% | -0.14% | -0.24% |  | 101% | 104% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 (ALF OFF) |  |  |  |  | Over VTM-3.0 (ALF ON) |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT |  | DecT |
| Class A1 | -0.17% | -1.99% | -1.85% | 102% | 103% | -0.05% | -1.56% | -1.52% | 100% |  | 100% |
| Class A2 | -0.01% | -0.33% | -0.31% | 103% | 103% | -0.02% | -0.50% | -0.35% | 97% |  | 98% |
| Class B | 0.04% | -0.61% | -0.70% | 104% | 103% | 0.04% | -0.69% | -0.73% | 102% |  | 101% |
| Class C | -0.01% | -0.38% | -0.35% | 102% | 103% | 0.01% | -0.27% | -0.25% | 101% |  | 104% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.03% | -0.77% | -0.76% | 103% | 103% | 0.00% | -0.71% | -0.68% | 100% |  | 101% |
| Class D | 0.01% | -0.26% | -0.26% | 100% | 103% | 0.01% | -0.28% | -0.27% | 100% |  | 103% |
| Class F | 0.00% | -0.17% | -0.24% | 102% | 103% | 0.00% | -0.18% | -0.17% | 100% |  | 103% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 (ALF OFF) |  |  |  |  | Over VTM-3.0 (ALF ON) |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.09% | -1.50% | -1.86% | 101% | 103% | 0.12% | -1.55% | -1.50% | 99% |  | 105% |
| Class C | -0.04% | -0.20% | -0.52% | 105% | 103% | 0.03% | -0.20% | -0.52% | 97% |  | 98% |
| Class E | 0.11% | -1.76% | -2.16% | 105% | 102% | 0.09% | -1.75% | -2.51% | 100% |  | 99% |
| Overall | 0.05% | -1.13% | -1.49% | 104% | 103% | 0.08% | -1.15% | -1.42% | 99% |  | 101% |
| Class D | 0.00% | -0.69% | -0.90% | 101% | 103% | 0.02% | -0.34% | -0.76% | 103% |  | 104% |
| Class F | 0.12% | 0.47% | -0.10% | 104% | 102% | 0.11% | 0.56% | 0.35% | 99% |  | 101% |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 (ALF OFF) |  |  |  |  | Over VTM-3.0 (ALF ON) |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.03% | -2.06% | -2.21% | 99% | 103% | 0.02% | -1.44% | -1.84% | 100% |  | 102% |
| Class C | 0.02% | -0.50% | -0.69% | 103% | 103% | 0.01% | -0.25% | -0.53% | 100% |  | 98% |
| Class E | -0.05% | -2.46% | -1.99% | 106% | 103% | -0.15% | -2.12% | -2.70% | 102% |  | 100% |
| Overall | -0.02% | -1.64% | -1.65% | 102% | 103% | -0.02% | -1.21% | -1.62% | 101% |  | 100% |
| Class D | -0.07% | -0.92% | -0.74% | 101% | 104% | -0.05% | -0.41% | -0.43% | 102% |  | 106% |
| Class F | -0.01% | 0.63% | 0.23% | 104% | 102% | 0.15% | 0.02% | 0.20% | 100% |  | 100% |

# Conclusion

Subject to the preferred CTU line buffer configuration we recommend CE11.1.6 (8 CTU line buffers), CE11.1.7 (6 CTU line buffers) or CE11.1.8 (4 CTU line buffers) to be considered for adoption in next version of VTM and VVC Working Draft.

# Reference

- A. Norkin, A. M. Kotra “Description of Core Experiment 11 (CE11): Deblocking”, JVET-L1031, October 2018, Macao, China.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, October 2018, Macao, China.

- VTM 3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_vtm.git

- D. Rusanovskyy, M.Karczewicz, “CE11: Test on long deblocking filtering from JVET-J0021/K0334 (CE11.1.4),” JVET-L0403, 12th Meeting: Macao, CN, 3–12 Oct. 2018

- M. Ikeda, T. Suzuki, “CE11: Long-tap deblocking filter for luma and chroma (CE11.1.6),” , JVET-L0327, 12th Meeting: Macao, CN, 3–12 Oct. 2018

- W. Zhu, K. Misra, P. Cowan, A. Segall, “CE11: Deblocking modifications for Large CUs both luma and chroma (Test 11.1.7a and CE11.1.7b),” JVET-L0405, 12th Meeting: Macao, CN, 3–12 Oct. 2018

- K. Andersson, Z. Zhang, R. Sjöberg, “CE11: Long deblocking filters for luma (CE11.1.1) and for both luma and chroma (CE11.1.9),” JVET-L0072, 12th Meeting: Macao, CN, 3–12 Oct. 2018

- V. Baroncini, A. Norkin, A. M. Kotra, “Subjective assessment of CE11 proposals,” JVET-L0611, 12th Meeting: Macao, CN, 3–12 Oct. 2018

# Patent rights declaration(s)

Sony Corp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).