| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0477 |
| --- | --- |

| Title: | CE2: Simplification of Affine constructed merge candidates (Test 2.2.6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Ciao Yang Yao-Jen Chang | Email: | yuciaoyang@fginnov.com |
| Source: | Foxconn |  |  |

_____________________________

- Abstract

The affine merge list in VTM3.0 includes first 5 available candidates out of 1 ATMVP candidate and 2 inherited candidates and 6 constructed candidates. This contribution focuses on the simplification of the Affine constructed merge candidates. The first experiment, Test 2.2.6.a, is related to the reduction of the maximum number of available candidates. At most 3 constructed candidates are allowed to be selected out of constructed merge candidates in 7 conditional cases in Test 2.2.6.a. The second experiment, Test 2.2.6.b, is related to the reduction on the number of conditional cases for the constructed merge candidates. At most 3 constructed merge candidates are allowed to be selected out of constructed merge candidates in 2 conditional cases in Test 2.2.6.b. The proposed simplifications are implemented on top of VTM3.0. Compared to VTM3.0, the results in Test 2.2.6.a are 0.00%/0.01%/-0.01% YUV-BDrate with 100%/100% coding time in RA condition, and 0.07%/-0.01%/0.18% YUV-BDrate with 99%/99% coding time in LB condition. The results in Test 2.2.6.b are 0.02%/0.04%/0.00% YUV-BDrate with 100%/100% coding time in RA condition, and 0.08%/0.14%/0.04% YUV-BDrate with 99%/98% coding time in LB condition. This contribution further studies two supplemental tests defined as Test c and Test d, which remove the checks of equal reference frames and the checks of unequal control point MVs on top of Test 2.2.6.a and Test 2.2.6.b. Compared to VTM3.0, the results in Test c are 0.01%/0.07%/0.00% YUV-BDrate with 100%/100% coding time in RA condition, and 0.02%/-0.12%/-0.02% YUV-BDrate with 100%/100% coding time in LB condition. The results in Test d are 0.04%/0.07%/0.01% YUV-BDrate with 99%/101% coding time in RA condition, and 0.06%/0.13%/0.03% YUV-BDrate with 100%/98% coding time in LB condition.

Introduction

The Affine constructed merge candidates in VTM-3.0 are composed of 6-parameter (6-param) constructed merge candidates and 4-parameter (4-param) constructed merge candidates. Define the indexes for 6-param constructed candidates and 4-param constructed candidates as follows:
1: [CP0, CP1, CP2]
2: [CP0, CP1, CP3]
3: [CP0, CP2, CP3]
4: [CP1, CP2, CP3]
5: [CP0, CP1]
6: [CP0, CP2]

There are 7 conditional cases to describe the enabled constructed candidates in VTM3.0 as shown in Table 1.

Table 1. Enabled constructed candidates in different conditional cases (VTM3.0)

| V: Availability X: Unavailability |  |  |  | The index of the enabled constructed candidates |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CP0 | CP1 | CP2 | CP3 |  |  |  |  |  |  |
| V | V | V | V | 0 | 1 | 2 | 3 | 4 | 5 |
| V | V | V | X | 0 | 4 | 5 |  |  |  |
| V | V | X | V | 1 | 4 |  |  |  |  |
| V | X | V | V | 2 | 5 |  |  |  |  |
| X | V | V | V | 3 |  |  |  |  |  |
| V | V | X | X | 4 |  |  |  |  |  |
| V | X | V | X | 5 |  |  |  |  |  |
| Others |  |  |  | - |  |  |  |  |  |

JVET-L0390 [1] simplifies the Affine constructed merge candidates, and was agreed to be tested in Core Experiment (CE) 2 [2] to evaluate the performance on top of common test conditions in VTM-3.0 [3]. The following table shows the description for the CE tests reported in this document:

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 2.2.6 | a) Simplification for 6-param and 4-param constructed candidates by setting P = 3 in 7 conditional cases b) Simplification for 6-param and 4-param constructed candidates by setting P = 3 in 2 conditional cases | JVET-L0390 | Y.-C. Yang (Foxconn) | H. Liu (Bytedance) |

Two supplemental tests which remove the equal checks related to control point MVs (CPMVs) and reference pictures on top of Test 2.2.6.a and 2.2.6.b are also provided with this contribution.

The proposed simplifications

There are four experiments related to Affine merge simplifications in this proposal. The first experiment is Test 2.2.6.a which reduces the maximum number of constructed merge candidates from 6 to 3 in 7 conditional cases. Table 2 describes the enabled constructed candidates in 7 conditional cases.

Table 2. Enabled constructed candidates in 7 conditional cases (Test 2.2.6.a & Test c)

| V: Availability X: Unavailability |  |  |  | The index of the enabled constructed candidates |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CP0 | CP1 | CP2 | CP3 |  |  |  |
| V | V | V | V | 0 | 1 | 2 |
| V | V | V | X | 0 | 4 | 5 |
| V | V | X | V | 1 | 4 |  |
| V | X | V | V | 2 | 5 |  |
| X | V | V | V | 3 |  |  |
| V | V | X | X | 4 |  |  |
| V | X | V | X | 5 |  |  |
| Others |  |  |  | - |  |  |

The second experiment is Test 2.2.6.b which reduces the maximum number of constructed merge candidates from 6 to 3, and further reduces the conditional cases from 7 to 2. Table 3 describes the enabled constructed candidates in 2 conditional cases.

Table 3. Enabled constructed candidates in 2 conditional cases (Test 2.2.6.b & Test d)

| V: Availability X: Unavailability |  |  |  | Constructed candidate index |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CP0 | CP1 | CP2 | CP3 |  |  |  |
| V | V | V | V | 0 | 1 | 2 |
| V | V | V | X | 0 | 4 | 5 |
| Others |  |  |  | - |  |  |

Two supplemental experiments Test c and Test d are further tested to remove the checks of equal reference frames and the checks of unequal CPMVs on top of Test 2.2.6.a and Test 2.2.6.b, respectively.

Experiment result

The proposed simplification of Affine constructed merge candidates was tested on top of VTM3.0 under common test conditions [3]. Table 4 shows the experimental results for Test 2.2.6.a. Table 5 shows the experimental results for Test 2.2.6.b. Table 6 shows the experiment results for Test c. Table 7 shows the experiment results for Test d.

Table 4. Summary of experimental results (Test 2.2.6.a over VTM3.0)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.01% | -0.02% | 100% | 100% |
| Class A2 | -0.01% | -0.02% | 0.07% | 100% | 100% |
| Class B | 0.00% | 0.01% | -0.05% | 100% | 100% |
| Class C | 0.01% | 0.02% | 0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.01% | 100% | 100% |
| Class D | 0.03% | -0.03% | 0.02% | 99% | 100% |
| Class F (optional) | 0.00% | 0.01% | 0.04% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.23% | 0.07% | 99% | 102% |
| Class C | 0.02% | 0.32% | 0.22% | 99% | 94% |
| Class E | 0.21% | -0.09% | 0.34% | 98% | 101% |
| Overall | 0.07% | -0.01% | 0.18% | 99% | 99% |
| Class D | 0.07% | 0.03% | -0.25% | 100% | 90% |
| Class F (optional) | -0.01% | -0.08% | 0.26% | 100% | 95% |

Table 5. Summary of experimental results (Test 2.2.6.b over VTM3.0)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.07% | 0.01% | 100% | 100% |
| Class A2 | 0.01% | 0.04% | 0.15% | 100% | 100% |
| Class B | 0.02% | 0.08% | -0.06% | 100% | 100% |
| Class C | 0.04% | 0.08% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.04% | 0.00% | 100% | 100% |
| Class D | 0.04% | -0.04% | -0.01% | 99% | 100% |
| Class F (optional) | 0.00% | 0.01% | 0.05% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.25% | -0.04% | 100% | 101% |
| Class C | 0.04% | 0.43% | 0.21% | 99% | 94% |
| Class E | 0.18% | 0.41% | -0.05% | 98% | 101% |
| Overall | 0.08% | 0.14% | 0.04% | 99% | 98% |
| Class D | 0.07% | 0.67% | -0.14% | 100% | 89% |
| Class F (optional) | -0.02% | 0.05% | 0.17% | 99% | 95% |

Table 6. Summary of experimental results (Test c over VTM3.0)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | 0.01% | 100% | 100% |
| Class A2 | 0.01% | 0.05% | 0.09% | 100% | 100% |
| Class B | 0.02% | 0.13% | -0.06% | 100% | 100% |
| Class C | 0.02% | 0.08% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.07% | 0.00% | 100% | 100% |
| Class D | 0.04% | -0.17% | -0.06% | 99% | 100% |
| Class F (optional) | -0.02% | -0.04% | -0.05% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.27% | -0.15% | 100% | 100% |
| Class C | 0.04% | 0.04% | 0.20% | 100% | 100% |
| Class E | -0.06% | -0.06% | -0.10% | 100% | 100% |
| Overall | 0.02% | -0.12% | -0.02% | 100% | 100% |
| Class D | 0.08% | 0.05% | 0.30% | 100% | 100% |
| Class F (optional) | 0.05% | 0.26% | -0.39% | 101% | 99% |

Table 7. Summary of experimental results (Test d over VTM3.0)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.07% | 0.00% | 100% | 100% |
| Class A2 | 0.02% | 0.12% | 0.09% | 100% | 100% |
| Class B | 0.02% | 0.03% | -0.04% | 101% | 101% |
| Class C | 0.06% | 0.08% | 0.04% | 97% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.07% | 0.01% | 99% | 101% |
| Class D | 0.04% | -0.01% | 0.10% | 100% | 100% |
| Class F (optional) | 0.00% | 0.03% | 0.03% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.06% | 0.15% | 101% | 101% |
| Class C | 0.05% | 0.31% | 0.09% | 100% | 93% |
| Class E | 0.08% | 0.24% | -0.23% | 99% | 100% |
| Overall | 0.06% | 0.13% | 0.03% | 100% | 98% |
| Class D | 0.14% | 0.09% | -0.02% | 101% | 91% |
| Class F (optional) | 0.05% | 0.38% | 0.38% | 100% | 99% |

Conclusion

This contribution proposes a simplified Affine merge list construction method. It is claimed that the proposed method can keep the same gain with lower complexity as compared with the VTM-3.0. It is suggested to adopt this proposal into the next version of VVC Working Draft.

Reference

- Y.-J. Chang and Y.-C. Yang, “CE4-related: Simplification of Affine merge common codebase,” JVET-L0390, Macao, CN, Oct. 2018.

- Y. He, C.-Y. Chen and C.-C Chen, “Description of Core Experiment 2 (CE2): Sub-block based motion prediction,” JVET-L1022, Macao, CN, Oct. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, CN, Oct. 2018.

Patent rights declaration(s)

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).