| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0478-v1 |
| --- | --- |

| Title: | Non-CE3: PDPC extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Adarsh K. Ramasubramonian Vadim Seregin Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121 | Email: | geertv@qti.qualcomm.com aramasub@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

It is proposed to extend PDPC to non-square block diagonal directions that were adopted into VTM3 for intra prediction. The reported BD-rate gain is 0.05% and 0.04% with runtimes (enc/dec) 100%/101% and 99%/99% for the AI and RA configurations, respectively.

# Proposal

In VTM3, directional modes corresponding with the diagonals of non-square blocks were adopted for intra prediction, while PDPC is currently applied to the top-right (mode 2) and bottom-left (mode 66) directions, which correspond with the diagonals of a square block only, and to the eight modes that are adjacent to the diagonals. It is proposed to extend PDPC to the non-square block diagonals and adjacent modes.

PDPC for diagonal prediction directions is implemented by identifying left and top sample locations in the reference sample arrays that consist of neighbouring reconstructed samples. The prediction sample pred(x,y) located at (x, y) is predicted with a diagonal intra prediction mode and its value is modified using the general PDPC expression as follows:

pred(x,y) = ( wL × R-1,y + wT × Rx,-1 – wTL × R-1,-1 + (64 – wL – wT + wTL) × pred(x,y) + 32 )  >>  6,        (Eq. 1)

where Rx,-1, R-1,y represent the reference samples located at the top and left of the current sample (x, y), respectively, and R-1,-1 represents the reference sample located at the top-left corner of the current block. This is illustrated in Figure 1.

The weights wL, wT, wTL, are calculated as follows for a block with dimensions width and height:

wT = 16 >> ( ( y<<1 ) >> shift ), wL = 16 >> ( ( x<<1 ) >> shift ), wTL = 0,

with shift = ( log2( width ) + log2( height ) – 2 ) >> 2

| (a) Top-right diagonal prediction | (b) Bottom-left diagonal prediction |
| --- | --- |

Figure 1: Definition of reference samples used by PDPC extension to diagonal angular intra modes of non-square blocks.

# Results

The common test conditions specified in JVET-L1010 for SDR video and the guidelines described in the CE3 test description document JVET-L1023 are followed. Test results are reported for the AI and RA test configurations. The VTM3.0 software is used for implementing the tests.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.13% | 0.03% | 100% | 102% |
| Class A2 | -0.07% | -0.02% | -0.06% | 102% | 97% |
| Class B | -0.06% | 0.00% | 0.06% | 99% | 102% |
| Class C | -0.02% | -0.04% | -0.05% | 101% | 102% |
| Class E | -0.07% | -0.08% | 0.04% | 100% | 100% |
| Overall | -0.05% | -0.05% | 0.01% | 100% | 101% |
| Class D | -0.03% | -0.12% | 0.02% | 99% | 100% |
| Class F | 0.07% | -0.02% | 0.17% | 100% | 101% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.06% | 0.01% | 100% | 101% |
| Class A2 | -0.03% | 0.08% | 0.15% | 99% | 100% |
| Class B | -0.06% | 0.15% | -0.10% | 98% | 98% |
| Class C | -0.02% | 0.07% | 0.18% | 98% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.10% | 0.05% | 99% | 99% |
| Class D | -0.03% | -0.27% | -0.22% | 98% | 95% |
| Class F | 0.05% | 0.22% | 0.11% | 100% | 101% |

# Conclusion

It is requested to adopt the proposed PDPC extension to non-square block diagonal prediction directions into VTM4.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).