| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0482-v1 |
| --- | --- |

| Title: | CE6-related: Implicit transform selection for Multi-hypothesis inter-intra mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya | Tel: Email: | +81-3-5494-3353 iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution proposes to apply implicit transform selection for multi-hypothesis inter-intra (MHIntra) prediction. Currently, multiple transform selection (MTS) is applied to a luma block, where DCT-II, DST-VII, and DCT-VIII are selectively applied for horizontal and vertical direction independently by signalling tu_mts_flag and mts_idx. The proposed method implicitly selects transform core without signalling mts_idx for blocks to which MHIntra is applied if the tu_mts_flag is true. Experimental results against VTM-3.0 with inter MTS enabled show negligible impact on BD-rate performance with reducing 4% of encoding time.

Introduction

In VTM-3.0, multiple transform selection (MTS) is introduced where DCT-II, DST-VII, and DCT-VIII are selectively applied for horizontal and vertical direction independently by signalling tu_mts_flag and mts_idx. Generally, the energy distribution of the residual signals of inter predicted block has no tendency, so that multiple core selection by signalling is efficient by selecting the best transform core among the possible combinations of transform cores.

Meanwhile, multi-hypothesis inter-intra (MHIntra) prediction was adopted in the 12th JVET meeting, in which prediction signal is generated by combination of inter merge prediction and intra prediction. In MHIntra prediction, neighboring reconstructed signals are used to modify inter prediction signal. Therefore, it can be assumed that the energy distribution of residual signals for MHIntra prediction is unbalanced. Specifically, it can be assumed that the residual signal near the top and left block boundary is lower than that of bottom and right region of the residual signal. This means that it might be inappropriate to apply DCT-VIII for blocks to which MHIntra prediction is applied.

Proposed method

In the proposed implicit transform selection, DST-VII or DCT-II is applied to MHInra predicted luma blocks according to the block size as shown in Table 1.

Table 1. Transform cores applied to MHIntra predicted block according to width/height condition

| Condition | width > Th && height <= Th | height > Th && width <= Th | height <= Th && width <= Th |
| --- | --- | --- | --- |
| Horizontal | DCT-II | DST-VII | DST-VII |
| Vertical | DST-VII | DCT-II | DST-VII |

Experimental results

The proposed algorithm was implemented on top of VTM-3.0. Simulations have been conducted under common test condition (CTC) and CTC with inter MTS enabled. The simulation results of random access, and low delay B configuration against VTM-3.0 with inter MTS enabled are shown in Table 1 and 2. Simulation results show negligible impact on BD-rate performance with reducing 4% of encoding time.

Table 1. Performance of the proposal against VTM-3.0 with inter MTS enabled (RA condition)

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.03% | -0.01% | 0.00% | 95% | 100% |
| Class A2 | 0.02% | -0.14% | -0.08% | 96% | 100% |
| Class B | 0.01% | -0.10% | -0.04% | 96% | 99% |
| Class C | 0.00% | -0.10% | -0.04% | 96% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.09% | -0.04% | 96% | 100% |
| Class D | 0.00% | -0.20% | -0.07% | 97% | 102% |
| Class F | -0.02% | 0.02% | -0.05% | 97% | 100% |

Table 2. Performance of the proposal against VTM-3.0 with inter MTS enabled (LB condition)

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | TBP | TBP | TBP | TBP | TBP |
| Class C | 0.01% | -0.15% | -0.06% | 97% | 100% |
| Class E | 0.01% | -0.16% | 0.37% | 99% | 99% |
| Overall | TBP | TBP | TBP | TBP | TBP |
| Class D | -0.04% | -0.06% | -0.15% | 98% | 100% |
| Class F | -0.12% | 0.57% | -0.05% | 97% | 100% |

Table 3 and 4 show the simulation results of random access and low delay B configuration against VTM-3.0 CTC.

Table 3. Performance of the proposal against VTM-3.0 (RA condition)

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.26% | 0.43% | 0.56% | 129% | 102% |
| Class A2 | -0.75% | 0.45% | 0.52% | 135% | 102% |
| Class B | -0.54% | 0.68% | 0.63% | 139% | 102% |
| Class C | -0.33% | 0.73% | 0.75% | 150% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.47% | 0.60% | 0.63% | 139% | 102% |
| Class D | -0.47% | 0.57% | 0.67% | 153% | 102% |
| Class F | -0.12% | 0.39% | 0.31% | 133% | 101% |

Table 4. Performance of the proposal against VTM-3.0 (LB condition)

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | TBP | TBP | TBP | TBP | TBP |
| Class C | -0.71% | 1.18% | 1.22% | 169% | 107% |
| Class E | -0.34% | 1.37% | 1.65% | 142% | 104% |
| Overall | TBP | TBP | TBP | TBP | TBP |
| Class D | -0.84% | 1.50% | 1.04% | 172% | 104% |
| Class F | -0.49% | 1.02% | 0.83% | 148% | 102% |

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and nondiscriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).