| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0484 |
| --- | --- |

| Title: | Non-CE4: Line buffer size reduction method for generalized bi prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Timofey Solovyev, Han Gao, Semih Esenlik, Sergey Ikonin, Jianle Chen | Tel: Email: | +79250089652 solovyev.timofey@huawei.com han.gao@huawei.com semih.esenlik@huawei.com sergey.ikonin@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution describes modification of generalized bi prediction index derivation process for merge mode and for affine mode. In order to reduce the line buffer size, borrowing of generalized bi prediction index is restricted to be within the current CTU line. The simulations results reportedly show 0.02% and 0.06% luma BD-rate loss for RA and LDB configurations respectively, without significant change in encoding and decoding times.

# Introduction

During the 12th JVET meeting in Macau, the generalized bi prediction [1] has been adopted into the VVC specification draft [2]. With GBi, the bi-prediction Pbi-pred is generated as following:

Pbi-pred=8-w*P0+w*P1+4≫3

where P0, P1 are motion compensated predictions, obtained from reference picture in list 0 and list 1 respectively and w is weighting parameter for list 1 prediction represented in 1/8 precision. In current GBi design, there are 5 weights {-2/8, 3/8, 4/8, 5/8, 10/8} available for low-delay picture and 3 weights {3/8, 4/8, 5/8} for non-low-delay picture. GBi is combined with regular merge and affine merge modes. In case of regular merge, GBi index can be borrowed from one of spatial candidates. In case of affine merge, GBi index for inherited candidates is borrowed from the corresponding neighboring block.

# Problem Statement

The problem of current design is that GBi indices of coding units located on CTU bottom boundary should be stored in the line buffer, because borrowing this indices across the top CTU boundary is allowed for regular and affine merge modes. If picture width is equal to w, then (w/4)*3 bits should be additionally stored in the line buffer. For example for the 4K resolution ( 3840 / 4 ) * 3 = 2880 bits should be stored in line buffer.

# Proposed Solutions

The proposed modification consists of two parts:

- For regular merge list construction disable borrowing GBi indices across top CTU boundary:

  - prior to merge list construction determine whether current block is located on top CTU boundary

  - for blocks located on top CTU boundary during the derivation of spatial candidates B0, B1 and B2 set GBi index equal to the default value – value corresponding to weight 1/2.

- For inherited affine merge candidates disable borrowing GBi indices across top CTU boundary:

  - during inherited affine merge candidate derivation, determine whether corresponding neighbouring block belongs to one of above CTUs (this check is already included in inherited affine merge candidate derivation process in VTM 3.0)

  - for such inherited candidates set GBi index equal to the default value – value corresponding to weight 1/2.

# Simulation Results

The proposed method is evaluated for random-access (RA) and Low-delay-B (LDB) configurations using VTM-3.0 software according to the common test conditions JVET-L1010 [3]. The detailed experimental results are provided in Table 1 and Table 2.

Table 1. Coding performance of proposed method over VTM-3.0 (RA)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.08% | 0.11% | 101% | 100% |
| Class A2 | -0.02% | 0.16% | 0.13% | 101% | 101% |
| Class B | 0.05% | 0.17% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.02% | 0.00% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.11% | 0.05% | 101% | 101% |
| Class D | 0.01% | -0.11% | 0.02% | 100% | 102% |
| Class F (optional) | 0.05% | 0.07% | 0.08% | 101% | 102% |

Table 2. Coding performance of proposed method over VTM-3.0 (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.12% | 0.19% | 100% | 100% |
| Class C | 0.02% | 0.35% | 0.19% | 100% | 100% |
| Class E | 0.08% | 0.25% | 0.15% | 100% | 101% |
| Overall | 0.06% | 0.13% | 0.18% | 100% | 100% |
| Class D | -0.03% | -0.05% | -0.07% | 100% | 100% |
| Class F (optional) | 0.18% | 0.29% | -0.36% | 100% | 101% |

# Conclusion

The proposed method changes regular and affine merge candidate list construction procedures disabling borrowing of generalized bi prediction index across top CTU boundary. This reduces size of the required line buffer by 2880 bits for the 4K resolution. The simulations results reportedly show 0.02% and 0.06% change in luma BD-rate for RA and LDB configurations respectively, without significant change in encoding and decoding times.

# References

- Y.-C. Su, Y. He, et al., “CE4-related: Generalized bi-prediction improvements combined from JVET-L0197 and JVET-L0296”, JVET-L0646, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- B. Bross, J. Chen, S. Liu “Versatile Video Coding (Draft 3)”, JVET-L01001_v10, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).