#### 8.4.6.3 Fractional sample interpolation process

##### 8.4.6.3.1 General

Inputs to this process are:

- a luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture,

- a variable sbWidth specifying the width of the current coding subblock,

- a variable sbHeight specifying the height of the current coding subblock,

- a motion vector mvLX,

- the selected reference picture sample array refPicLX,

- the bidirectional optical flow flag bdofFlag,

- a variable cIdx specifying the colour component index of the current block.

Outputs of this process are:

- an (sbWidth +bdofOffset)x(sbHeight +bdofOffset) array predSamplesLX of prediction sample values.

The bidirectional optical flow boundary offset bdofOffset is derived as follows:

bdofOffset = bdofFlag  ?  2  :  0		(8-691)

- If cIdx is equal to 0, the following applies:

  - Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in 1/16-sample units. These variables are used only in this clause for specifying fractional-sample locations inside the reference sample arrays refPicLX.

  - For each luma sample location ( xL = 0..sbWidth − 1 +bdofOffset, yL = 0..sbHeight − 1 +bdofOffset ) inside the prediction luma sample array predSamplesLX, the corresponding prediction luma sample value predSamplesLX[ xL ][ yL ] is derived as follows:

- The variables xIntL, yIntL, xFracL and yFracL are derived as follows:

xIntL = xSb + ( mvLX[ 0 ]  >>  4 ) + xL		(8-692)

yIntL = ySb + ( mvLX[ 1 ]  >>  4 ) + yL		(8-693)

xFracL = mvLX[ 0 ] & 15		(8-694)

yFracL = mvLX[ 1 ] & 15		(8-695)

  - If bdofFlag is equal to TRUE and one or more of the following conditions are true, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma integer sample bilinear interpolation filtering generation process as specified in clause 8.4.6.3.3 with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicLX as inputs:

      - bdofFlag is equal to TRUE.

      - xL is equal to 0.

      - xL is equal to sbWidth + 1.

      - yL is equal to 0.

      - yL is equal to sbHeight + 1.

  - Otherwise, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma sample 8-tap interpolation filtering process as specified in clause 8.4.6.3.2 with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicLX as inputs.

- Otherwise (cIdx is not equal to 0), the following applies:

  - Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in 1/32 sample units. These variables are used only in this clause for specifying general fractional-sample locations inside the reference sample arrays refPicLX.

  - For each chroma sample location ( xC = 0..sbWidth − 1, yC = 0.. sbHeight − 1 ) inside the prediction chroma sample arrays predSamplesLX, the corresponding prediction chroma sample value predSamplesLX[ xC ][ yC ] is derived as follows:

- The variables xIntC, yIntC, xFracC and yFracC are derived as follows: [Ed. (SL): Shall we make it consistent: /2 or /SubWidthC and /SubHeightC?]

xIntC = ( xSb / SubWidthC ) + ( mvLX[ 0 ]  >>  5 ) + xC	(8-696)

yIntC = ( ySb / SubHeightC ) + ( mvLX[ 1 ]  >>  5 ) + yC	(8-697)

xFracC = mvLX[ 0 ] & 31		(8-698)

yFracC = mvLX[ 1 ] & 31		(8-699)

  - The prediction sample value predSamplesLX[ xC ][ yC ] is derived by invoking the process specified in clause 8.4.6.3.4 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicLX as inputs.

##### 8.4.6.3.3 Luma integer sample bilinear interpolation filtering generation process

Inputs to this process are:

- a luma location in full-sample units ( xIntL, yIntL ),

- a luma location in fractional-sample units ( xFracL, yFracL ),

- the luma reference sample array refPicLXL,

Output of this process is a predicted luma sample value predSampleLXL

The variable shift1 is set equal to Min( 4, BitDepthY − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthY ).

The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 8-10.

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

xInti = sps_ref_wraparound_enabled_flag  ?  ClipH( sps_ref_wraparound_offset, picW, ( xIntL + i − 3 ) )	(8-707)
	:  Clip3( 0, picW − 1, xIntL + i )

yInti = Clip3( 0, picH − 1, yIntL + i )		(8-708)

The predicted luma sample value predSampleLXL is derived as follows:

–	If both xFracLand yFracL are equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL = refPicLXL[ xInt0 ][ yInt0 ] << shift3	(8-709)

–	Otherwise, if xFracL is not equal to 0 and yFracL is equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=01fL[ xFracL ][ i ] * refPicLXL[ xInti ][ yInt0 ]   >>  shift1	(8-710)

–	Otherwise, if xFracL is equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=01fL[ yFracL ][ i ] * refPicLXL[ xInt0 ][ yInti ]   >>  shift1	(8-711)

–	Otherwise, if xFracL is not equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

- The sample array temp[ n ] with n = 0..1, is derived as follows:

temp[ n ] =  i=01fL[ xFracL ][ i ] * refPicLXL[ xInti ][ yIntn ]   >>  shift1	(8-712)

- The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL =  i=01fL[ yFracL ][ i ] * temp[ i ]   >>  shift2	(8-713)

Table 8-10 – Specification of the luma bilinear interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p.

| Fractional sample position p | interpolation filter coefficients |  |
| --- | --- | --- |
|  | fL[ p ][ 0 ] | fL [ p ][ 1 ] |
| 1 | 60 | 4 |
| 2 | 56 | 8 |
| 3 | 52 | 12 |
| 4 | 48 | 16 |
| 5 | 44 | 20 |
| 6 | 40 | 24 |
| 7 | 36 | 28 |
| 8 | 32 | 32 |
| 9 | 28 | 36 |
| 10 | 24 | 40 |
| 11 | 20 | 44 |
| 12 | 16 | 48 |
| 13 | 12 | 52 |
| 14 | 8 | 56 |
| 15 | 4 | 60 |