| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0488-v2 |
| --- | --- |

| Title: | CE-2.4.5: Sub-block MV clip in affine prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Gao, Xiang Li, Meng Xu, Shan Liu | Tel: Email: | maxwellgao@tencent.com xlxiangli@tencent.com mengxxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the simulation results of CE-2.4.5 to reduce the memory bandwidth in affine prediction, where the MVs of four 4x4 sub-blocks within one 8x8 block are constrained such that the max difference between integer parts of the four 4x4 sub-block MVs is no more than 1 pixel. The simulation is conducted on VTM-3.0. It is reported that the worst-case memory bandwidth requirement by affine prediction is reduced from 15.125 luma sample per pixel to 8 luma sample per pixel at the cost of minor BD-rate increase (0.01% and -0.02% for RA and LDB configurations, respectively).

Introduction

In affine prediction, the uni-directional inter prediction for 4x4 block needs to load up to (4+7)*(4+7)=121 luma samples, thus the per-pixel memory bandwidth is 7.6 luma sample per pixel. For bi-directional affine prediction, the per-pixel memory bandwidth is up to 15.125 luma sample per pixel. To reduce the per-pixel memory bandwidth in affine prediction, in JVET-L0317 [1], the MVs of four 4x4 sub-blocks within one 8x8 block are constrained such that the max difference between integer parts of the four 4x4 sub-block MVs is no more than 1 pixel. Consequently, the worst case memory bandwidth is reduced to (8+7+1)x(8+7+1)/(8x8)x2=8 luma sample per pixel. The proposed method was decided to be further studied in Core Experiment 2 [2] as CE-2.4.5.

Algorithm description

In the proposed method, the MVs of all 4x4 sub-blocks inside the current block are firstly calculated with affine model. Then, the MVs of all four 4x4 sub-blocks within each 8x8 block are clipped to guarantee the max difference between integer parts of the four MVs no more than 1 pixel.

The clipping process applied to each 8x8 block is described as follows:

- The minimal and maximal values for the MV components, MVminx, MVminy, MVmaxx, MVmaxy are firstly determined for each 8x8 block as follows:

  - Get the minimal MV component among the four 4x4 sub-block MVs

MVminx = min(min(MVx0, MVx1), min(MVx2, MVx3))

MVminy = min(min(MVy0, MVy1), min(MVy2, MVy3))

  - Use the integer part of MVminx and MVminy as the minimal MV component

MVminx = MVminx & 0xfffffff0

MVminy = MVminy & 0xfffffff0

  - The maximal MV component is calculated as follows:

MVmaxx = MVminx + (2 << MV_precision) – 1

MVmaxy = MVminy + (2 << MV_precision) – 1

- The MV components of each 4x4 block inside this 8x8 block are clipped as follows:

MVxi = min(MVmaxx, MVxi)

MVyi = min(MVmaxy, MVyi)

where (MVxi, MVyi) is the MV of ith sub-block within one 8x8 block, where i is 0, 1, 2, 3; MV_precision is equal to 4 corresponding to 1/16 motion vector fraction accuracy. Since the difference between integer parts of MVminx and MVmaxx (MVminy and MVmaxy) is 1 pixel, the max difference between integer parts of the four 4x4 sub-block MVs is no more than 1 pixel.

It should be noted that the proposed method can be implemented with SIMD optimization. Sample code is also included in CE software.

Simulation results

The proposed method is implemented on the top of VTM-3.0. The common test conditions defined in [3] is used in the simulation. Table 1 shows the BD-Rate of the proposed method relative to VTM-3.0. Please note that the running time in Table 1 is not reliable due to system issues.

Table. 1. The BD-Rate of the proposed method relative to VTM-3.0.

Conclusion

This contribution reports the simulation results of the method, which clip the MVs of four 4x4 sub-blocks within one 8x8 block such that the max difference between integer parts of the four 4x4 sub-block MVs is no more than 1 pixel. It is reported that the worst-case memory bandwidth in affine prediction is significantly reduced at the cost of minor BD-rate increase. It is proposed to adopt this method.

Reference

[1] M. Gao, X. Li, M. Xu and S. Liu, JVET-L0317, “CE4-related: Sub-block MV clipping in affine prediction”, Oct. 2018.

[2] Y. He, C. Y. Chen, and C. C. Chen, JVET-L1022, “Description of Core Experiment 2 (CE2): Sub-block based motion prediction”, Oct. 2018.

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, JVET-L1010, “JVET common test conditions and software reference configurations for SDR video”, Oct. 2018.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).