| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0492 |
| --- | --- |

| Title: | CE10-related: Simplified multi-hypothesis intra-inter mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Xiang Li and Shan Liu | Email: | {leolzhao, xinzzhao, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, a simplified multi-hypothesis intra-inter mode is proposed. Firstly, the size of intra candidate mode list of intra-inter mode is reduced from 4 to 1 such that if the signaled intra-inter mode flag is true, extra intra mode coding is not needed. Secondly, for the intra-inter prediction process, PDPC filter is directly applied to the inter prediction samples to get the final prediction values, and intra smoothing filter, intra prediction process, and weighted intra and inter prediction process are avoided. It is reportedly shown that, the proposed method achieved 0.05% and -0.05% luma BD-rates with 99% and 99% encoding time and 98% and 98% decoding time for RA and LDB respectively.

Introduction

In VTM3.0 [1], multi-hypothesis intra-inter prediction [2] combines intra prediction and merge indexed inter prediction. In a merge CU, an intra prediction mode index is signaled for merge mode to select an intra prediction mode from an intra candidate mode list when intra-inter prediction is used. For luma component, the intra candidate mode list includes 4 intra prediction modes: DC, planar, horizontal, and vertical modes, and the size of the intra candidate mode list can be 3 or 4 depending on the block shape. When the CU width is larger than the double of CU height, horizontal mode is removed from the intra mode list and when the CU height is larger than the double of CU width, vertical mode is removed from the intra mode list. For chroma component, DM is always applied without extra signaling.

In addition to getting the inter prediction sample values, intra-inter prediction process in VTM3.0 includes the following 5 steps: 1) reference sample availability check and substitution, 2) reference sample filtering, 3) horizontal and vertical planar predictor generation and averaging, 4) PDPC, and 5) weighted average of intra prediction and inter prediction. In Step 5, one extra buffer is added to store the intra prediction values.

Proposed method

Firstly, it is proposed to reduce the size of intra candidate list of intra-inter mode from 4 to 1 such that if the signaled intra-inter mode flag is true, extra signaling is avoided.

Secondly, for the intra-inter prediction process, PDPC filter is directly applied to the inter prediction samples to get the final prediction values, and steps (2), (3) and (5) are avoided. In addition, no extra buffer is needed in the proposed method since the intra prediction process is not used. To be specific, the PDPC filtering process for intra-inter mode is formulated as follows, and changes are highlighted:

shift = ( log2( width ) + log2( height ) – 2 ) >> 2.               (1)

wT = 32 >> ( ( y<<1 ) >> shift ), wL = 32 >> ( ( x<<1 ) >> shift ), wTL = ( wL>>4 ) + ( wT>>4 ),    (2)

pred(x,y) = ( wL × R-1,y + wT × Rx,-1 – wTL × R-1,-1 + (64 – wL – wT + wTL) × Interpred(x,y) + 32 )  >>  6,   (3)

wherein Rx,-1, R-1,y represent the reference samples located at the top and left of current sample (x, y), respectively, R-1,-1 represents the reference sample located at the top-left corner of the current block, and Interpred(x,y) represents the inter prediction sample values.

Alternatively, in this contribution, the following PDPC filtering process as shown in equation (4), wherein the top-left neighboring sample is excluded from the filter process, is also tested.

pred(x,y) = ( wL × R-1,y + wT × Rx,-1 +  (64 – wL – wT ) × Interpred(x,y) + 32 )  >>  6,		(4)

Experimental results

The proposed method is implemented on top of VTM3.0 software [1]. The simulations are performed on Linux cluster using VTM RA and LDB configurations under the common test condition described in JVET-L1010 [3].

In the following two tests, both the first and second simplifications are applied. Equation (3) is employed in the first test whereas equation (4) is employed in the second test. The test results are provided in Table 1 and Table 2.

Table 1: Coding performance of Test 1 over VTM-3.0

|  | Random Access Main10 |  |  |  |  | Low Delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.22% | -0.01% | 98% | 96% |  |  |  |  |  |
| Class A2 | 0.04% | 0.02% | 0.21% | 97% | 97% |  |  |  |  |  |
| Class B | 0.05% | -0.03% | -0.05% | 99% | 97% | -0.08% | -0.19% | -0.10% | 99% | 97% |
| Class C | 0.08% | 0.06% | 0.15% | 100% | 100% | -0.02% | 0.09% | 0.06% | 98% | 98% |
| Class E |  |  |  |  |  | -0.05% | 0.09% | -0.55% | 99% | 98% |
| Overall | 0.05% | -0.04% | 0.06% | 99% | 98% | -0.05% | -0.03% | -0.16% | 99% | 98% |
| Class D | 0.03% | -0.13% | 0.04% | 100% | 102% | -0.08% | -0.15% | 0.08% | 100% | 100% |
| Class F | 0.00% | -0.07% | 0.00% | 101% | 100% | 0.01% | -0.51% | -0.22% | 99% | 98% |

Table 2: Coding performance of Test 2 over VTM-3.0

|  | Random Access Main10 |  |  |  |  | Low Delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.15% | -0.08% | 100% | 101% |  |  |  |  |  |
| Class A2 | 0.04% | 0.16% | 0.22% | 99% | 100% |  |  |  |  |  |
| Class B | 0.08% | 0.12% | -0.02% | 99% | 97% | -0.06% | -0.22% | 0.07% | 101% | 103% |
| Class C | 0.04% | 0.01% | 0.13% | 100% | 101% | -0.04% | -0.11% | 0.16% | 99% | 93% |
| Class E |  |  |  |  |  | -0.01% | 0.17% | -0.72% | 100% | 94% |
| Overall | 0.05% | 0.04% | 0.05% | 99% | 99% | -0.04% | -0.09% | -0.10% | 100% | 97% |
| Class D | 0.00% | 0.04% | -0.02% | 99% | 98% | -0.08% | -0.37% | -0.34% | 100% | 100% |
| Class F | 0.02% | -0.05% | 0.01% | 99% | 100% | 0.23% | 0.27% | 0.22% | 98% | 99% |

Conclusion

In this contribution, a simplified multi-hypothesis intra-inter mode is proposed, wherein the size of intra candidate list of intra-inter mode is reduced from 4 to 1, and PDPC filter is directly applied to the inter prediction samples to get the final prediction values. It is reportedly shown that 0.05% and -0.05% luma BD-rates is achieved with about small encoding time decrease in RA and LDB configuration respectively. Therefore, it is proposed to adopt this proposal.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, and S.-M. Lei “CE10.1.1: Multi-hypothesis prediction for improving AMVP mode, skip or merge mode, and intra mode,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L0100, Macao, CN, 3–12 Oct. 2018.

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L1010, Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).