| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0493 |
| --- | --- |

| Title: | CE3-related: Simplified look-up table for CCLM mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Xiang Li, and Shan Liu | Email: | {leolzhao, xinzzhao, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In VTM3.0, two look-up tables (LUT) are employed in CCLM mode to avoid division operations, and size of each LUT is 512. In this contribution, one LUT is removed and the size of the remaining LUT is reduced by 87.5% (from 512 to 64). It is reportedly shown that, the proposed method achieved 0.01% and 0.00% luma BD-rates with around 100% encoding and decoding time for All Intra (AI) and Random Access (RA) configurations, respectively.

Introduction

In VTM3.0 [1], min-max method [2] was adopted to reduce the complexity of CCLM mode. Parameters α and β are derived by 2 pairs A and B, of which the luma components are the minimum and maximum values inside the set of neighboring Luma samples as depicted in Figure 1.

Y=αx+β

A

B

Figure 1: Illustration of the straight line between minimum and maximum Luma value

The linear model parameters α and β are obtained according to the following equation, where x denotes luma component and y represents chroma component:

α=yB-yAxB-xA

β=yA- αxA

The division operation in the above equation can be avoided and replaced by a multiplication and a shift in the following equations, wherein variables maxY, minY, maxC, and minC denote the luma sample value of Point B,  luma sample value of Point A, chroma sample value of Point B, and  chroma sample value of Point A respectively.

shift = ( BitDepthC > 8 )  ?  BitDepthC − 9  :  0	(8-153)

add = shift  ?  1 << ( shift − 1)  :  0		(8-154)

diff = ( maxY − minY + add ) >> shift		(8-155)

k = 16		(8-156)

    - If diff is greater than 0, the following applies:

div = ( ( maxC − minC ) * ( Floor( 232 / diff ) − Floor( 216 / diff ) * 216 ) + 215 ) >> 16 (8-157)

a = ( ( maxC − minC ) * Floor( 216 / diff ) + div + add ) >> shift	(8-158)

Since the range of variable “diff” in equation (8-157) and (8-158) is from 1 ~ 512, ( Floor( 232 / diff ) − Floor( 216 / diff ) * 216 )  and Floor( 216 / diff ) can only have 512 values. In VTM3.0, these operations are pre-calculated and stored in two different look-up tables (LUT), namely g_aiLMDivTableLow and g_aiLMDivTableHigh. In addition, the value of diff is used to specify the entry index of the LUT.

Proposed method

Firstly, the LUT (g_aiLMDivTableLow) is removed and equation (8-158) is modified as follows.

a = ( ( maxC − minC ) * Floor( 216 / diff ) + add ) >> shift	         (8-158)

Secondly, the range of diff is divided into multiple intervals, and diff is further quantized for each interval. After that, the quantized value of diff is used to specify the entry index of LUT. Three alternative tests with different thresholds and quantization step sizes are shown below:

- In test 1, the size of LUT is reduced to 120. When diff is larger than 64, the step size is set equal to 8; otherwise, the step size is set equal to 1, as shown below:

diff=56+(diff≫3),  &diff>64diff,  &diff≤64

- In test 2, the size of LUT is reduced to 88. When diff is larger than 64, the step size is set equal to 8; otherwise, the step size is set equal to 2, as shown below:

diff=24+(diff≫3),  &diff>64(diff≫1),  &diff≤64

- In test 3, the size of LUT is reduced to 64. When diff is larger than 256, the step size is set equal to 32; otherwise, if diff is larger than 64, the step size is set equal to 8; otherwise, the step size is set equal to 2.

diff=48+(diff≫5),  &diff>25624+(diff≫3),  &64<diff≤256(diff≫1),  &diff≤64

Experimental results

The proposed method is implemented on top of VTM3.0 software [1]. The simulations are performed on Linux cluster using AI and RA configurations under the common test condition described in JVET-L1010 [3].

Three alternative tests mentioned in Section 2 are conducted, wherein the first modification (removing one LUT) is included in all 3 tests. The experimental results of Test 1 ~ Test 3 are provided in Table 1 ~ Table 3, respectively.

Table 1: Coding performance of Test 1 (LUT size 120) over VTM-3.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.26% | 0.02% | 97% | 96% | 0.02% | -0.27% | 0.00% | 94% | 96% |
| Class A2 | 0.00% | -0.08% | 0.00% | 97% | 97% | 0.01% | 0.12% | 0.13% | 96% | 98% |
| Class B | 0.01% | -0.06% | -0.10% | 99% | 99% | -0.01% | 0.09% | -0.10% | 95% | 96% |
| Class C | -0.02% | -0.10% | -0.16% | 98% | 98% | 0.01% | -0.01% | -0.03% | 95% | 95% |
| Class E | 0.01% | 0.01% | 0.01% | 97% | 97% |  |  |  |  |  |
| Overall | 0.00% | -0.09% | -0.06% | 98% | 98% | 0.01% | 0.00% | -0.02% | 95% | 96% |
| Class D | 0.00% | -0.10% | -0.02% | 98% | 98% | -0.02% | -0.09% | -0.11% | 95% | 95% |
| Class F | 0.00% | -0.06% | 0.04% | 97% | 97% | -0.02% | 0.10% | 0.05% | 93% | 94% |

Table 2: Coding performance of Test 2 (LUT size 88) over VTM-3.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.17% | 0.03% | 98% | 98% | 0.05% | -0.22% | 0.05% | 96% | 98% |
| Class A2 | 0.00% | -0.07% | -0.04% | 98% | 98% | -0.02% | -0.03% | 0.07% | 96% | 100% |
| Class B | 0.00% | -0.07% | -0.08% | 100% | 102% | 0.00% | 0.03% | -0.16% | 96% | 101% |
| Class C | 0.00% | -0.06% | -0.16% | 98% | 100% | 0.00% | -0.02% | 0.16% | 95% | 100% |
| Class E | 0.01% | 0.09% | 0.06% | 99% | 101% |  |  |  |  |  |
| Overall | 0.00% | -0.06% | -0.05% | 99% | 100% | 0.00% | -0.04% | 0.01% | 96% | 100% |
| Class D | 0.00% | -0.04% | 0.11% | 100% | 101% | -0.01% | -0.37% | -0.06% | 95% | 99% |
| Class F | 0.02% | -0.09% | 0.02% | 98% | 100% | -0.01% | 0.04% | 0.02% | 94% | 99% |

Table 3: Coding performance of Test 3 (LUT size 64) over VTM-3.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.20% | -0.01% | 99% | 99% | 0.06% | -0.20% | 0.03% | 96% | 96% |
| Class A2 | 0.00% | -0.04% | -0.05% | 98% | 98% | -0.01% | 0.08% | 0.08% | 97% | 99% |
| Class B | 0.00% | -0.07% | -0.15% | 99% | 99% | -0.02% | 0.06% | -0.08% | 96% | 97% |
| Class C | -0.01% | -0.08% | -0.11% | 100% | 96% | 0.00% | 0.03% | 0.13% | 96% | 95% |
| Class E | 0.02% | -0.04% | 0.10% | 100% | 99% |  |  |  |  |  |
| Overall | 0.01% | -0.08% | -0.06% | 99% | 98% | 0.00% | 0.00% | 0.03% | 96% | 97% |
| Class D | -0.01% | -0.09% | 0.05% | 99% | 98% | 0.01% | -0.44% | -0.19% | 96% | 96% |
| Class F | 0.01% | 0.07% | -0.01% | 98% | 98% | 0.01% | 0.07% | 0.08% | 95% | 94% |

Spec text changes

The following text describes test 3 of the proposed method. It is based on VVC draft3 v7 [4] section 8.2.4.2.8 The added texts are highlighted with in green, and the deleted texts are marked with strikethrough.

8.2.4.2.8	Specification of INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra prediction mode

Inputs to this process are:

–	the intra prediction mode predModeIntra,

–	a sample location ( xTbC, yTbC ) of the top-left sample of the current transform block relative to the top-left sample of the current picture,

–	a variable nTbW specifying the transform block width,

–	a variable nTbH specifying the transform block height,

–	chroma neighbouring samples p[ x ][ y ], with x = −1, y = 0..2 * nTbH − 1 and x = 0.. 2 * nTbW − 1, y = − 1.

Output of this process are predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The current luma location ( xTbY, yTbY ) is derived as follows:

( xTbY, yTbY )  =  ( xTbC << 1, yTbC << 1 )	               	(8-116)

……

- The variables a, b, and k are derived as follows:

    - If numSampL is greater than 0, and numSampT is greater than 0, the following applies:

k = 0		(8-150)

a = 0		(8-151)

b = 1 << ( BitDepthC − 1) 		(8-152)

    - Otherwise, the following applies:

shift = ( BitDepthC > 8 )  ?  BitDepthC − 89  :  0	(8-153)

add = shift  ?  1 << ( shift − 1)  :  0		(8-154)

diff = ( maxY − minY + add ) >> shift		(8-155)

k = 156		(8-156)

    - If diff is greater than 0, the following applies:

diff = (diff > 32) ? (diff > 128 ? (48 + (diff >> 4) : 24 + (diff >> 2)): diff         (8-157)

div = ( ( maxC − minC ) * ( Floor( 232 / diff ) − Floor( 216 / diff ) * 216 ) + 215 ) >> 16	(8-157)

a = ( ( maxC − minC ) * Floor( 216 / diff ) + div + add ) >> shift	(8-158)

    - Otherwise, the following applies:

a = 0	(8-159)

b = minC − ( ( a * minY ) >> k )		(8-160)

- The prediction samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0.. nTbH − 1 are derived as follows:

predSamples[ x ][ y ] = Clip1C( ( ( pDsY[ x ][ y ] * a ) >> k ) + b )	    (8-161)

Conclusion

In this contribution, one LUT is removed for CCLM mode and the size of the remaining LUT is reduced from 512 to 64. It is reportedly that, 0.01% and 0.00% BD-rates with around 100% encoding and decoding time is achieved in AI and RA configuration respectively. Therefore, it is proposed to adopt this proposal.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS

- G. Laroche, J. Taquet, C. Gisquet, and P. Onno “CE3-5.1: On cross-component linear model simplification,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L0191, Macao, CN, 3–12 Oct. 2018.

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L1010, Macao, CN, 3–12 Oct. 2018.

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 2),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L1001, Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).