| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0498-r1 |
| --- | --- |

| Title: | CE6: MTS up to 16-length (Test 6.3.8) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jason Jung, Dongcheol Kim, Geonjung Ko, Ju-Hyung (John) Son, Jin Sam Kwak (WILUS) Xin Zhao, Xiang Li, Shan Liu (Tencent) | Tel: Email: | jason.jung@wilusgroup.com xinzzhao@tencent.com |
| Source: | WILUS Inc., Tencent |  |  |

_____________________________

- Abstract

This contribution reports the test results of CE6-3.8 on Multiple Transform Selection (MTS) restriction using up to 16-point DST-7/DCT-8. With the proposed method, horizontal and/or vertical transform type can be signaled for blocks with width and/or height up to size 16, such that 32-point DST-7/DCT-8 transform is avoided which is deemed to be high complexity. It is reportedly shown that, on top of VTM-3.0, average 0.33%, 0.17% and 0.06% luma BD-Rate increase is observed under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, with 91%, 99% and 99% overall encoding time and 93% 99% and 100% overall decoding time, respectively.

Introduction

In VVC Draft 3 [1], MTS is applied up to 32x32, which means up to 32-point DST-7/DCT-8 transforms need to be supported. It was claimed that 32-point DST-7/DCT-8 with full matrix multiplication is a relatively high complexity burden because 32 multiplications are needed per sample for doing 32-point DST-7/DCT-8 in one direction.

Proposed method

To reduce the complexity of MTS, it is proposed that the size of DST-7/DCT-8 is limited to be up to 16-point. To achieve this goal, two Core Experiment 6 (CE6) [2] tests are defined, i.e., CE6-3.8a and CE6-3.8b, as described in the following Table 1.

Table 1: Descriptions on CE6 Test 6.3.8

| Test # | Description |
| --- | --- |
| 6-3.8a | Signal horizontal transform if width is less than or equal to 16-point and height is less than or equal to 32, and signal vertical transform if height is less than or equal to 16-point and and width is less than or equal to 32 |
| 6-3.8b | Signal horizontal transform if width is less than or equal to 16-point and height is less than or equal to 64, and signal vertical transform if height is less than or equal to 16-point and and width is less than or equal to 64 |

With the proposed method, horizontal transform selection is enabled for a block if the block width is smaller than or equal to 16, and vertical transform selection is enabled for a block if the block height is smaller than or equal to 16. In addition, for large blocks with sizes larger than a threshold, e.g., 32x32 or 64x64, transform selection is not signaled for either horizontal or vertical direction.

Results

Experiments are conducted under CTC condition [3] and the additional test condition (VTM-3.0 with inter MTS enabled) defined in CE6 description [2]. Detailed results are reported in Table 2 and Table 3 for CE6.3.8a and CE6.3.8b, respectively.

Table 2: Coding performance of Test CE6-3.8a

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |  | Over VTM-3.0, InterMTS on |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y |  | U | V | EncT | DecT |
| Class A1 | 0.66% | 0.99% | 1.05% | 89% | 88% | 0.66% |  | 0.99% | 1.05% | 89% | 89% |
| Class A2 | 0.35% | 0.40% | 0.36% | 92% | 92% | 0.35% |  | 0.40% | 0.36% | 92% | 92% |
| Class B | 0.33% | 0.40% | 0.42% | 91% | 91% | 0.33% |  | 0.40% | 0.42% | 92% | 91% |
| Class C | 0.09% | 0.09% | -0.10% | 93% | 97% | 0.09% |  | 0.09% | -0.10% | 94% | 96% |
| Class E | 0.29% | 0.24% | 0.33% | 93% | 94% | 0.29% |  | 0.24% | 0.33% | 94% | 95% |
| Overall | 0.33% | 0.40% | 0.38% | 91% | 92% | 0.33% |  | 0.40% | 0.38% | 92% | 92% |
| Class D | 0.06% | -0.04% | 0.06% | 94% | 98% | 0.06% |  | -0.04% | 0.06% | 95% | 97% |
| Class F | 0.07% | 0.11% | 0.17% | 92% | 94% | 0.07% |  | 0.11% | 0.17% | 94% | 97% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |  | Over VTM-3.0, InterMTS on |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y |  | U | V | EncT | DecT |
| Class A1 | 0.34% | 0.57% | 0.82% | 98% | 98% | 0.47% |  | 0.44% | 0.69% | 91% | 97% |
| Class A2 | 0.13% | 0.27% | 0.28% | 98% | 99% | 0.28% |  | 0.15% | 0.16% | 92% | 98% |
| Class B | 0.20% | 0.61% | 0.50% | 98% | 98% | 0.32% |  | 0.39% | 0.38% | 92% | 98% |
| Class C | 0.08% | 0.07% | 0.37% | 99% | 102% | 0.09% |  | 0.20% | 0.30% | 91% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.18% | 0.39% | 0.48% | 98% | 99% | 0.28% |  | 0.30% | 0.38% | 91% | 98% |
| Class D | 0.02% | -0.36% | -0.24% | 98% | 99% | 0.10% |  | -0.35% | -0.01% | 93% | 98% |
| Class F | 0.05% | 0.12% | 0.27% | 98% | 101% | 0.06% |  | 0.09% | 0.26% | 92% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |  | Over VTM-3.0, InterMTS on |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y |  | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.12% | -0.15% | -0.01% | 99% | 102% | 0.37% |  | 0.27% | 0.02% | 89% | 96% |
| Class C | 0.06% | 0.30% | 0.33% | 99% | 97% | 0.15% |  | 0.02% | 0.26% | 90% | 97% |
| Class E | 0.00% | 0.94% | 0.31% | 99% | 99% | 0.23% |  | 0.31% | -0.42% | 93% | 97% |
| Overall | 0.07% | 0.27% | 0.19% | 99% | 100% | 0.26% |  | 0.20% | -0.01% | 90% | 96% |
| Class D | 0.07% | 0.28% | 0.55% | 100% | 102% | 0.12% |  | -0.22% | -0.39% | 93% | 99% |
| Class F | 0.14% | 0.50% | 0.05% | 98% | 97% | 0.04% |  | -0.04% | -0.19% | 91% | 98% |

Table 3: Coding performance of Test CE6-3.8b

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |  | Over VTM-3.0, InterMTS on |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y |  | U | V | EncT | DecT |
| Class A1 | 0.66% | 0.99% | 1.05% | 89% | 89% | 0.66% |  | 0.99% | 1.05% | 89% | 89% |
| Class A2 | 0.35% | 0.40% | 0.36% | 91% | 93% | 0.35% |  | 0.40% | 0.36% | 92% | 92% |
| Class B | 0.33% | 0.40% | 0.42% | 92% | 92% | 0.33% |  | 0.40% | 0.42% | 92% | 91% |
| Class C | 0.09% | 0.09% | -0.10% | 93% | 97% | 0.09% |  | 0.09% | -0.10% | 94% | 96% |
| Class E | 0.29% | 0.24% | 0.33% | 92% | 93% | 0.29% |  | 0.24% | 0.33% | 93% | 94% |
| Overall | 0.33% | 0.40% | 0.38% | 91% | 93% | 0.33% |  | 0.40% | 0.38% | 92% | 93% |
| Class D | 0.06% | -0.04% | 0.06% | 96% | 100% | 0.06% |  | -0.04% | 0.06% | 97% | 99% |
| Class F | 0.07% | 0.11% | 0.17% | 94% | 97% | 0.07% |  | 0.11% | 0.17% | 96% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |  | Over VTM-3.0, InterMTS on |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y |  | U | V | EncT | DecT |
| Class A1 | 0.31% | 0.33% | 0.57% | 98% | 97% | 0.43% |  | 0.33% | 0.58% | 94% | 97% |
| Class A2 | 0.12% | 0.24% | 0.25% | 98% | 99% | 0.25% |  | 0.15% | 0.17% | 94% | 98% |
| Class B | 0.18% | 0.50% | 0.43% | 98% | 97% | 0.30% |  | 0.32% | 0.32% | 93% | 98% |
| Class C | 0.07% | 0.03% | 0.45% | 99% | 102% | 0.10% |  | 0.19% | 0.36% | 93% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.17% | 0.29% | 0.43% | 99% | 99% | 0.26% |  | 0.25% | 0.35% | 93% | 98% |
| Class D | 0.02% | -0.46% | -0.20% | 99% | 99% | 0.10% |  | -0.39% | 0.02% | 94% | 99% |
| Class F | 0.05% | 0.10% | 0.27% | 99% | 101% | 0.08% |  | 0.11% | 0.33% | 94% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |  | Over VTM-3.0, InterMTS on |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y |  | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.08% | -0.01% | 0.22% | 100% | 103% | 0.34% |  | 0.14% | -0.04% | 92% | 97% |
| Class C | 0.03% | 0.19% | 0.26% | 97% | 96% | 0.17% |  | -0.09% | 0.12% | 92% | 97% |
| Class E | 0.05% | 0.92% | 0.16% | 101% | 100% | 0.18% |  | 0.33% | 0.14% | 94% | 98% |
| Overall | 0.06% | 0.29% | 0.22% | 99% | 100% | 0.24% |  | 0.11% | 0.06% | 92% | 97% |
| Class D | 0.01% | 0.05% | 0.65% | 101% | 102% | 0.17% |  | -0.11% | 0.23% | 94% | 98% |
| Class F | -0.11% | -0.21% | -0.56% | 99% | 99% | 0.05% |  | 0.03% | -0.18% | 93% | 94% |

Complexity analysis

Memory requirements for storing transform cores

Memory requirements for storing DCT-2 cores are same as VTM 3 and the costs for storing MTS cores are describe in the following Table 4.

Table 4: Memory requirements for storing transform cores of MTS

| Transform size | VTM 3 |  | CE6-3.8 |  |
| --- | --- | --- | --- | --- |
|  | Number of transform types | Required bit | Number of transform types | Required bit |
| 4-point | 2 | 256 bits | 2 | 256 bits |
| 8-point | 2 | 1024 bits | 2 | 1024 bits |
| 16-point | 2 | 4096 bits | 2 | 4096 bits |
| 32-point | 2 | 16384 bits | 0 | 0 |
| Total | 8 | 2.72 KB | 6 | 0.672 KB |

Combinations of transform type and transforms block sizes

Table 5 and Table 6 summarize combinations of transform type and transforms block sizes for CE6-3.8a and CE6-3.8b, respectively.

Table 5 Combinations of transform type and transform block size in CE6-3.8a

| Block size | MTS_CU_flag | Primary transform type |  |
| --- | --- | --- | --- |
|  |  | Horizontal | Vertical |
| width <= 16 && height <= 16 | 0 | DCT-II | DCT-II |
|  | 1 | DST-VII / DCT-VIII | DST-VII / DCT-VIII |
| width <= 16 && height == 32 | 0 | DCT-II | DCT-II |
|  | 1 | DST-VII / DCT-VIII | DCT-II |
| width == 32 && height <= 16 | 0 | DCT-II | DCT-II |
|  | 1 | DCT-II | DST-VII / DCT-VIII |
| Others | 0 | DCT-II | DCT-II |

Table 6 Combinations of transform type and transform block size in CE6-3.8b

| Block size | MTS_CU_flag | Primary transform type |  |
| --- | --- | --- | --- |
|  |  | Horizontal | Vertical |
| width <= 16 && height <= 16 | 0 | DCT-II | DCT-II |
|  | 1 | DST-VII / DCT-VIII | DST-VII / DCT-VIII |
| width <= 16 && height > 16 && height <= 64 | 0 | DCT-II | DCT-II |
|  | 1 | DST-VII / DCT-VIII | DCT-II |
| width > 16 && width <= 64 && height <= 16 | 0 | DCT-II | DCT-II |
|  | 1 | DCT-II | DST-VII / DCT-VIII |
| Others | 0 | DCT-II | DCT-II |

Other complexity metrics

- Operation counts: Same as VTM 3.

- The minimum bit-precision required by the transforms: Same as VTM 3.

- Analysis of implementation and arithmetic commonalities: Same as VTM 3.

- Worst-case analysis (e.g., multiplications per pixels): Same as VTM 3.

Conclusions

In this contribution, a modified MTS scheme with restriction using up to 16-length transforms is proposed. With the proposed method, horizontal and/or vertical transform can be signaled if block width and/or height is smaller than or equal to 16. The proposed method avoids 32-point DST-7/DCT-8 and reduces both the encoder and decoder run-time for intra coding.

References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” JVET-L1001, 12th meeting, Macao, CN, 3–12 Oct. 2018.

- A. Said, X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signalling,” JVET-L1026, 12th meeting, Macao, CN, 3–12 Oct. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th meeting, Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).