| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET- M0503 |
| --- | --- |

| Title: | CE3: Chroma intra prediction simplification (Test 3.4.1 and 3.4.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chang-Hao Yau1, Ching-Chieh Lin2 and Chun-Lung Lin3 2870 Zanker Road, Suite 140, San Jose, CA 95134, USA | Tel: Email: | +886-3-5915059 changhao.yau@itri.com1 jacklin@itri.com2 chunlung@itri.com3 |
| Source: | ITRI International |  |  |

_____________________________

- Abstract

This contribution presents the results of chroma intra prediction when Multiple Direct Mode Signalling (MDMS) is used. Two tests were completed in this contribution. The first test (CE3-3.4.1) reduces the numbers of pruning checks for MDMS based on VTM3.0, and the second test (CE3-3.4.2) further simplifies MDMS by reducing 2 candidates. The AI results for CE3-3.4.1 show 0.02%, 0.96% and 1.03% coding gain in terms of average BD-rate of Y, Cb, and Cr without any change of the encoding time. The AI results for CE3-3.4.2 show 0.03% coding loss in terms of average BD-rate of Y, but the coding gain in terms of average BD-rate of Cb and Cr for AI is 0.34% and 0.32%. The encoding itme for CE3-3.4.2 is 3% less than VTM3.0.

Introduction

The chroma intra mode candidate list in BMS3.0 includes cross-component linear mode (CCLM), Multi-directional linear mode (MDLM), default modes and derived Mode (DM). The default modes have planar mode, vertical mode, horizontal mode and DC mode. The DM directly reuses the intra prediction mode of the centre luma coding block which the current chroma coding block corresponds to. If the DM is the same as the mode in default modes, the mode in default modes is replaced by vdiagonal mode.

Proposed method

CE3-3.4.1: simplified MDMS

Multiple Direct Mode Signalling (MDMS) is a chroma signalling method for the non-LM modes. In this test, the default modes and the DM for VTM3.0 (5 candidates) are replaced by applying MDMS. The concept of MDMS is similar to MPM in the luma component. In this test, there are 5 candidates for signalling. Because the original MDMS needs many pruning checks[1], this test reduces the numbers of pruning checks by using 2 DMs (bottom-right and centre). The derivation order of simplified MDMS is shown in Figure 1.

Figure 1. The derivation order of SMDMS

CE3-3.4.2: Simplify the number of MDMS candidate

The proposed modification reduces the numbers of MDMS candidates. In this test, there are 3 candidates for signalling and 2 DMs (top-right and bottom-left) are reused. The derivation order of MDMS is shown in Figure 2.

Figure 2. The derivation order of SMDMS

Experimental results

The proposed modifications are implemented based on VTM3.0 reference software. According to the CE3 description in JVET-L1023 [2] and the common test conditions in JVET-L1010 [3], the proposed methods were evaluated under intra-only (AI) and random-access (RA) configuration with VTM3.0 configuration.

CE3-3.4.1

Table 1 and table 2 show the experimental results of the proposed method (CE3-3.4.1) for AI and RA respectively. The AI results for CE3-3.4.1 show 0.02% (Y), 0.96 (U) and 1.03 (V) gain in terms of average BD-rate and the encoder run-time is not changed compared to VTM3.0 anchor.

Table 1. Results of the CE3-3.4.1 for all intra (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.62% | -0.82% | 101% | 100% |
| Class A2 | -0.01% | -0.76% | -0.66% | 100% | 100% |
| Class B | -0.01% | -0.90% | -1.09% | 101% | 100% |
| Class C | -0.03% | -0.86% | -1.12% | 100% | 99% |
| Class E | -0.01% | -1.71% | -1.38% | 100% | 99% |
| Overall | -0.02% | -0.96% | -1.03% | 101% | 99% |
| Class D | -0.01% | -0.55% | -0.65% | 100% | 99% |
| Class F (optional) | -0.03% | -0.46% | -0.62% | 100% | 99% |

Table 2. Results of the CE3-3.4.1 for random access (RA)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.60% | -0.55% | 100% | 100% |
| Class A2 | 0.06% | -1.09% | -0.59% | 100% | 100% |
| Class B | 0.04% | -0.98% | -1.65% | 100% | 100% |
| Class C | 0.07% | -1.16% | -0.97% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.98% | -1.04% | 100% | 100% |
| Class D | 0.07% | -1.53% | -0.91% | 101% | 100% |
| Class F (optional) | 0.04% | -0.46% | -0.55% | 100% | 100% |

CE3-3.4.2

Table 3 and Table 4 show the experimental results of the proposed method (CE3-3.4.2) for AI and RA respectively. The simulation results (CE3-3.4.2) for AI show 0.03% coding loss in terms of average BD-rate of Y, but the coding gain in terms of average BD-rate of Cb and Cr for AI are 0.34% and 0.32%. Moreover, the encoding time for CE3-3.4.2 can reduce 3% compared to VTM3.0.

Table 3. Results of the CE3-3.4.2 for all intra (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.17% | -0.27% | 97% | 100% |
| Class A2 | 0.06% | -0.26% | -0.22% | 95% | 100% |
| Class B | 0.02% | -0.36% | -0.42% | 99% | 100% |
| Class C | -0.01% | -0.28% | -0.34% | 98% | 100% |
| Class E | 0.11% | -0.66% | -0.28% | 97% | 100% |
| Overall | 0.03% | -0.34% | -0.32% | 97% | 100% |
| Class D | 0.01% | -0.25% | -0.11% | 96% | 99% |
| Class F (optional) | 0.02% | -0.05% | -0.20% | 96% | 100% |

Table 4. Results of the CE3-3.4.2 for random access (RA)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.45% | -0.53% | 99% | 100% |
| Class A2 | 0.14% | -0.95% | -0.42% | 99% | 100% |
| Class B | 0.10% | -0.96% | -1.47% | 100% | 100% |
| Class C | 0.12% | -0.96% | -0.89% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | -0.86% | -0.92% | 99% | 100% |
| Class D | 0.09% | -1.15% | -0.63% | 100% | 100% |
| Class F (optional) | 0.08% | -0.24% | -0.23% | 99% | 100% |

Conclusion

This contribution reports the experimental results of the simplified MDMS. CE3-3.4.1 reduces the numbers of pruning checks by using two DMs located at the bottom-right and the centre luma coding block. The simulation results of CE3-3.4.1 show 0.02% (Y), 0.96 (U) and 1.03 (V) gain without encoder run-time change. CE3-3.4.2 further simplifies MDMS by reducing 2 candidates. The simulation results (CE3-3.4.2) for AI show 0.03% (Y) coding loss, but the coding gain of Cb and Cr for AI are 0.34% and 0.32%. Moreover, the encoding time for CE3-3.4.2 can reduce 3% compared to VTM3.0.

References

- C.-H. Yao, P.-H. Lin, C.-C. Lin, B.-J. Fuh, C.-L. Lin, “CE3-related: Simplification of MDMS derivation”, JVET-L0630, October 2018.

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3: Intra Prediction and Mode Coding”, JVET-L1023, October 2018.

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, October 2018.

Patent rights declaration(s)

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).