| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0508-v1 |
| --- | --- |

| Title: | AHG9: Test Results of Dense Residual Convolutional Neural Network based In-Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yingbin Wang, Zhenzhong Chen, Yiming Li Wuhan University Liang Zhao, Shan Liu, Xiang Li Tencent | Tel: Email: | zzchen@whu.edu.cn shanl@tencent.com |
| Source: | Tencent, Wuhan University |  |  |

_____________________________

- Abstract

This contribution reports the test results for dense residual convolutional neural network based in-loop filter (DRNLF) [1] according to the methodology in [2]. The proposed DRNLF is implemented on VTM 3.0 [3]. Simulation results report -2.17%, -1.47%, -1.48% BD-rate savings for luma, and both chroma components compared with VTM 3.0 under AI configuration, and -2.15%, -3.04%, -1.96% for RA configuration, and -2.06%, -3.73%, -2.86% for LDB configuration.

Introduction

Dense residual convolutional neural network based in-loop filter (DRNLF) is firstly proposed in [4]. In last JVET meeting [1], multiple modifications have been carried out to simplify the network in terms of network structure, training materials, and training process. With the proposed modifications, the number of the network parameters have been reduced from 810K to 22K.

In current VTM software, multiple in-loop filters including deblocking filter (DF), sample adaptive offset (SAO) and adaptive loop filter (ALF) are applied. The proposed DRNLF is introduced into VTM as an additional filter placed after DF, and before SAO and ALF. The decision of whether to apply proposed filter or not is based on the rate-distortion optimization (RDO).

This document reports coding performance of the proposed DRNLF on VTM 3.0.

Simulation results

The proposed DRNLF is integrated into VTM 3.0 as an additional filter. The simulation is conducted according to the methodology in [2].

In the training phase, DIV2K dataset [5] is utilized to generate the training sets and validating set for the proposed network. There are 800 images in its training set and 100 images in its validation set. The images in DIV2K dataset are converted from RGB color space to YUV color space before compression. The images are compressed by VTM 3.0 with All Intra (AI) configuration using QP values in {22, 27, 32, 37}. L1 loss is used as the loss function for training. To balance the coding gain between different components, we set the weights of Y, Cb, Cr to 10, 1, 1, respectively. The loss is minimized by ADAM [16] optimizer. 300 epochs of training is applied. For each epoch, 8 small patches with size of 64×64 are randomly extracted from the images to generate the training samples. The learning rate is initialized as 5×10-4 and is divided by half every 50 epochs.

In evaluation, VTM 3.0 serves as the reference software. All the sequences recommended in the common test conditions (CTC) [6] are evaluated. The QP values are 22, 27, 32, and 37, respectively. The results of the experiments running using CPU only are provided in Table 1-3. And The results of the experiments running with GPU are provided in Table 4-5.

Table 1 Experimental Results of the Proposed Scheme (All Intra, CPU Only)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.54% | -1.65% | -1.81% | 118% | 4686% |
| Class A2 | -1.82% | -1.61% | -0.74% | 109% | 3374% |
| Class B | -1.33% | -1.57% | -2.12% | 107% | 3876% |
| Class C | -2.85% | -1.61% | -1.45% | 105% | 4989% |
| Class E | -3.65% | -0.84% | -0.86% | 110% | 7491% |
| Overall | -2.17% | -1.47% | -1.48% | 109% | 4615% |
| Class D | -3.57% | -1.74% | -1.94% | 104% | 4487% |

Table 2 Experimental Results of the Proposed Scheme (Random Access, CPU Only)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.63% | -3.41% | -1.98% | 110% | 9471% |
| Class A2 | -2.58% | -3.46% | -0.98% | 109% | 7208% |
| Class B | -1.67% | -3.14% | -2.59% | 109% | 8240% |
| Class C | -2.07% | -2.32% | -1.89% | 108% | 10466% |
| Class E |  |  |  |  |  |
| Overall | -2.15% | -3.04% | -1.96% | 109% | 8792% |
| Class D | -2.50% | -2.16% | -1.97% | 107% | 8169% |

Table 3 Experimental Results of the Proposed Scheme (Low delay B, CPU Only)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.53% | -4.86% | -3.45% | 109% | 10483% |
| Class C | -1.84% | -4.45% | -3.43% | 107% | 12003% |
| Class E | -3.21% | -0.87% | -1.12% | 124% | 11755% |
| Overall | -2.06% | -3.73% | -2.86% | 112% | 11286% |
| Class D | -2.50% | -3.23% | -3.07% | 107% | 9654% |

Table 4 Experimental Results of the Proposed Scheme (All Intra, with GPU)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.54% | -1.65% | -1.82% | 97% | 706% |
| Class A2 | -1.82% | -1.60% | -0.74% | 98% | 603% |
| Class B | -1.33% | -1.57% | -2.12% | 98% | 646% |
| Class C | -2.85% | -1.61% | -1.45% | 98% | 737% |
| Class E | -3.65% | -0.84% | -0.86% | 98% | 953% |
| Overall | -2.17% | -1.47% | -1.48% | 98% | 712% |
| Class D | -3.57% | -1.74% | -1.94% | 99% | 934% |

Table 5 Experimental Results of the Proposed Scheme (Random Access, with GPU)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.62% | -3.32% | -2.02% | 94% | 1091% |
| Class A2 | -2.58% | -3.44% | -0.99% | 95% | 944% |
| Class B | -1.66% | -3.14% | -2.56% | 96% | 1020% |
| Class C | -2.08% | -2.29% | -1.88% | 97% | 1197% |
| Class E |  |  |  |  |  |
| Overall | -2.15% | -3.01% | -1.96% | 95% | 1062% |
| Class D | -2.51% | -2.21% | -1.99% | 97% | 1029% |

Table 6 Experimental Results of the Proposed Scheme (Low delay B, with GPU)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.53% | -4.90% | -3.48% | 93% | 1305% |
| Class C | -1.80% | -4.17% | -3.38% | 95% | 1328% |
| Class E | -3.28% | -1.13% | -1.12% | 92% | 1275% |
| Overall | -2.06% | -3.71% | -2.86% | 93% | 1305% |
| Class D | -2.47% | -3.34% | -2.75% | 96% | 1158% |

Conclusion

In this contribution, the effect of DRNLF is investigated. Simulation results report -2.17%, -1.47%, -1.48% BD-rate savings for luma, and both chroma components compared with VTM 3.0 under AI configuration, and -2.15%, -3.04%, -1.96% for RA configuration, and -2.06%, -3.73%, -2.86% for LDB configuration.

References

[1] Yingbin Wang, Zhenzhong Chen, Yiming Li, Liang Zhao, Shan Liu, Xiang Li, “Dense Residual Convolutional Neural Network based In-Loop Filter,” JVET-L0242, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

[2] Yiming Li, Shan Liu, “Methodology and reporting template for neural network coding tool testing,” JVET-L1006, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

[3] VTM 3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-3.0.

[4]	Yingbin Wang, Zhenzhong Chen, Yiming Li, Liang Zhao, Shan Liu, Xiang Li, “Dense Residual Convolutional Neural Network based In-Loop Filter,” JVET-K0391, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 11th Meeting: Ljubljana, SI, 10–18 July 2018.

[5]	DIV2K, https://data.vision.ee.ethz.ch/cvl/DIV2K/.

[6]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L1010, Oct. 2018, Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

Wuhan University and Tencent America LLC do not have any current or pending patent rights relating to the technology described in this contribution.