| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 | Document: JVET-M0510-v1 |
| --- | --- |

| Title: | AHG9: CNN-based in-loop filter proposed by USTC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuanying Dai, Dong Liu, Yue Li, Feng Wu | Email: | daiyy@mail.ustc.edu.cn dongeliu@ustc.edu.n lytt@mail.ustc.edu.cn fengwu@ustc.edu.cn |
| Source: | University of Science and Technology of China |  |  |

_____________________________

- Abstract

This contribution presents the simulation results of an efficient network for loop filter. To reduce storage space and reduce complexity, we build two light weight deep convolutional neural networks by reduce network parameters. Simulation results report -0.76%, -0.56%, -0.69% BD-rate savings for Y, Cb, and Cr components compared with VTM3.0 under AI configuration, -%, -%, -% for RA configuration, and -0.76%, -0.56%, -0.69% for LDB configuration.

Introduction

Lossy compression of image and video yields visually annoying artifacts including blocking, blurring, ringing, etc., especially at low bit rates. In-loop filtering techniques can reduce these artifacts, improve quality, and achieve coding gain accordingly. In this proposal, two lightweight deep convolutional neural networks are proposed.

Training stage

The proposed CNN-based filter is in located between DF and SAO in VTM3.0. To generate training data, only DF is enabled.

- 2.1 Network structure

Residue learning technique is integrated into network. To reduce memory usage, parameters of several layers are shared, which means we can achieve the performance of deeper network with fewer parameters. As shown in Fig.1, layers of the same color represent the same parameters.

Fig 1. The structure of the first proposed network

We also investigate the depth-wise separable convolution as shown in Fig 2.

Fig 2. The structure of the second proposed network

- 2.2 Network setting

The network settings in training stage are shown in Table 1.

Table 1. Network setting in training stage

| Mandatory | Information in Training Stage |  |  |
| --- | --- | --- | --- |
|  | Basic Settings: |  |  |
|  |  | learning rate: | (1e-4) |
|  |  | optimizer: | (ADAM) |
|  |  | batch size: | (64) |
|  |  | epoch: | (200) |
|  |  | loss function: | (L2) |
|  |  | training GPU: | (GTX 1080ti) |
|  |  | training time: | (4.5h) |
|  |  | framework: | (tensorflow) |
| Optional | Other Settings: |  |  |
|  |  | (e.g. patch size:) | (64x64) |
|  |  | (e.g. additional dataset (besides DIV2k):) | (None) |
|  |  | (e.g. QP set for generating training dataset) | (22,27,32,37) |
|  |  | (etc) |  |

Inference stage

In inference stage, the filters in VTM3.0 including DF, SAO and ALF are all enabled. The additional CNN-based filter is located between DF and SAO. The tests are to be conducted under the common test conditions (CTC). The test QP values are 22, 27, 32, and 37.

Table 2 gives the information of inference. Only CPU result are given.

Table 2. Information in Inference Stage

|  | Network Details |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total Conv. Layers | Total FC Layers | Frame-work | Param. Num | GFLOPs | Mem.P(MB) | Mem.T(MB) |
| test1 | 10 | 0 | tensorflow | 7521 | W*H*2.15*1e-5 | 0.029 | W*H*6.1*1e-4 |
| test2 | 15 | 0 | tensorflow | 6608 | W*H*8.2*1e-6 | 0.025 | W*H*9.2*1e-4 |
|  | All Intra Main10 Over VTM-3.0 |  |  |  |  |  |  |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time(GPU) | Decoding Time(GPU) | Encoding Time(CPU) | Decoding Time(CPU) |
| test1 | -0.96 | -0.32 | -0.45 | - | - | 1.25 | 21.62 |
| test2 | -0.79 | -0.17 | -0.23 | - | - | 1.24 | 13.19 |
|  | Random Access Main10 Over VTM-3.0 |  |  |  |  |  |  |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time(GPU) | Decoding Time(GPU) | Encoding Time(CPU) | Decoding Time(CPU) |
| test1 |  |  |  | - | - |  |  |
| test2 |  |  |  | - | - |  |  |
|  | Low delay B Main10 Over VTM-3.0 |  |  |  |  |  |  |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time(GPU) | Decoding Time(GPU) | Encoding Time(CPU) | Decoding Time(CPU) |
| test1 | -0.76 | -0.56 | -0.69 | - | - | 1.17 | 4.40 |
| test2 | -0.64 | -0.50 | -0.31 | - | - | 1.00 | 3.29 |

Conclusions

This contribution presents two lightweight network structure. Both strike a good balance between coding performance and complexity.