| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0514-v1 |
| --- | --- |

| Title: | Removal of CIP from Multi-hypothesis Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen1, Wei-Jung Chien2, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +886-3-566-1980 1chunchic@qti.qualcomm.com, 2wchien@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This report proposes to remove the constrained intra prediction (CIP) from multi-hypothesis intra prediction (MHIntra). Specifically, the CIP flag does take effect on the derivation process of reference samples when MHIntra flag is on. It is reported that the proposed method reduces the Y/U/V BD-rate loss from VTM-3.0 with CIP=1 by 0.83%/0.44%/0.43% for Random Access and 0.93%/2.01%/2.09% for Low Delay B.

# Introduction

In the 12th JVET meeting, a multi-hypothesis intra prediction mode (MHIntra [1]) combining intra prediction and inter prediction was adopted into the Working Draft v3 [2] of VVC. As intra prediction is required in this new prediction mode, the same derivation process as for the L-shaped intra reference samples is used. Thus, the constrained intra prediction (CIP) flag also takes effect on the MHIntra mode. It is noted that CIP is a picture-level coding tool which confines the source of intra reference samples to causal neighboring intra blocks only in order to avoid error propagation from previously decoded pictures. However, as MHIntra is composed partially of inter-picture prediction, drifting errors (if any) would be carried over from previously decoder pictures to blocks coded by MHIntra in the current picture. No matter CIP is applied or not, MHIntra cannot always avoid propagating errors from previously decoded pictures. As such, this report proposes to remove CIP from MHIntra.

# Technical Description

In addition to CIP flag, the derivation process of the L-shaped reference samples needs to take the MHIntra flag of the current block into consideration, when checking whether immediately causal neighboring blocks are available or not. Specifically, this proposal allows that the source of the L-shaped reference samples can be inferred from causal neighboring inter blocks, when the either one of the following conditions is satisfied:

- CIP flag is off (same as VTM-3.0);

- CIP flag is on and the current coding unit is a MHIntra block.

# Experimental Results

The experiments of the proposed technique were conducted using the VTM-3.0 software and the common test conditions [3] to collect BD-rate numbers, encoding-time ratio and decoding-time ratio. VTM anchor result was generated using the same VTM-3.0 software and the common test conditions. None of the original macros in the software was changed. Parallel encoding and decoding were applied to Random Access cases for anchors and tests. Besides, it is note that a software bug in VTM-3.0 incurs mismatch between encoding and decoding YUV files when CIP=1. This bug was reported in Ticket 125 (https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/125) and the corresponding bugfix is applied to the experiments.

There are two testes conducted and reported and the respective performance results generated on top of the VTM anchor were summarized in Table 1.

- (Test a) VTM-3.0 w/ CIP=1 + Ticket 125 bugfix + Removal of CIP from MHIntra.

- (Test b) VTM-3.0 w/ CIP=1 + Ticket 125 bugfix.

- Performance results of (a) VTM-3.0 w/ CIP=1 and removal of CIP from MHIntra and (b) VTM-3.0 w/ CIP=1. The anchor is VTM-3.0 w/ CIP=0.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (a) CIP=1 & Removal of CIP form MHIntra |  |  |  |  | (b) CIP=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 5.38% | 17.23% | 20.43% | 99% | 101% | 6.33% | 16.99% | 19.66% | 99% | 98% |
| Class A2 | 2.66% | 7.00% | 5.19% | 100% | 101% | 3.41% | 7.96% | 6.46% | 99% | 100% |
| Class B | 3.26% | 7.41% | 8.60% | 99% | 99% | 4.22% | 8.12% | 9.23% | 99% | 98% |
| Class C | 3.76% | 7.20% | 8.02% | 98% | 99% | 4.42% | 7.45% | 8.48% | 100% | 98% |
| Overall | 3.70% | 9.24% | 10.13% | 99% | 100% | 4.53% | 9.68% | 10.56% | 99% | 99% |
| Class D | 2.83% | 6.16% | 5.96% | 99% | 100% | 3.40% | 6.17% | 6.25% | 96% | 99% |
| Class F | 2.76% | 4.78% | 4.48% | 98% | 100% | 3.16% | 5.11% | 4.94% | 100% | 100% |
|  | Low Delay B Main 10 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | 1.94% | 4.92% | 6.28% | 98% | 97% | 3.20% | 7.69% | 9.11% | 98% | 97% |
| Class C | 2.13% | 5.33% | 6.26% | 101% | 98% | 3.10% | 6.95% | 8.15% | 101% | 99% |
| Class E | 0.49% | 0.89% | 0.98% | 101% | 99% | 0.81% | 2.16% | 2.12% | 101% | 99% |
| Overall | 1.64% | 4.05% | 4.95% | 100% | 98% | 2.57% | 6.06% | 7.04% | 100% | 98% |
| Class D | 1.47% | 4.28% | 4.37% | 99% | 98% | 2.17% | 6.27% | 6.66% | 100% | 97% |
| Class F | 3.99% | 7.47% | 7.37% | 97% | 98% | 4.56% | 8.38% | 8.85% | 97% | 98% |

# Conclusion

In this report, a method for removing CIP from MHIntra mode was presented. The reported result shows reduction of BD-rate loss incurred by CIP=1. Accordingly, it is recommended for the committee to consider integrating the proposed method into the next release of VTM software and VVC working draft.

# References

- M.-S. Chiang, C.-W. Hsu and et. al., “CE10.1.1: Multi-hypothesis Prediction for Improving AMVP Mode, Skip or Merge Mode, and Intra Mode,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L0100, Oct. 2018.

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 3),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1001, Oct. 2018.

- F. Bossen, J. Boyce and et. al., “JVET Common Test Conditions and Software Reference Configurations for SDR Video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1010, Oct. 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).