| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0519-v2 |
| --- | --- |

| Title: | Non-CE: Context modeling for coding the prediction mode flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a modified context modeling scheme for entropy coding the syntax element pred_mode_flag. In the proposed method, additional context variables are employed to further exploit contextual information. The proposed method reportedly generates -0.11% and -0.07% luma BD-rates for the RA and LB settings, respectively, under the VTM3.0 CTCs, while increasing the number of context variables by 3.

In version 2 of this document, further simulation results are provided for coding screen content. The proposed method reportedly generates -0.30% and -0.12% luma BD-rates for the AI and RA settings, respectively, for Class TGM under the VTM3.0 CTCs with CPR being enabled. The proposed method further adding one single context variable for chroma CUs reportedly generates -0.40% and -0.07% luma BD-rates for the AI and RA settings, respectively, for Class TGM under the VTM3.0 CTCs with CPR being enabled.

# Introduction

In VVC working draft (WD) 3 as specified in [1], the syntax element pred_mode_flag is entropy coded using a single context variable. In this contribution, additional context variables are employed to further exploit contextual information.

# Proposed method

In the proposed method, context modeling for entropy coding the syntax element pred_mode_flag is conditioned on the coding unit (CU) prediction modes of the two neighbouring CUs and further conditioned on the size of the current CU using four context variables. The context index increment ctxInc is derived as follows:

ctxInc = ( ((predModeA == MODE_INTRA )? 1:0) + ((predModeL == MODE_INTRA)? 1:0)),

ctxInc += ( ctxInc == 0 && cuSizeC < 256 )? 3 : 0,

where the variables predModeA and predModeL represent the CU prediction modes of the above neighbouring CU and left neighbouring CU, respectively, and the variable cuSizeC represents the CU size in the number of luma samples. Four context variables are assigned for coding the syntax element pred_mode_flag.

For screen content coding with both CPR and dual-tree partitioning being enabled, one single context variable can be further added for coding the prediction modes in chroma CUs. In this way, five context variables are assigned for coding the syntax element pred_mode_flag.

# Experimental results

The proposed method was integrated into the VTM3.0 reference software [2]. The coding results generated by the proposed method were evaluated against the anchor results generated by the unmodified VTM3.0 reference software. Table 1 summarizes the experimental results conducted under the CTCs specified in [3]. The detailed simulation results can be found in the attached spreadsheets.

Further simulation results are provided for coding screen content. Table 2 summarizes the experimental results of the proposed method using four context variables conducted according to the description of Core Experiment 8 on screen content coding tools specified in [4]. Table 3 summarizes the experimental results of the proposed method further comprising one additional context variable dedicated for chroma CUs (using five context variables totally). Please note runtime results in Tables 2 & 3 are not accurate.

Table 1:  Summary of the average BD bitrate and run time results under the CTCs

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.07% | -0.01% | 100% | 101% |
| Class A2 | -0.10% | 0.08% | 0.06% | 100% | 99% |
| Class B | -0.10% | 0.07% | 0.07% | 100% | 100% |
| Class C | -0.13% | -0.07% | 0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | 0.00% | 0.04% | 100% | 100% |
| Class D | -0.05% | -0.03% | 0.09% | 100% | 100% |
| Class F | -0.10% | -0.03% | -0.01% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.12% | -0.10% | 100% | 99% |
| Class C | -0.12% | 0.14% | 0.25% | 101% | 101% |
| Class E | -0.04% | 0.05% | 0.31% | 100% | 101% |
| Overall | -0.07% | 0.01% | 0.12% | 100% | 100% |
| Class D | -0.07% | 0.42% | -0.13% | 101% | 100% |
| Class F | 0.10% | 0.17% | 0.04% | 101% | 99% |

Table 2:  Summary of the average BD bitrate and run time results for SCC using four context variables

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.03% | 140% | 118% |
| Class A2 | -0.02% | -0.07% | -0.01% | 142% | 121% |
| Class B | -0.01% | -0.03% | -0.03% | 141% | 122% |
| Class C | 0.00% | 0.12% | -0.04% | 142% | 127% |
| Class E | -0.03% | -0.05% | -0.10% | 141% | 124% |
| Overall | -0.01% | 0.00% | -0.03% | 141% | 122% |
| Class D | -0.03% | 0.04% | -0.12% | 142% | 128% |
| Class F | -0.14% | -0.17% | -0.17% | 139% | 123% |
| TGM | -0.30% | -0.33% | -0.35% | 141% | 126% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | 0.03% | -0.11% | 140% | 118% |
| Class A2 | -0.07% | -0.02% | -0.03% | 139% | 121% |
| Class B | -0.10% | -0.09% | 0.00% | 138% | 124% |
| Class C | -0.08% | -0.20% | 0.08% | 139% | 127% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.08% | 0.00% | 139% | 123% |
| Class D | -0.06% | -0.04% | 0.02% | 140% | 131% |
| Class F | -0.07% | -0.01% | -0.02% | 139% | 128% |
| TGM | -0.12% | -0.15% | -0.17% | 137% | 121% |

Table 3:  Summary of the average BD bitrate and run time results for SCC using five context variables

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | -0.03% | 142% | 120% |
| Class A2 | -0.01% | -0.01% | -0.01% | 139% | 119% |
| Class B | -0.01% | -0.01% | -0.06% | 138% | 118% |
| Class C | 0.02% | 0.01% | -0.03% | 138% | 123% |
| Class E | -0.03% | 0.05% | -0.06% | 136% | 118% |
| Overall | 0.00% | 0.01% | -0.04% | 138% | 119% |
| Class D | -0.01% | 0.14% | -0.10% | 144% | 139% |
| Class F | -0.14% | -0.25% | -0.17% | 139% | 124% |
| TGM | -0.40% | -0.40% | -0.39% | 133% | 118% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | 0.13% | 0.01% | 133% | 114% |
| Class A2 | -0.07% | -0.06% | -0.19% | 132% | 117% |
| Class B | -0.10% | 0.00% | 0.08% | 137% | 126% |
| Class C | -0.10% | 0.05% | 0.10% | 139% | 125% |
| Class E |  |  |  |  |  |
| Overall | -0.10% | 0.03% | 0.02% | 136% | 122% |
| Class D | -0.04% | 0.13% | 0.25% | 140% | 130% |
| Class F | -0.10% | 0.02% | 0.05% | 136% | 126% |
| TGM | -0.07% | -0.10% | -0.07% | 138% | 127% |

# Conclusions

The proposed method increases the number of context variables by three to further exploit contextual information. The proposed method reportedly generates -0.11% and -0.07% luma BD-rates for the RA and LB settings, respectively, under the VTM3.0 CTCs, and -0.30% and -0.12% luma BD-rates for the AI and RA settings, respectively, for Class TGM under the VTM3.0 CTCs with CPR being enabled. The proposed method further adding one single context variable for chroma CUs reportedly generates -0.40% and -0.07% luma BD-rates for the AI and RA settings, respectively, for Class TGM under the VTM3.0 CTCs with CPR being enabled.

# References

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 3),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1001, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- The software VTM3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0.

- F. Bossen, J. Boyce, X. Li, and V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1010, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- X. Xu, Y.-C. Chao, Y.-C. Sun, J. Xu, "Description of Core Experiment 8: Screen Content Coding Tools”, Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1028, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).