| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0522-v2 |
| --- | --- |

| Title: | CE6: MTS support for large rectangular blocks (Test 6.3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Vadim Seregin, Amir Said, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | hegilmez@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents the results of CE6.3.2, where a method to support MTS for large rectangular blocks is tested. The experimental results show incremental coding gains over VTM-3.0 with inter MTS enabled on average, yet for class A sequences up to 0.1% RA coding gains are achieved.

The proposed method is also tested for by restricting the maximum MTS size to 16 from 32. As compared to restricting the MTS size directly in VTM-3.0 (via parameter changes), restricting the MTS size using the proposed method reduces the coding losses from 0.55% to 0.33% for AI, from 0.45% to 0.28% for RA and from 0.35% to 0.26% for LDB.

# Introduction

In VTM-3.0, pairs of DCT-2, DST-7 or DCT-8 are used for separable transformation of residual block, where DCT-2 is used up to size 64x64 and DST-7 and DCT-8 are allowed up to size 32x32. When MTS (AMT) flag is 0, DCT-2 is used for both horizontal and vertical transformation. If the MTS flag is 1, one of the four separable transform options derived from DST-7 and DCT-8 is signalled and used to transform a block.

VTM-3.0 has the following issue that it does not allow using MTS transforms for blocks of size Nx64 or 64xN for N < 64, although DST7/DCT8 can be applied on the shorter side of the block (with size N). Note that, blocks with size Nx64 or 64xN can still use different types of N-point MTS transforms for N < 64, which are available in VTM-3.0.

In VTM-3.0, another issue is the complexity of 32-point DST7/DCT8 transforms, which can be addressed in the current software by restricting the maximum MTS size to 16, but this will also restrict large rectangular blocks to only use DCT2. For MxN/NxM blocks, where N ≤ 16 ≤ M, DST7/DCT8 can be used for the shorter side (with size N). The proposed method enables MTS for such rectangular blocks.

# Proposed method: Enabling MTS for large blocks and signaling modification

To incorporate DST-7 or DCT-8 for large rectangular blocks with sizes Nx64 and 64xN for N<64, the 64-point DCT-2 is used to transform the longer side of the block, and N-point DCT-8 or N-point DST-7 is used for the shorter side. One bit is signalled to determine whether DST-7 or DCT-8 is used for the shorter side of the block, and no signaling is required for the side using DCT-2. Since the current VTM has both DST-7 and DCT-8 available for N<64, no additional memory or transform kernels are required to implement this method. This method is tested in CE6.3.2a.

The proposed method is also tested (in CE6.3.2b) when the maximum MTS size is reduced to 16 and compared against restricting maximum MTS size to 16 directly in VTM-3.0 (restricting without the proposed method, that is tested in CE6.3.2c).

# Experimental Results

## Results of the proposed method, tested in CE 6.3.2a

The proposed method is implemented based on VTM-3.0 software and tested (in CE 6.3.2a) under CTC and also by enabling the inter MTS (Inter AMT) via EMT=3 and EMTFast =3. The results are presented in Tables 1 and 2.

Note that, under CTC conditions, proposed method has no effect on coding performance of intra coded blocks since in VTM-3.0 dimensions Nx64 and 64xN are not allowed in I-frames.

Table 1: Results of the proposed method (CE6.3.2a) over VTM-3.0.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.34% | -0.37% | 100% | 99% |
| Class A2 | -0.03% | -0.07% | -0.01% | 100% | 98% |
| Class B | -0.03% | -0.04% | -0.20% | 100% | 99% |
| Class C | -0.02% | 0.03% | -0.05% | 98% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.09% | -0.16% | 99% | 98% |
| Class D | 0.01% | -0.02% | 0.05% | 99% | 97% |
| Class F | -0.01% | -0.04% | 0.00% | 98% | 96% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.35% | 0.21% | 100% | 100% |
| Class C | 0.03% | 0.10% | 0.14% | 98% | 97% |
| Class E | 0.01% | 0.40% | -0.13% | 97% | 93% |
| Overall | 0.02% | -0.02% | 0.10% | 99% | 97% |
| Class D | 0.00% | 0.18% | -0.07% | 98% | 95% |
| Class F | 0.06% | 0.10% | 0.06% | 97% | 94% |

Table 2: Results of the proposed method (CE6.3.2a) over VTM-3.0 with Inter MTS enabled.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.27% | -0.17% | 104% | 102% |
| Class A2 | -0.04% | -0.02% | 0.03% | 101% | 101% |
| Class B | -0.04% | -0.11% | -0.10% | 101% | 99% |
| Class C | -0.01% | 0.13% | 0.10% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.06% | -0.03% | 102% | 100% |
| Class D | -0.02% | -0.06% | 0.05% | 100% | 98% |
| Class F | -0.01% | -0.01% | 0.01% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | 0.13% | -0.27% | 102% | 100% |
| Class C | 0.01% | 0.05% | 0.15% | 101% | 101% |
| Class E | 0.01% | 0.11% | -0.54% | 100% | 95% |
| Overall | -0.02% | 0.10% | -0.20% | 101% | 99% |
| Class D | 0.00% | 0.21% | -0.39% | 100% | 102% |
| Class F | -0.12% | -0.01% | -0.40% | 102% | 99% |

## Results of CEs 6.3.2b and 6.3.2c, restricting MTS size to be 16

Tables 3 and 4 show the results of 6.3.2b and 6.3.2c, respectively. In addition, the CE results obtained by enabling Inter MTS are presented in Tables 5 and 6.

Based on the presented results below, restricting the maximum MTS size to 16 using the proposed method (CE6.3.2b) provides considerable coding gains over directly restricting the maximum MTS size to 16 via parameter changes available in VTM-3.0 (CE6.3.2c).

Table 3: Results of CE6.3.2b over VTM-3.0 under CTC.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.66% | 0.99% | 1.05% | 88% | 88% |
| Class A2 | 0.35% | 0.40% | 0.36% | 92% | 93% |
| Class B | 0.33% | 0.40% | 0.42% | 91% | 92% |
| Class C | 0.09% | 0.09% | -0.10% | 94% | 96% |
| Class E | 0.29% | 0.24% | 0.33% | 92% | 94% |
| Overall | 0.33% | 0.40% | 0.38% | 92% | 93% |
| Class D | 0.06% | -0.04% | 0.06% | 95% | 96% |
| Class F | 0.07% | 0.11% | 0.17% | 94% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.34% | 0.57% | 0.82% | 97% | 100% |
| Class A2 | 0.13% | 0.27% | 0.28% | 98% | 99% |
| Class B | 0.20% | 0.61% | 0.50% | 96% | 97% |
| Class C | 0.08% | 0.07% | 0.37% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.18% | 0.39% | 0.48% | 97% | 99% |
| Class D | 0.02% | -0.36% | -0.24% | 100% | 101% |
| Class F | 0.05% | 0.12% | 0.27% | 98% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.12% | -0.15% | -0.01% | 98% | 101% |
| Class C | 0.06% | 0.30% | 0.33% | 99% | 100% |
| Class E | 0.00% | 0.94% | 0.31% | 99% | 97% |
| Overall | 0.07% | 0.27% | 0.19% | 99% | 99% |
| Class D | 0.07% | 0.28% | 0.55% | 99% | 100% |
| Class F | 0.14% | 0.50% | 0.05% | 98% | 97% |

Table 4: Results of CE6.3.2c over VTM-3.0 under CTC.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.05% | 1.42% | 1.50% | 81% | 85% |
| Class A2 | 0.63% | 0.69% | 0.59% | 85% | 95% |
| Class B | 0.55% | 0.56% | 0.61% | 86% | 92% |
| Class C | 0.17% | 0.13% | 0.12% | 90% | 96% |
| Class E | 0.48% | 0.49% | 0.54% | 87% | 94% |
| Overall | 0.55% | 0.62% | 0.63% | 86% | 92% |
| Class D | 0.14% | 0.14% | 0.05% | 91% | 97% |
| Class F | 0.10% | 0.11% | 0.17% | 90% | 95% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.61% | 0.89% | 1.25% | 94% | 98% |
| Class A2 | 0.26% | 0.41% | 0.42% | 96% | 100% |
| Class B | 0.36% | 0.90% | 0.77% | 94% | 97% |
| Class C | 0.09% | 0.22% | 0.32% | 97% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.32% | 0.62% | 0.68% | 95% | 99% |
| Class D | 0.05% | -0.28% | -0.03% | 98% | 101% |
| Class F | 0.08% | 0.36% | 0.18% | 96% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.15% | 0.08% | 0.44% | 97% | 100% |
| Class C | 0.11% | 0.34% | 0.44% | 98% | 101% |
| Class E | 0.08% | 1.39% | -0.69% | 98% | 97% |
| Overall | 0.12% | 0.49% | 0.16% | 98% | 99% |
| Class D | 0.03% | 0.37% | 0.40% | 98% | 100% |
| Class F | 0.05% | 0.14% | 0.15% | 97% | 97% |

Table 5: Results of CE6.3.2b over VTM-3.0.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.66% | 0.99% | 1.05% | 88% | 88% |
| Class A2 | 0.35% | 0.40% | 0.36% | 92% | 93% |
| Class B | 0.33% | 0.40% | 0.42% | 91% | 92% |
| Class C | 0.09% | 0.09% | -0.10% | 94% | 96% |
| Class E | 0.29% | 0.24% | 0.33% | 92% | 94% |
| Overall | 0.33% | 0.40% | 0.38% | 92% | 93% |
| Class D | 0.06% | -0.04% | 0.06% | 95% | 96% |
| Class F | 0.07% | 0.11% | 0.17% | 94% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.47% | 0.44% | 0.69% | 91% | 97% |
| Class A2 | 0.28% | 0.15% | 0.16% | 91% | 100% |
| Class B | 0.32% | 0.39% | 0.38% | 90% | 96% |
| Class C | 0.09% | 0.20% | 0.30% | 92% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.28% | 0.30% | 0.38% | 91% | 97% |
| Class D | 0.10% | -0.35% | -0.01% | 94% | 100% |
| Class F | 0.06% | 0.09% | 0.26% | 93% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.37% | 0.27% | 0.02% | 88% | 94% |
| Class C | 0.15% | 0.02% | 0.26% | 91% | 98% |
| Class E | 0.23% | 0.31% | -0.42% | 91% | 93% |
| Overall | 0.26% | 0.20% | -0.01% | 90% | 95% |
| Class D | 0.12% | -0.22% | -0.39% | 93% | 97% |
| Class F | 0.04% | -0.04% | -0.19% | 91% | 99% |

Table 6: Results of CE6.3.2c over VTM-3.0 with Inter MTS enabled.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.05% | 1.42% | 1.50% | 81% | 85% |
| Class A2 | 0.63% | 0.69% | 0.59% | 85% | 95% |
| Class B | 0.55% | 0.56% | 0.61% | 86% | 92% |
| Class C | 0.17% | 0.13% | 0.12% | 90% | 96% |
| Class E | 0.48% | 0.49% | 0.54% | 87% | 94% |
| Overall | 0.55% | 0.62% | 0.63% | 86% | 92% |
| Class D | 0.14% | 0.14% | 0.05% | 91% | 97% |
| Class F | 0.10% | 0.11% | 0.17% | 90% | 95% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.72% | 0.73% | 0.98% | 85% | 97% |
| Class A2 | 0.47% | 0.24% | 0.16% | 86% | 99% |
| Class B | 0.53% | 0.58% | 0.61% | 84% | 95% |
| Class C | 0.15% | 0.14% | 0.02% | 86% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.45% | 0.43% | 0.43% | 85% | 97% |
| Class D | 0.16% | -0.17% | 0.08% | 89% | 100% |
| Class F | 0.11% | 0.24% | 0.14% | 88% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.55% | 0.17% | 0.34% | 82% | 93% |
| Class C | 0.20% | -0.24% | 0.12% | 85% | 99% |
| Class E | 0.22% | 0.64% | -1.12% | 87% | 95% |
| Overall | 0.35% | 0.15% | -0.10% | 84% | 95% |
| Class D | 0.20% | -0.03% | -0.36% | 89% | 98% |
| Class F | 0.10% | -0.31% | 0.63% | 86% | 96% |

# Complexity metrics

## Operation and memory counts: The proposed method tested in CE6.3.2a introduces no additional computational complexity and memory requirements over VTM-3.0.

## The bit-precision required by the transforms: The same precision in the current VTM-3.0 is used.

## Specify if a transform requires multiple iterations: No additional iteration is required from VTM-3.0.

## Encoder and decoder times: These measurements are presented in Section 3.

# The proposed text for specification document

The proposed syntax changes for JVET-L1001 version 9 are as follows, where the changes are colored in red and the removed syntax elements are crossed out as red.

#### 7.3.4.11 Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) |  |
| tu_cbf_luma[ x0 ][ y0 ] | ae(v) |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( ( tu_cbf_luma[ x0 ][ y0 ] ¦ ¦ tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) && treeType != DUAL_TREE_CHROMA ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta_abs | ae(v) |
| if( cu_qp_delta_abs ) |  |
| cu_qp_delta_sign_flag | ae(v) |
| } |  |
| } |  |
| if( ( ( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) && sps_mts_intra_enabled_flag ) ¦ ¦ ( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER ) && sps_mts_inter_enabled_flag ) ) && tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ¦¦ ( tbHeight <= 32 ) ) |  |
| tu_mts_flag[ x0 ][ y0 ] | ae(v) |
| if( tu_cbf_luma[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth / 2 ), Log2( tbHeight / 2 ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth / 2 ), Log2( tbHeight / 2 ), 2 ) |  |
| } |  |

…

#### 8.5.4 Transformation process for scaled transform coefficients

#### 8.5.4.1  General

…

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived in Tables 8-16, 8-17 and 8-18 depending on mts_idx[ xTbY ][ yTbY ][ cIdx ].

The variables nonZeroW and nonZeroH are derived as follows:

nonZeroW = Min( nTbW, 32 )		(8-837)

nonZeroH = Min( nTbH, 32 )		(8-838)

…

Table 8-16 – Specification of trTypeHor and trTypeVer depending on mts_idx[ x ][ y ][ cIdx ] for log2TbWidth <= 5 && log2TbHeight <= 5

| mts_idx[ xTbY ][ yTbY ][ cIdx ] | trTypeHor | trTypeVer |
| --- | --- | --- |
| −1 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 2 | 1 |
| 2 | 1 | 2 |
| 3 | 2 | 2 |

Table 8-17 – Specification of trTypeHor and trTypeVer depending on mts_idx[ x ][ y ][ cIdx ] for log2TbWidth <= 5 && log2TbHeight == 6

| mts_idx[ xTbY ][ yTbY ][ cIdx ] | trTypeHor | trTypeVer |
| --- | --- | --- |
| −1 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 2 | 0 |

Table 8-18 – Specification of trTypeHor and trTypeVer depending on mts_idx[ x ][ y ][ cIdx ] for log2TbWidth == 6 && log2TbHeight <= 5

| mts_idx[ xTbY ][ yTbY ][ cIdx ] | trTypeHor | trTypeVer |
| --- | --- | --- |
| −1 | 0 | 0 |
| 0 | 0 | 1 |
| 1 | 0 | 2 |

# References

- A. Said and X. Zhao, “Description of Core Experiment 6: Transforms and Transform signaling,” JVET-L1026, 12th meeting, Macao, CN, Oct. 2018.

- F. Bossen, J.Boyce, X. Li, V. Seregin and K. Suehring,  “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th meeting, Macao, CN, Oct. 2018.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).