| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Marrakesh, MA, 9–18 January 2019 | Document: JVET-M0529-v1 |
| --- | --- |

| Title: | AHG14: Normative Recovery Point Indication |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Rickard Sjöberg Mitra Damghanian | Email: | martin.m.pettersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes to add a recovery point indication NAL unit to the VVC specification, using a new non-VLC NAL unit type. The position of the recovery point indication NAL unit in the bitstream specifies the start of the recovery period. The end of the recovery period, the recovery point picture, is specified by a recovery_poc_cnt UVLC code word, which is the only payload element in the proposed recovery point indication NAL unit RBSP. It is further proposed that starting the decoding at a recovery point is supported through the following process:

- decode a recovery point indication NAL unit that defines a recovery point begin (RPB) access unit and the start and end picture of the recovery point

- start decoding at the start picture

- generate unavailable pictures from the RPS of the start picture

- suppress output of the start picture and all pictures that follow in decoding order until the end picture is decoded

In comparison to the recovery point SEI message in HEVC, the proposed solution is normatively specified and does not contain the exact match flag nor the broken link flag. It is claimed that having the indication of the recovery point normatively specified in VVC has the following advantages:

- The encoder will know that a decoder shall support the recovery point indication NAL unit. Thus, no additional IRAP pictures need to be encoded to ensure random access when the decoder implementation is unknown.

- By a normative recovery point signaling in the VVC specification, there is no need for external specifications to mandate the SEI message.

- Indicating the presence of recovery points non-normatively in an SEI message may cause issues if support of recovery point coding tools would be specified normatively in VVC, for example if syntax or processes are different for any picture in the recovery period than for other pictures.

In order to make recovery points normative in VVC, the contribution proposes that a CVS may start with a recovery point signaled by the new NAL unit type. Alternatives for normatively specifying the recovery point are discussed in the contribution.

# Introduction

Both AVC and HEVC specify a recovery point SEI message which assists a decoder in determining when the decoding process will produce acceptable pictures for display after the decoder initiates random access at the recovery point. The SEI message is optional for the decoder to support although it is made mandatory in some external specifications, such as DVB [1].

Table 1: Recovery point SEI message syntax in HEVC

| recovery_point( payloadSize ) { | Descriptor |
| --- | --- |
| recovery_poc_cnt | se(v) |
| exact_match_flag | u(1) |
| broken_link_flag | u(1) |
| } |  |

This contribution addresses the following mandate from AHG14 on progressive intra refresh:

“Study normative solutions to improve intra refresh performance against encoder-only intra refresh”

Regardless if coding tools are normatively specified in VVC to support recovery points or not, it is assessed that a normative solution of indicating the start and end of a recovery point would be beneficial.

# Problem statement

The following issues have been identified with indicating recovery points non-normatively using a SEI message:

- The encoder may not be aware if an unknown decoder supports the recovery point SEI message or not and may thus need to send periodic IRAP pictures to ensure that the decoder will be able to tune into a bitstream. This is sub-optimal in terms of bandwidth and creates a bottleneck in low-delay applications.

- The recovery point SEI message may not be supported in a consistent way for external specifications, which may cause interoperability issues.

For instance, in DVB [1] it is mandated for the decoder that “HEVC IRDs (Integrated Receiver-Decoders) shall support the use of SEI of the following message types: … - Recovery Point SEI message” and for the encoder: “When present, the recovery_poc_cnt shall be set to 0, exact_match_flag shall be set to 1, and broken_link_flag shall be set to 0”

In ISO/IEC 14496-15 [2] it is stated that “the video coding system can include the concept of a ‘gradual decoding refresh’ or random access recovery point recovery. This may be signaled in the bit-stream using a mechanism such as the recovery point SEI message.“. No restrictions are set for the exact_match_flag or the broken link flag in ISO/IEC 14996-15.

Thus, the usage of the recovery point SEI message for HEVC is normatively mandated for decoders in DVB with restrictions for the flags for the encoder. For ISO/IEC 14496-15 the support for the recovery point SEI message is optional without restrictions for the flags.

- It is assessed that it would be useful to allow a CVS to start with a recovery point, e.g. after splitting low-delay bitstreams encoded with recovery points. However, a bitstream starting with a non-IRAP picture and a recovery point SEI message is not a conforming bitstream in neither of AVC, HEVC, nor the VVC draft, since a CVS may only start with an IRAP picture.

- Indicating the presence of recovery points non-normatively in a SEI message may cause issues if support of recovery point coding tools would be specified normatively in VVC, for example if syntax or processes are different for any picture in the recovery period than for other pictures.

# Discussion

In this section we discuss the benefits of specifying the indication of the recovery point normatively in VVC, a couple of design considerations and some options for signaling the normative recovery point indication.

## Benefits of a normative recovery point indication

Specifying a recovery point normatively in VVC has the following advantages:

- Decoder support of recovery points. The encoder will know that a decoder shall support the recovery point indication NAL unit. Thus, no additional IRAP pictures need to be encoded to ensure random access when the decoder implementation is unknown.

- Alignment between external specifications. By a normative signaling of the recovery point indication in the VVC specification, there is no need for external specifications to mandate the SEI message. Thus, the usage of recovery points would be aligned between external specifications.

- Splitting of bitstreams. A bitstream encoded with normative recovery points may be divided into legal CVSs at the recovery points. A CVS may thus start with a recovery point access unit.

- Specific coding level tools may be used for recovery points. AHG14 is investigating potential benefits of specifying normative coding level tools for progressive intra refresh in VVC. Indicating the presence of recovery points normatively enables normative progressive intra coding tools to be used for random access in a straight-forward way.

Thus, we conclude that a normative solution for specifying recovery points including indicating the start and end of a recovery point would be beneficial.

## Options for signaling the normative recovery point indication

Here we discuss some options for normatively specifying the recovery point indication:

- In a picture header. It has been discussed that VVC may include a picture header to efficiently code header data that are identical between slices (or tile groups). The indication of the recovery point may be specified here. However, the recovery point indication would then not be well exposed to the systems layer.

- As a new NAL unit with a non-VCL NAL unit type and with the recovery point specified in the recovery point indication NAL unit. Specifying the recovery point indication as a NAL unit type makes it easy to access at the systems layer. A decoder seeking to perform a random access in a bitstream only needs to parse the NAL unit headers in the bitstream to find where to tune in. Also, in comparison to the PPS approach, signaling overhead is only made when the recovery point indication is signaled (PPS would require a flag for turning it off).

- In a VCL NAL unit with a new NAL unit type. An alternative is to specify the start of a recovery point as a new VCL NAL unit type. The end of the recovery point may be specified in another new VCL NAL unit type. This approach also has the benefit that it is easy to access the recovery point information from the systems layer. One potential problem with this approach is that the recovery point indication becomes tied to the picture type. To allow for recovery points in pictures with different picture types, NAL unit types for all combinations are needed. Another issue is if the NAL unit with the NAL unit type specifying the end of the recovery point is lost. Moreover, overlapping recovery points may be problematic.

It is assessed that specifying the recovery point indication as a new NAL unit with a non-VCL NAL unit type and with the recovery point specified in the recovery point indication NAL unit, is the best choice.

## Design considerations for normative recovery points

### Recovery point begin (RPB) access unit

To indicate as early as possible that an access unit is the start of a recovery point it is desired to put the recovery point indication early in the access unit, before any VCL NAL units. The access unit containing the recovery point indication is referred to as the recovery point begin (RPB) access unit and the picture in the access unit is referred to as the RPB picture.

The associated SPS and PPS shall be activated prior to the decoding of the pictures associated with the recovery point.

### Align access of recovery points and IRAP picture

In order to normatively specify that an access unit containing a recovery point indication may be used for random access or to start a CVS it is desired to align the access of recovery points with the access of IRAP pictures.

We therefore propose to define that a CVS may start with either an IRAP picture or an RPB picture.

### Generation of unavailable reference pictures

Compared to the recovery point SEI message in HEVC, a normative recovery point solution should preferably have a more precise handling of unavailable reference pictures. One way of solving this is to mandate generation of all reference pictures in the RPS (or similar picture management system) of the RPB picture.

### Suppression of picture output

When the decoding is started at a RPB picture, either after a random access operation has been initialized at the RBP access unit or if the RBP access unit is the first access unit in a CVS, the RPB and the pictures following the RPB picture in decoding order until but not including the recovery point picture, may not have correct output.

We therefore propose that the RPB picture and all pictures that follow the RPB picture in decoding order until but not including the recovery point picture, shall not be output, if the decoding is started at the RPB picture.

# Proposal

It is proposed to add a recovery point indication NAL unit to the VVC specification, using a new non-VLC NAL unit type. The position of the recovery point indication NAL unit in the bitstream specifies the start of the recovery period. The end of the recovery period, the recovery point picture, is specified by a recovery_poc_cnt UVLC code word, which is the only payload element in the proposed recovery point indication NAL unit RBSP. It is further proposed that starting the decoding at a recovery point is supported through the following process:

- decode a recovery point indication NAL unit that defines a recovery point begin (RPB) access unit and the start and end picture of the recovery point

- start decoding at the start picture

- generate unavailable pictures from the RPS of the start picture

- suppress output of the start picture and all pictures that follow in decoding order until the end picture is decoded

It is assessed that the exact_match_flag specified in the recovery point SEI message in HEVC, is not useful when specifying the recovery point in a normative way since an exact match is required. It is also assessed that the broken_link flag may be useful, but it is not considered in this proposal since BLA pictures are not defined for VVC.

## 4.1 Proposed changes to the VVC draft text

The following syntax and semantics are proposed to be included the next VVC draft. Changes are highlighted in red on top of the current VVC draft JVET-L1001v5.

# Definitions

…

3.18	coded video sequence (CVS): A sequence of access units that consists, in decoding order, of an IRAP access unit or an RPB access unit, followed by zero or more access units that are not IRAP access units, including all subsequent access units up to but not including any subsequent access unit that is an IRAP access unit.

…

3.74	recovery point: A point in the bitstream where the next bit in the bitstream is the first bit of a RPB access unit.

3.75	recovery point begin (RPB) access unit: An access unit that contains a recovery point indication NAL unit.

3.76	recovery point begin (RPB) picture: The coded picture in an RPB access unit.

3.77	recovery point period: The set of pictures including an RPB picture and all pictures that follow the RPB picture until and including the recovery point picture indicated by the recovery point indication NAL unit in the access unit containing the RPB picture.

3.78	recovery point picture: The last coded picture in decoding order in a recovery point period.

…

#### 7.3.2.5	Recovery point indication RBSP syntax

| recovery_point_indication_rbsp( ) { | Descriptor |
| --- | --- |
| recovery_poc_cnt | se(v) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.4.2.2	NAL unit header semantics

…

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | NON_IRAP_NUT | Coded slice segment of a non-IRAP picture slice_layer_rbsp( ) | VCL |
| 1 | IRAP_NUT | Coded slice of an IRAP picture slice_layer_rbsp( ) | VCL |
| 2-15 | RSV_VCL_NUT | Reserved VCL NAL Units | VCL |
| 16 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 17 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 18 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 19 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 20 | RPI_NUT | Recovery point indication recovery_point_indication_rbsp( ) | non-VCL |
| 20, 21, 22 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 2223-26 | RSV_NVCL | Reserved | non-VCL |
| 27-31 | UNSPEC | Unspecified | non-VCL |

…

When nal_unit_type is equal to RPI_NUT, TemporalId shall be equal to 0.

…

7.4.3.5	Recovery Point Indication RBSP semantics

The RPI NAL unit shall precede any VCL NAL units in the access unit containing the RPI NAL unit. All VCL NAL units in an access unit containing the RPI NAL unit shall have TemporalId equal to 0.

If an RPB access unit containing an RPI NAL unit is not the first access unit in the CVS and a random access operation is not initiliazed at the RPB access unit, the RPI NAL unit in the RPB access unit shall be ignored.

Otherwise, if an RPB access unit containing an RPI NAL unit is the first access unit in the CVS or a random access operation is initialized at the RPB access unit, the following applies:

- The decoder shall generate all reference pictures included in the RPS [Ed note: update after picture management system in VVC has been defined] of the recovery point begin (RPB) picture of the RPB access unit according to the process described in [see example specification text 8.2.1.1 below for this].

- The poc_msb_cycle_val for the RPB picture shall be set to 0 when deriving the PicOrderCntVal for the RPB picture

- The RPB picture and all pictures that follow the RPB picture in decoding order shall be decoded.

- The RPB picture and all pictures that follow the RPB picture in decoding order until but not including the recovery point picture, shall not be output.

- Any SPS or PPS RBSP that is referred to by the picture in a RPB access unit or by any picture following that picture in decoding order shall be available to the decoding process prior to its activation.

It is a requirement of bitstream conformance that the decoded pictures that follow the recovery point picture in decoding order shall be an exact match to the pictures that would be produced by starting the decoding process at the location of an IRAP or RPB access unit that precedes the RPB picture that belong to the same recovery point period as the recovery point picture in decoding order, if any, in the bitstream.

recovery_poc_cnt specifies picture order count of the recovery point picture. The picture that follows the current picture in decoding order that has PicOrderCntVal equal to the PicOrderCntVal of the current picture plus the value of recovery_poc_cnt is referred to as the recovery point picture. The recovery point picture shall not precede the current picture in decoding order. The value of recovery_poc_cnt shall be in the range of −MaxPicOrderCntLsb / 2 to MaxPicOrderCntLsb / 2 − 1, inclusive.

## 8.2	Tile group decoding process

### 8.2.1	Decoding process for generating unavailable reference pictures

#### 8.2.1.1	General decoding process for generating unavailable reference pictures

[Ed note: The text below is to be updated after VVC reference picture management has been defined]

This process is invoked for any RPB picture in the bitstream if the corresponding RPB access unit is the first access unit in the CVS or a random access operation is initialized at the RPB access unit.

The following applies:

- For each RefPicSetStCurrBefore[ i ], with i in the range of 0 to NumPocStCurrBefore − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in clause 8.2.1.2 and the following applies:

–	The value of PicOrderCntVal for the generated picture is set equal to PocStCurrBefore[ i ].

–	The value of PicOutputFlag for the generated picture is set equal to 0.

–	The generated picture is marked as "used for short-term reference".

–	RefPicSetStCurrBefore[ i ] is set to be the generated reference picture.

–	The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id.

- For each RefPicSetStCurrAfter[ i ], with i in the range of 0 to NumPocStCurrAfter − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in clause 8.2.1.2 and the following applies:

–	The value of PicOrderCntVal for the generated picture is set equal to PocStCurrAfter[ i ].

–	The value of PicOutputFlag for the generated picture is set equal to 0.

–	The generated picture is marked as "used for short-term reference".

–	RefPicSetStCurrAfter[ i ] is set to be the generated reference picture.

–	The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id.

- For each RefPicSetStFoll[ i ], with i in the range of 0 to NumPocStFoll − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in clause 8.2.1.2, and the following applies:

- The value of PicOrderCntVal for the generated picture is set equal to PocStFoll[ i ].

- The value of PicOutputFlag for the generated picture is set equal to 0.

- The generated picture is marked as "used for short-term reference".

- RefPicSetStFoll[ i ] is set to be the generated reference picture.

- The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id of the current picture.

- For each RefPicSetLtCurr[ i ], with i in the range of 0 to NumPocLtCurr − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in clause 8.2.1.2 and the following applies:

–	The value of PicOrderCntVal for the generated picture is set equal to PocLtCurr[ i ].

–	The value of slice_pic_order_cnt_lsb for the generated picture is inferred to be equal to ( PocLtCurr[ i ] & ( MaxPicOrderCntLsb − 1 ) ).

–	The value of PicOutputFlag for the generated picture is set equal to 0.

–	The generated picture is marked as "used for long-term reference".

–	RefPicSetLtCurr[ i ] is set to be the generated reference picture.

–	The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id.

- For each RefPicSetLtFoll[ i ], with i in the range of 0 to NumPocLtFoll − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in clause 8.2.1.2, and the following applies:

- The value of PicOrderCntVal for the generated picture is set equal to PocLtFoll[ i ].

- The value of slice_pic_order_cnt_lsb for the generated picture is inferred to be equal to ( PocLtFoll[ i ] & ( MaxPicOrderCntLsb − 1 ) ).

- The value of PicOutputFlag for the generated picture is set equal to 0.

- The generated picture is marked as "used for long-term reference".

- RefPicSetLtFoll[ i ] is set to be the generated reference picture.

- The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id of the current picture.

#### 8.2.1.2	Generation of one unavailable picture

When this process is invoked, an unavailable picture is generated as follows:

–	The value of each element in the sample array SL for the picture is set equal to 1  <<  ( BitDepthY − 1 ).

–	When ChromaArrayType is not equal to 0, the value of each element in the sample arrays SCb and SCr for the picture is set equal to 1  <<  ( BitDepthC − 1 ).

–	The prediction mode CuPredMode[ x ][ y ] is set equal to MODE_INTRA for x = 0..pic_width_in_luma_samples − 1, y = 0..pic_height_in_luma_samples − 1.

# References

- ETSI TS 101 154, v2.4.1, Digital Video Broadcasting (DVB); Specification for the use of Video and Audio Coding in Broadcast and Broadband Applications, February 2018

- ISO/IEC FDIS 14496-15:2014 Carriage of network abstract layer (NAL) unit structured video in the ISO base media file format (w16169)

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).