| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0531 |
| --- | --- |

| Title: | Crosscheck of JVET-M0273 (CE2-related: Early awareness of accessing temporal blocks in sub-block merge list construction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Ruoyang Yu | Email: | ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

JVET-M0273 proposes three modifications to the sub-block merge candidate list generation in VTM-3.0: 1) only 1 spatial neighboring block, instead of 4, is checked to derive initial MV; 2) ATMVP candidate is always available; 3) remove TMVP in affine candidate construction process. This cross-check contribution reports that the description in M0273 matches the software and the overall objective results are matched.

Very minor differences (0.0001 PSNR) in some cases are observed, but it is believed to be caused by the known bug [1] in VTM-3.0 which leads to different results on different platforms.

# Results

The following three tests were run under VTM-3.0 RA and LDB common test configurations:

- Test #1: ATMVP changes (first two modifications)

- Test #2: The third modification

- Test #3: All three modifications

Note that encoding and decoding are inaccurate since they are run on machines with different capabilities.

## Test #1

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | -0.01% | 83% | 93% |
| Class A2 | -0.05% | 0.03% | 0.07% | 79% | 94% |
| Class B | 0.00% | 0.04% | -0.04% | 78% | 89% |
| Class C | 0.02% | 0.02% | 0.02% | 83% | 91% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.01% | 81% | 92% |
| Class D | 0.04% | -0.02% | 0.02% | 83% | 87% |
| Class F | -0.03% | -0.06% | -0.06% | 81% | 92% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.17% | 0.25% | 87% | 91% |
| Class C | 0.02% | 0.05% | 0.20% | 75% | 79% |
| Class E | 0.06% | 0.16% | -0.12% | 78% | 84% |
| Overall | 0.03% | -0.01% | 0.14% | 81% | 85% |
| Class D | -0.02% | -0.29% | 0.26% | 81% | 82% |
| Class F | -0.05% | 0.14% | -0.15% | 71% | 76% |

## Test #2

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.03% | 0.03% | 80% | 89% |
| Class A2 | 0.01% | 0.01% | 0.11% | 82% | 92% |
| Class B | 0.01% | 0.07% | -0.03% | 80% | 97% |
| Class C | 0.02% | -0.03% | 0.00% | 82% | 89% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.02% | 81% | 92% |
| Class D | 0.02% | -0.03% | 0.03% | 87% | 92% |
| Class F | 0.03% | -0.01% | 0.04% | 87% | 105% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.28% | -0.14% | 82% | 86% |
| Class C | 0.09% | 0.26% | -0.02% | 80% | 83% |
| Class E | 0.11% | 0.10% | 0.13% | 80% | 88% |
| Overall | 0.09% | 0.00% | -0.04% | 81% | 86% |
| Class D | 0.10% | 0.07% | -0.39% | 74% | 74% |
| Class F | 0.11% | 0.29% | -0.02% | 73% | 79% |

## Test #3

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.01% | 0.04% | 89% | 99% |
| Class A2 | -0.03% | 0.05% | 0.06% | 83% | 96% |
| Class B | 0.00% | -0.01% | -0.03% | 82% | 99% |
| Class C | 0.06% | 0.08% | 0.05% | 82% | 94% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.03% | 0.03% | 83% | 97% |
| Class D | 0.06% | 0.02% | -0.02% | 83% | 95% |
| Class F | -0.02% | -0.02% | -0.05% | 83% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.27% | -0.10% | 82% | 88% |
| Class C | 0.07% | 0.30% | 0.00% | 81% | 84% |
| Class E | 0.13% | 0.24% | -0.04% | 79% | 84% |
| Overall | 0.09% | 0.05% | -0.05% | 81% | 86% |
| Class D | 0.13% | 0.41% | -0.01% | 73% | 72% |
| Class F | -0.01% | 0.27% | 0.08% | 77% | 82% |

# Reference

[1]. https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/merge_requests/135