| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0588 |
| --- | --- |

| Title: | Crosscheck of JVET-M0271 (CE10-related: Merge list construction process for triangular prediction mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Huan Dou, Zhipin Deng, Lidong Xu | Tel: Email: | huan.dou@intel.com zhipin.deng@intel.com lidong.xu@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution reports a cross-checking of contribution JVET-M0271 related to merge list construction process for triangular prediction mode.

It is reported that the result obtained during this cross-checking matches the result described in JVET-M0271.

# Introduction

In JVET-M0271[1], it is proposed to simplify the TPM merge list construction process using three methods. In solution #1, the regular merge list construction process is reused. Only regular merge candidates may be pruned and there is no TPM candidate pruning operations. In solution #2, the same spatial and one temporal blocks may be accessed to derive regular merge candidates without pruning. Pruning is only invoked to derived TPM candidates. Different from solution #2, solution #3 need to always access to 2 temporal blocks.

# Test Result

The cross-check test is performed on top of VTM3.0. The results of all three tests are shown in the following table, and they match the results shown in JVET-M0271.

## Solution #1

The regular merge list construction process (before invoking pairwise bi-predication) is fully reused.

## Solution #2

In this solution, one regular motion candidate before pruning is directly used to derive TPM candidates and full pruning is applied to the first M TPM candidates. In our simulation results, M is set to 6.

## Solution #3

The general process is the same as that in solution #2. The only difference is two instead of one temporal blocks (block 6 and 7 in Figure 1) are always checked.

# Patent rights declaration(s)

Intel Corporation does not have any current or pending patent rights relating to the technology described in this contribution

# Reference

- L. Zhang, K. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong, “CE10-related: Merge list construction process for triangular prediction mode”, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.