| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0600-v2 |
| --- | --- |

| Title: | AHG10: Quality dependency factor based rate control for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zizheng Liu, Zhenzhong Chen, Yiming Li Wuhan University Yeqing Wu, Shan Liu Tencent | Email: | zzchen@whu.edu.cn shanl@tencent.com |
| Source: | Wuhan University, Tencent |  |  |

_____________________________

- Abstract

This contribution presents some improvements based on the current rate control scheme proposed in JVET-K0390. With the proposed quality dependency factor based bit allocation algorithm, when using the anchor bit rate of VTM 3.0 as the target, there are 0.34%/3.45%/3.02% for Y/U/V coding efficiency improvements in random access (RA) configuration when compared with the rate control algorithm in JVET-K0390.

Introduction

The R-lambda rate control algorithm for HEVC, JCTVC-K0103 [1] and M0036 [2], were introduced and adopted in HM at JCT-VC. Recently JVET-K0390 [3] further developed the R-lambda rate control algorithm by updating the rate-distortion (R-D) parameters of skip and non-skip CTUs separately with different strategies. In JVET-L0241 [4] an adaptive lambda ratio estimation method was proposed for RA configuration. In this proposal, we introduce a quality dependency factor (QDF) derived from skip ratio into frame-level bit allocation process to enhance the performance of rate control.

Proposed Implementation

In this proposal, we introduce a quality dependency factor (QDF) to describe the quality dependency among different coding levels in the hierarchical coding structure.

A GOP in RA configuration contains 16 pictures belonging to 5 coding levels, where the pictures in higher level will choose the pictures in the lower as their reference pictures. Thus, there exists quality dependency among different pictures in different coding level, which can be expressed as:

(1)

where  mean the coding levels and ,  means the distortion.

We executed some experiments and found that quality dependency factor has strong relationship with the skip ratio of the coding picture. According to the experimental results, the QDF can be obtained by a linear function with the skip ratio as:

(2)

Where  represents the skip ratio of th coding levels which can be acquired from last encoded GOP,  and  are the parameters obtained by some analysis of VTM software when rate control is not applied. The parameters  and  can be seen in Table 1.

Table 1 Parameters of relationships between QDF and skip ratio

| Level |  |  |
| --- | --- | --- |
| 2 | 0.5847 | -0.0782 |
| 3 | 0.5468 | -0.1364 |
| 4 | 0.6539 | -0.203 |
| 5 | 0.8623 | -0.4676 |

The QDF of other coding levels can be get by:

(3)

Supposing the target number of bits for this GOP is , the picture level bit allocation problem is to solve the constrained optimization problem as:

(4)

This problem can be converted to an unconstrained optimization problem by introducing a Lagrange multiplier:

(5)

The solution of this problem can be expressed as ratios of lambdas:

(6)

Where  represents the influence of picture , in this proposal,  is determined by QDFs from the pictures that will take th picture as their reference picture:

(7)

Results

The experiments were based on VTM 3.0 and were carried out on HPC cluster in Linux platform. Compared with the method in JVET-K0390, our method can achieve 0.34%/3.45%/3.02% for Y/U/V coding efficiency improvements in random access configuration as shown in Table 2. The performance of the proposed method compared to fixed QP method is also shown in Table 3.

Table 2 Proposed method vs. JVET-K0390

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% |  | -3.21% |  | -2.16% |  | 0.98 |  | 0.97 |  |
| Class A2 | -0.18% |  | -3.38% |  | -3.25% |  | 0.98 |  | 0.99 |  |
| Class B | -0.15% | -4.18% | -3.93% | 0.98 | 1.01 |
| Class C | -0.81% | -2.79% | -2.36% | 0.99 | 1.01 |
| Class E |  |  |  |  |  |
| Overall | -0.34% |  | -3.45% |  | -3.02% |  | 0.99 |  | 1.00 |  |
| Class D | -0.81% | -4.90% | -4.15% | 0.98 | 1.02 |
| Class F | -1.65% |  | -2.43% |  | -2.26% |  | 0.97 |  | 0.99 |  |

Table 3 Proposed method vs. Fixed QP

|  |  |  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Random Access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  |  |  | U |  |  | V |  |  | EncT |  | DecT |  |  | Y |  |  |  |  | U |  | V |  |  |  | EncT |  |  | DecT |  |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.78% |  |  |  |  | 4.16% |  |  | 4.36% |  |  | 0.99 |  |  | 1.06 |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.50% |  |  |  |  | 2.51% |  | 2.38% |  |  |  | 0.99 |  |  | 0.98 |  |  |
| Class B | -2.21% |  |  |  | -11.73% |  |  | -13.01% |  |  | 1.01 |  | 1.03 |  |  | 3.30% |  |  |  |  | 2.23% |  | 3.09% |  |  |  | 1.02 |  |  | 1.11 |  |  |
| Class C | -2.63% |  |  |  | -8.77% |  |  | -8.04% |  |  | 1.01 |  | 1.01 |  |  | 1.79% |  |  |  |  | -0.29% |  | 0.96% |  |  |  | 1.00 |  |  | 1.06 |  |  |
| Class E | 0.26% |  |  |  | -5.43% |  |  | -5.55% |  |  | 0.97 |  | 1.01 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  | -1.74% |  |  |  | -9.17% |  |  | -9.49% |  |  | 1.00 |  | 1.02 |  |  | 4.43% |  |  |  |  | 2.00% |  |  | 2.63% |  |  | 1.00 |  |  | 1.06 |  |
| Class D | -1.01% |  |  |  | -5.16% |  |  | -3.35% |  |  | 1.02 |  | 1.02 |  |  | 3.15% |  |  |  |  | 9.02% |  | 11.51% |  |  |  | 1.00 |  |  | 1.03 |  |  |
| Class F |  |  |  | 9.86% |  |  | -2.59% |  |  | -3.05% |  |  |  |  | 1.04 |  |  | 1.00 |  | 9.53% |  |  |  | 5.87% |  | 5.12% |  |  | 1.15 |  |  | 1.03 |

Patent rights declaration(s)

Wuhan University and Tencent America LLC do not have any current or pending patent rights relating to the technology described in this contribution.

Reference

- B. Li, H. Li, L. Li, and J. Zhang, “Rate control by R-lambda model for HEVC,” JCT-VC K0103, Oct. 2012.

- B. Li, H. Li, and L. Li, “Adaptive bit allocation for R-lambda model rate control in HM,” JCT-VC M0036, Apr. 2013.

- Y. Li, Z. Chen, X. Li and S. Liu, “Rate Control for VVC,” JVET-K0390, Jul. 2018.

- Z. Liu, Y. Li, Z. Chen, X. Li and S. Liu, “AHG10: Adaptive lambda ratio estimation for rate control in VVC,” JVET-L0241, Oct. 2018.