| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0653-v1 |
| --- | --- |

| Title: | Non-CE3: Harmonization of integer-slope directional modes without interpolation filtering process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Jianle Chen | Email: | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co. |  |  |

_____________________________

# Abstract

This document proposes Harmonization of integer-slope directional modes that do not require interpolation filtering process. The experimental results reportedly shows the following overall results: 0.00% (Y), 0.02% (U), -0.01% (V) and 0.00% (Y), -0.03% (U), 0.06% (V) for AI and RA configurations, respectively. The running time is negligibly changed at both encoder and decoder sides for AI and RA configurations as compared with VTM-3.0.

# Introduction

In the JVET-L meeting, 4-tap interpolation filters for intra prediction were adopted into the VTM software and the VVC spec draft as proposed in [1]. Thus, in the 3rd revision of the VVC spec draft, the directional intra modes with indices of 2, 34, and 66 are treated similar to Planar mode, i.e. using reference sample filtering rather than interpolation filtering to enable computationally simple processing. However, adopting JVET-L0279 that introduces more integer-slope directional modes, which do not require interpolation filtering, resulted in the inconsistency of integer-slope mode processing in the VVC draft text. Thus, we propose to consistently treat every integer-slope mode.

# Proposal

In this contribution, we propose to applying [1, 2, 1]/4 filter to reference samples with copying them into a block to be predicted rather than performing 4-tap interpolation filtering for every sample within a block to be predicted. Similar reference sample processing is already used in the case of Planar intra-prediction mode where PDPC is applied to either unfiltered or filtered reference samples subject to block size. The proposed modification of the VVC draft 3 text is attached together with this document.

# Experiment Results

The results obtained using the common test conditions [2] are reportedly presented in Table 1 for AI and RA configurations.

Table 1. Experimental results for this proposal

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | -0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.06% | 0.02% | 100% | 100% |
| Class B | 0.00% | 0.02% | -0.04% | 99% | 99% |
| Class C | 0.01% | 0.04% | -0.02% | 100% | 100% |
| Class E | 0.01% | 0.01% | -0.01% | 99% | 100% |
| Overall | 0.00% | 0.02% | -0.01% | 100% | 100% |
| Class D | 0.00% | 0.10% | 0.13% | 100% | 99% |
| Class F | 0.00% | 0.07% | 0.13% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.13% | 0.05% | 100% | 99% |
| Class A2 | -0.03% | -0.09% | 0.19% | 100% | 99% |
| Class B | -0.01% | 0.10% | -0.03% | 100% | 100% |
| Class C | 0.03% | -0.05% | 0.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | 0.06% | 100% | 100% |
| Class D | 0.00% | -0.34% | 0.14% | 100% | 100% |
| Class F | 0.01% | -0.10% | 0.02% | 100% | 100% |

# Conclusion

We propose to adopt the proposed changes into the VTM software and the VVC spec draft.

# References

- A. Filippov, V. Rufitskiy, J. Chen, G. Van der Auwera, A.K. Ramasubramonian, V. Seregin, T. Hsieh, M. Karczewicz, “CE3: A combination of tests 3.1.2 and 3.1.4 for intra reference sample interpolation filter”, JVET-L0628, 12th JVET Meeting, Macao, China, October 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, 12th JVET Meeting, Macao, China, October 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).