| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0661 |
| --- | --- |

| Title: | AhG-13: On Merge List Size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li, Xiaozhong Xu, Shan Liu | Tel: Email: | {xlxiangli, xiaozhongxu, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In CTC, by default regular merge list size is set to 6 while sub-block merge list size is set to 5. It is reported that regular merge list size 6 brings minor coding benefit over size 5, i.e., 0.06%, 0.10% and 0.09% luma BD-rate reduction. It is proposed to use size 5 in CTC.

# Introduction

In CTC for VTM-3.0, the size of regular merge list is set to 6 while the size of sub-block merge list is 5. During the integration of VTM-3.0, it was observed that minor coding benefit was obtained by size 6 over size 5. It is proposed to use the same size for regular merge list and sub-block merge list, namely size 5.

# Simulation results

Experiment was performed under CTC condition [2]. The anchor is VTM-3.0 CTC setting, the test is VTM-3.0 with MaxNumMergeCand=5 and MaxNumAffineMergeCand=5.

Table 1 VTM-3.0 with MaxNumMergeCand=5 vs VTM-3.0 CTC (MaxNumMergeCand=6)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.03% | 0.01% | 100% | 101% |
| Class A2 | 0.06% | 0.09% | 0.19% | 100% | 100% |
| Class B | 0.08% | 0.17% | 0.01% | 100% | 101% |
| Class C | 0.07% | 0.07% | 0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.10% | 0.06% | 100% | 100% |
| Class D | 0.00% | -0.02% | 0.03% | 100% | 102% |
| Class F | 0.06% | 0.04% | 0.04% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.14% | -0.02% | 0.30% | 99% | 100% |
| Class C | 0.05% | 0.34% | 0.26% | 100% | 101% |
| Class E | 0.11% | -0.02% | 0.30% | 101% | 101% |
| Overall | 0.10% | 0.10% | 0.29% | 100% | 100% |
| Class D | 0.07% | 0.14% | 0.12% | 101% | 101% |
| Class F | 0.10% | 0.24% | 0.22% | 100% | 98% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.12% | 0.30% | 0.41% | 100% | 100% |
| Class C | 0.10% | 0.35% | 0.18% | 99% | 99% |
| Class E | 0.01% | -0.21% | -0.43% | 98% | 97% |
| Overall | 0.09% | 0.19% | 0.12% | 99% | 99% |
| Class D | 0.06% | -0.15% | 0.15% | 98% | 97% |
| Class F | 0.18% | -0.20% | 0.02% | 99% | 100% |

# Conclusions

In VTM-3.0 CTC, the size of regular merge list size is set to 6. However, it is observed that size 6 does not bring much benefit over size 5. It is proposed to use the same size (5) for both regular merge and sub-block merge in CTC.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” JVET-L1001, 12th meeting, Macao, CN, 3–12 Oct. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th meeting, Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.