| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0685 |
| --- | --- |

| Title: | Non-CE7: On derivation of quantization parameter predictor |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Misra, A. Segall | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

# Abstract

In the current VVC WD, the derivation of quantization parameter predictor qPY_PRED may in some cases be based on a previous quantization group in decoding order that is not spatially close to the current coding unit (CU). For example, the previous quantization group in decoding order may belong to the rightmost Coding Tree Unit (CTU) in the CTU row above. This contribution therefore proposes two possible options to address the above:

Option 1: qPY_PRED be derived based on spatial neighbors (when available)

Option 2: A high-level flag is used to control the quantization parameter predictor derivation process, which enables support for both Option 1 and the current design in the VVC WD if so desired.

# Introduction

In current VVC WD [1], qPY_PRED is typically computed as:

qPY_PRED = ( qPY_A + qPY_B + 1 ) >> 1

where

qPY_A in most cases, is set equal to the QpY of the coding unit (CU) to the left of the current CU;

qPY_B in most cases, is set equal to the QpY of the coding unit above the current CU.

However, in some cases, each of qPY_A and qPY_B may be set equal to a variable qPY_PREV, where qPY_PREV is set equal to the QpY of the last coding unit in the previous quantization group in decoding order or is set equal to SliceQpY. Such calculations of qPY_PRED may be less than ideal because the last CU in the previous quantization group in decoding order may not be spatially close to the current CU and might therefore not serve as a good predictor (or not even known at the encoder due to the use of non-normative, parallelization strategies). In some cases, the previous quantization group in decoding order may belong to the rightmost Coding Tree Unit (CTU) in the CTU row above. The contribution therefore proposes that qPY_PRED be computed as follows:

qPY_PRED = ( qPY_A + qPY_B + 1 ) >> 1; when both qPY_A and qPY_B are available;

qPY_PRED = qPY_A; when only qPY_A is available;

qPY_PRED = qPY_B; when only qPY_B is available; or

qPY_PRED = SliceQpY; when neither of qPY_A and qPY_B are available.

We label this approach Option 1.

Alternatively, a high-level flag is used to control the quantization parameter predictor derivation employed with Option 1 being used when the flag is 1, Otherwise the current design in VVC WD is employed

Changes to the VVC WD are listed in appendix included at the end of this document.

# Recommendations

The contribution proposes two possible options that enables derivation of quantization parameter predictor based solely on spatial neighbors. It is recommended that the committee adopt one of these options in the VVC WD.

# References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” JVET-L1001 12th Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, 3–12 Oct. 2018

# Patent rights declaration(s)

Sharp Laboratories of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).