| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0702_r1 |
| --- | --- |

| Title: | CE2-related: Adaptive sub-block MV clip for affine blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li, Min Gao, Shan Liu | Tel: Email: | {xlxiangli, maxwellgao, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, adaptive sub-block MV clip is proposed for affine blocks to reduce worst case memory bandwidth while keeping coding efficiency based on constraining max difference among 4x4 level MVs within an 8x8 sub-blocks to no larger than 1 pixel. It is further proposed to use different thresholds for x and y components for better trade-off. It is reported 0.16%/0.06%/0.03%, 0.09%/0.03%/0.01%, 0.11%/0.03%/0.03%, and 0.01%/-0.02%/0.00% luma BD-rate increase over VTM-3.0 for RA, LDB, and LDP when {Tx=0, Ty=0}, {Tx=1, Ty=0}, {Tx=0, Ty=1}, and {Tx=1, Ty=1}, respectively. With the proposed methods, the worst case memory bandwidth for affine blocks is reportedly reduced to 7.03, 7.5, 7.5 and 8 luma samples/pixel when {Tx=0, Ty=0}, {Tx=1, Ty=0}, {Tx=0, Ty=1}, and {Tx=1, Ty=1}, respectively. Note that 7.03 luma samples/pixel is the memory bandwidth for regular 8x8 bi-predicted blocks. SIMD implementation of the proposed method is also provided.

# Introduction

In affine prediction, the uni-directional inter prediction for 4x4 block needs to load up to (4+7)*(4+7)=121 luma samples, thus the per-pixel memory bandwidth is 7.6 luma sample per pixel. For bi-directional affine prediction, the per-pixel memory bandwidth is up to 15.125 luma sample per pixel. To reduce the per-pixel memory bandwidth in affine prediction, CE-2.4.5 (JVET-M0488) proposes to constrain the MVs of four 4x4 block within one 8x8 sub-block such that the max difference between integer parts of the four 4x4 sub-block MVs is no more than 1 pixel. Consequently, the worst case memory bandwidth is reduced to (8+7+1)x(8+7+1)/(8x8)x2=8 luma sample per pixel.

# Proposed methods

To achieve better tradeoff between worst case memory bandwidth and coding performance, adaptive clip is proposed based on the distance between MV and the integer part of MV.

For each 8x8 sub-block in an affine block, the minimal and maximal MVmin and MVmax are first found. Then MVminr and MVmaxr are derived as follows, where T is the threshold which indicates the maximal allowable difference among the 4x4 level MVs within the 8x8 sub-block, in terms of pixel. Note that when T = 0, 4x4 level MVs share the same integer part within the 8x8 sub-block.

MVminr=MVmin >> 4 << 4

MVmaxr=(MVmax>> 4 << 4) + ((T+1)<< 4)-1

Then MVmin and MVmax are updated as follows, where offset = ((T+1)<<4)-1

if ((MVmin - MVminr) < (MVmaxr - MVmax))

{

MVmin = MVminr;

MVmax = MVmin + offset;

}

else

{

MVmax = MVmaxr;

MVmin = MVmax - offset;

}

After getting MVmax and MVmin, they are used to clip each 4x4 level MV within the 8x8 sub-block.

It is further proposed to use different Tx and Ty for different tradeoff between worst case memory and coding efficiency. The following table summarizes the worst case memory bandwidth for different Tx and Ty. It should be noted that when Tx=Ty=0, the worst case memory bandwidth of affine is the same as regular bi-predicted 8x8 blocks.

| Tx | Ty | Worst case memory bandwidth for affine blocks |  |
| --- | --- | --- | --- |
| 0 | 0 | 2x(8+7+0)x(8+7+0)/(8x8)=7.03125 |  |
| 1 | 0 | 2x(8+7+1)x(8+7+0)/(8x8)=7.5 |  |
| 0 | 1 | 2x(8+7+0)x(8+7+1)/(8x8)=7.5 |  |
| 1 | 1 | 2x(8+7+1)x(8+7+1)/(8x8)=8 |  |

# Simulation results

Experiments are conducted to evaluate the proposed methods. Each experiment is performed under CTC condition [2]. The anchor for both the experiments is VTM-3.0.

## Tx=0, Ty=0

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.01% | 0.05% | 99% | 99% |
| Class A2 | 0.46% | 0.27% | 0.28% | 98% | 99% |
| Class B | 0.13% | 0.16% | 0.00% | 98% | 96% |
| Class C | 0.06% | 0.00% | 0.02% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | 0.11% | 0.07% | 99% | 97% |
| Class D | 0.07% | 0.06% | 0.03% | 99% | 98% |
| Class F | 0.29% | 0.21% | 0.18% | 100% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.20% | 0.16% | 99% | 100% |
| Class C | 0.04% | 0.23% | -0.01% | 99% | 94% |
| Class E | 0.08% | -0.01% | 0.19% | 100% | 93% |
| Overall | 0.06% | -0.01% | 0.11% | 99% | 96% |
| Class D | 0.03% | 0.39% | -0.05% | 101% | 96% |
| Class F | 0.19% | 0.31% | -0.11% | 101% | 95% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.42% | 0.17% | 101% | 100% |
| Class C | 0.10% | 0.20% | -0.10% | 101% | 93% |
| Class E | -0.11% | -0.01% | -0.43% | 99% | 91% |
| Overall | 0.03% | 0.24% | -0.07% | 100% | 95% |
| Class D | 0.01% | 0.03% | 0.36% | 98% | 92% |
| Class F | 0.39% | 0.26% | 0.58% | 100% | 94% |

## Tx=1, Ty=0

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.03% | -0.03% | 100% | 98% |
| Class A2 | 0.25% | 0.10% | 0.18% | 99% | 98% |
| Class B | 0.07% | 0.03% | 0.03% | 99% | 96% |
| Class C | 0.02% | -0.03% | 0.03% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.01% | 0.05% | 100% | 97% |
| Class D | 0.00% | -0.14% | -0.03% | 100% | 100% |
| Class F | 0.08% | 0.07% | 0.11% | 100% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.12% | 0.05% | 100% | 98% |
| Class C | 0.04% | 0.11% | 0.06% | 100% | 96% |
| Class E | 0.04% | -0.04% | -0.39% | 102% | 96% |
| Overall | 0.03% | -0.02% | -0.06% | 101% | 97% |
| Class D | 0.01% | 0.12% | 0.03% | 102% | 95% |
| Class F | 0.15% | 0.13% | -0.06% | 102% | 94% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.28% | 0.27% | 101% | 99% |
| Class C | 0.06% | 0.24% | 0.01% | 100% | 93% |
| Class E | -0.08% | -0.15% | -0.77% | 100% | 91% |
| Overall | 0.01% | 0.16% | -0.08% | 101% | 95% |
| Class D | 0.00% | 0.05% | 0.20% | 100% | 94% |
| Class F | 0.20% | -0.20% | 0.11% | 101% | 95% |

## Tx=0, Ty=1

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.01% | 0.02% | 100% | 99% |
| Class A2 | 0.30% | 0.17% | 0.20% | 99% | 98% |
| Class B | 0.09% | 0.05% | -0.01% | 99% | 96% |
| Class C | 0.05% | -0.08% | 0.07% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.11% | 0.03% | 0.06% | 100% | 97% |
| Class D | 0.01% | 0.09% | 0.10% | 100% | 99% |
| Class F | 0.24% | 0.16% | 0.21% | 100% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.20% | -0.15% | 100% | 99% |
| Class C | 0.05% | 0.20% | 0.28% | 100% | 96% |
| Class E | 0.01% | 0.37% | -0.59% | 101% | 95% |
| Overall | 0.03% | 0.08% | -0.12% | 100% | 97% |
| Class D | 0.05% | 0.20% | -0.37% | 100% | 93% |
| Class F | 0.14% | 0.14% | 0.04% | 102% | 93% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.08% | 0.27% | 101% | 98% |
| Class C | 0.08% | 0.05% | -0.03% | 100% | 94% |
| Class E | 0.00% | -0.24% | -0.97% | 99% | 91% |
| Overall | 0.03% | -0.01% | -0.14% | 100% | 95% |
| Class D | 0.03% | -0.45% | -0.21% | 99% | 94% |
| Class F | 0.29% | -0.02% | -0.08% | 101% | 94% |

## Tx=1, Ty=1

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.03% | -0.05% | 100% | 99% |
| Class A2 | 0.00% | 0.03% | 0.04% | 100% | 99% |
| Class B | 0.00% | 0.07% | -0.07% | 100% | 96% |
| Class C | 0.02% | -0.14% | 0.00% | 101% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | -0.03% | 100% | 97% |
| Class D | -0.01% | -0.05% | -0.04% | 100% | 99% |
| Class F | 0.01% | -0.01% | 0.01% | 101% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.25% | -0.03% | 101% | 98% |
| Class C | -0.02% | 0.03% | -0.03% | 100% | 95% |
| Class E | -0.09% | -0.01% | 0.34% | 104% | 95% |
| Overall | -0.02% | -0.10% | 0.06% | 102% | 96% |
| Class D | 0.05% | 0.14% | 0.24% | 100% | 93% |
| Class F | 0.06% | 0.28% | 0.15% | 102% | 93% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.24% | 0.16% | 101% | 98% |
| Class C | -0.01% | -0.04% | -0.06% | 100% | 93% |
| Class E | 0.02% | -0.37% | -0.93% | 99% | 90% |
| Overall | 0.00% | -0.01% | -0.18% | 100% | 94% |
| Class D | -0.01% | -0.16% | -0.31% | 99% | 93% |
| Class F | 0.10% | -0.09% | -0.43% | 101% | 95% |

# Conclusions

In this contribution, adaptive clip is proposed to reduce worst case memory bandwidth for affine blocks while keeping coding efficiency. It is further proposed to use different threshold for x and y components. Simulations show that different tradeoff can be achieved with the proposed methods.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” JVET-L1001, 12th meeting, Macao, CN, 3–12 Oct. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th meeting, Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).