| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0707-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-M0188 : CE7-related: On quantization parameter signalling considering CU area |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu |  | xiaozhongxu@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the results of a crosscheck for JVET-M0188: CE7-related: On quantization parameter signalling considering CU area. Results generated by the crosschecker match what were provided by the proponent.

# Experimental results

The crosscheck has been performed on VTM-3.0 software. The corresponding simulations were conducted on Linux OS and a GCC 6.2.0 compiler. The tests listed in table 2 and 4 which are proposed by the proponents in [1] have been crosschecked.

Table 2: Test results

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -1.37% | -0.92% | 306% | 108% |
| Class A2 | -0.35% | -1.41% | -1.05% | 304% | 106% |
| Class B | -0.32% | -0.82% | -1.23% | 304% | 104% |
| Class C | -0.36% | -1.14% | -1.36% | 308% | 105% |
| Class E | -0.44% | -1.06% | -1.30% | 302% | 105% |
| Overall | -0.35% | -1.12% | -1.19% | 305% | 105% |
| Class D | -0.25% | -1.01% | -1.17% | 306% | 106% |
| Class F | -0.63% | -1.17% | -1.25% | 305% | 104% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -1.41% | -0.66% | 268% | 104% |
| Class A2 | -0.36% | -1.26% | -0.97% | 267% | 104% |
| Class B | -0.29% | -0.67% | -1.18% | 271% | 101% |
| Class C | -0.17% | -0.70% | -0.54% | 270% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -0.94% | -0.86% | 269% | 102% |
| Class D | -0.24% | -1.09% | -0.85% | 272% | 97% |
| Class F | -0.50% | -0.65% | -0.73% | 278% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.19% | -1.49% | -1.63% | 273% | 107% |
| Class C | 0.22% | -0.71% | -0.89% | 272% | 100% |
| Class E | 0.13% | -0.65% | -0.39% | 279% | 100% |
| Overall | 0.19% | -1.02% | -1.08% | 274% | 103% |
| Class D | -0.05% | -1.96% | -1.13% | 270% | 98% |
| Class F | 0.04% | -0.68% | -1.42% | 278% | 101% |

Table 4: Test results

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.07% | 0.60% | 305% | 108% |
| Class A2 | -0.14% | -0.25% | -0.19% | 300% | 106% |
| Class B | -0.13% | 0.33% | -0.10% | 300% | 105% |
| Class C | -0.32% | -0.64% | -1.02% | 301% | 105% |
| Class E | -0.24% | 0.36% | 0.15% | 297% | 104% |
| Overall | -0.15% | -0.02% | -0.16% | 300% | 106% |
| Class D | -0.26% | -0.66% | -0.92% | 301% | 108% |
| Class F | -0.55% | -0.68% | -0.92% | 299% | 105% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.21% | -0.62% | -0.10% | 286% | 105% |
| Class A2 | -0.12% | -0.57% | -0.38% | 285% | 104% |
| Class B | -0.06% | -0.18% | -0.44% | 288% | 101% |
| Class C | 0.02% | -0.47% | -0.48% | 288% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.42% | -0.37% | 287% | 102% |
| Class D | -0.02% | -0.68% | -0.69% | 288% | 98% |
| Class F | -0.45% | -0.11% | -0.43% | 294% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.33% | -1.11% | -0.93% | 295% | 106% |
| Class C | 0.45% | -0.29% | -0.59% | 291% | 100% |
| Class E | 0.14% | 1.07% | -0.08% | 309% | 101% |
| Overall | 0.32% | -0.29% | -0.61% | 297% | 103% |
| Class D | 0.27% | -0.21% | -0.32% | 284% | 96% |
| Class F | 0.04% | -0.71% | -0.91% | 302% | 101% |

# Conclusion

All of the results match the ones presented by the proponents in [1].

# Reference

[1] O. Chubach, T.-D. Chuang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek), “CE7-related: On quantization parameter signalling considering CU area”. JVET-M0188, Marrakech, MA, Jan. 2019.