| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0727-v1 |
| --- | --- |

| Title: | CE5: Results of tests CE5.1.5, CE5.1.6, and CE5.1.7 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | H. Kirchhoffer, C. Bartnik, P. Haase, S. Matlage, J. Stegemann, D. Marpe, H. Schwarz, T. Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-184 heiner.kirchhoffer@ hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution presents the results of the tests CE5.1.5, CE5.1.6, and CE5.1.7. These tools include state-based probability update of JVET-L0462 with fixed and variable window sizes and a 32x8x8-bit table containing the pre-calculated values of the LPS range table. The context model initialization values and the variable window sizes were retrained jointly.

# Results of experiments

The proposed changes are integrated into vtm-3.0. The simulations are performed using the common test conditions.

## CE5.1.5: 2 states (8+12 bits), fixed window sizes

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,85% | -1,36% | -1,34% | 100% | 95% |
| Class A2 | -0,81% | -1,60% | -1,44% | 101% | 98% |
| Class B | -0,76% | -1,44% | -1,41% | 101% | 98% |
| Class C | -0,73% | -1,38% | -1,40% | 101% | 100% |
| Class E | -0,69% | -1,60% | -1,45% | 100% | 99% |
| Overall | -0,76% | -1,47% | -1,41% | 101% | 98% |
| Class D | -0,70% | -1,30% | -1,09% | 101% | 102% |
| Class F (optional) | -0,60% | -0,96% | -0,89% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,65% | -0,83% | -0,90% | 96% | 92% |
| Class A2 | -0,60% | -0,41% | -0,56% | 97% | 93% |
| Class B | -0,68% | -0,80% | -0,75% | 97% | 93% |
| Class C | -0,57% | -1,03% | -0,57% | 98% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0,63% | -0,79% | -0,70% | 97% | 93% |
| Class D | -0,49% | -1,15% | -0,48% | 99% | 97% |
| Class F (optional) | -0,40% | -0,45% | -0,47% | 98% | 91% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,69% | -0,20% | 0,33% | 94% | 86% |
| Class C | -0,64% | -0,58% | -0,11% | 97% | 99% |
| Class E | -0,73% | -0,06% | -0,09% | 92% | 92% |
| Overall | -0,68% | -0,29% | 0,08% | 94% | 91% |
| Class D | -0,59% | -0,31% | 0,23% | 98% | 101% |
| Class F (optional) | -0,22% | -0,46% | 0,01% | 95% | 90% |

## CE5.1.6: 2 states, variable window size

### States use 8 and 12 bits

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,90% | -1,33% | -1,41% | 102% | 95% |
| Class A2 | -0,86% | -1,59% | -1,46% | 103% | 98% |
| Class B | -0,82% | -1,51% | -1,46% | 103% | 97% |
| Class C | -0,82% | -1,49% | -1,61% | 103% | 100% |
| Class E | -0,77% | -1,63% | -1,71% | 102% | 98% |
| Overall | -0,83% | -1,51% | -1,53% | 103% | 98% |
| Class D | -0,81% | -1,24% | -1,12% | 103% | 103% |
| Class F (optional) | -0,60% | -1,12% | -1,10% | 103% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,78% | -0,76% | -0,94% | 97% | 92% |
| Class A2 | -0,71% | -0,22% | -0,61% | 97% | 93% |
| Class B | -0,81% | -0,80% | -0,75% | 97% | 92% |
| Class C | -0,73% | -1,16% | -0,72% | 99% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0,76% | -0,77% | -0,75% | 98% | 93% |
| Class D | -0,76% | -1,25% | -0,82% | 100% | 96% |
| Class F (optional) | -0,46% | -0,75% | -0,70% | 99% | 90% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,81% | -0,41% | 0,19% | 94% | 86% |
| Class C | -0,71% | -0,72% | -0,19% | 98% | 100% |
| Class E | -0,75% | -0,13% | -0,66% | 92% | 92% |
| Overall | -0,76% | -0,44% | -0,15% | 95% | 92% |
| Class D | -0,69% | -0,13% | -0,12% | 99% | 101% |
| Class F (optional) | -0,47% | -0,36% | -0,26% | 95% | 90% |

### States use 10 and 14 bits

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1,01% | -1,49% | -1,39% | 104% | 94% |
| Class A2 | -0,97% | -1,64% | -1,48% | 104% | 99% |
| Class B | -0,91% | -1,40% | -1,40% | 104% | 97% |
| Class C | -0,86% | -1,26% | -1,41% | 103% | 99% |
| Class E | -0,84% | -1,49% | -1,59% | 103% | 97% |
| Overall | -0,91% | -1,44% | -1,44% | 104% | 97% |
| Class D | -0,81% | -1,05% | -1,05% | 103% | 102% |
| Class F (optional) | -0,73% | -0,95% | -0,86% | 103% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,82% | -0,99% | -1,09% | 97% | 92% |
| Class A2 | -0,76% | -0,54% | -0,71% | 97% | 94% |
| Class B | -0,87% | -0,96% | -0,92% | 98% | 93% |
| Class C | -0,77% | -1,07% | -0,70% | 99% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0,81% | -0,92% | -0,85% | 98% | 93% |
| Class D | -0,66% | -0,93% | -0,89% | 100% | 97% |
| Class F (optional) | -0,54% | -0,65% | -0,64% | 98% | 89% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,79% | -0,28% | 0,18% | 95% | 86% |
| Class C | -0,68% | -0,67% | -0,33% | 98% | 98% |
| Class E | -0,66% | -0,32% | -0,05% | 92% | 92% |
| Overall | -0,72% | -0,42% | -0,05% | 95% | 92% |
| Class D | -0,73% | -0,05% | -0,31% | 99% | 100% |
| Class F (optional) | -0,44% | -0,58% | -0,58% | 96% | 91% |

## CE5.1.7: 1 state (12 bits), variable window sizes

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,72% | -1,24% | -1,06% | 100% | 94% |
| Class A2 | -0,71% | -1,34% | -1,21% | 100% | 98% |
| Class B | -0,65% | -1,07% | -1,06% | 100% | 98% |
| Class C | -0,61% | -0,94% | -1,02% | 100% | 99% |
| Class E | -0,60% | -1,13% | -1,23% | 100% | 99% |
| Overall | -0,66% | -1,12% | -1,10% | 100% | 98% |
| Class D | -0,62% | -0,98% | -0,89% | 100% | 102% |
| Class F (optional) | -0,47% | -0,70% | -0,60% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,52% | -0,90% | -1,06% | 96% | 92% |
| Class A2 | -0,47% | -0,80% | -0,85% | 96% | 93% |
| Class B | -0,58% | -0,94% | -1,10% | 97% | 93% |
| Class C | -0,52% | -1,16% | -0,76% | 98% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0,53% | -0,96% | -0,95% | 97% | 93% |
| Class D | -0,53% | -1,37% | -1,03% | 99% | 97% |
| Class F (optional) | -0,30% | -0,54% | -0,47% | 98% | 91% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,53% | -0,74% | -0,40% | 94% | 87% |
| Class C | -0,48% | -0,81% | -0,36% | 97% | 98% |
| Class E | -0,56% | -0,31% | -0,95% | 91% | 91% |
| Overall | -0,52% | -0,66% | -0,52% | 94% | 92% |
| Class D | -0,51% | -0,85% | -1,12% | 98% | 101% |
| Class F (optional) | -0,31% | -0,52% | -0,46% | 95% | 90% |

# Patent rights declaration

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).