| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0736 |
| --- | --- |

| Title: | CE10-related: Triangular prediction with MMVD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Max Bläser Johannes Sauer | Tel: Email: | +49-241-80-27677 blaeser@ient.rwth-aachen.de |
| Source: | RWTH Aachen University |  |  |

_____________________________

# Abstract

This contribution proposes the combination of triangular prediction with merge with motion vector difference (MMVD) techniques. The proposed method achieves a BD-rate change of -0.14% for RA and -0.33% for LB with an increase of 157% for RA and 224% for LB in encoder complexity. No significant increase in decoder complexity is reported.

# Introduction

In JVET-J0020 [1], JVET-K0144 [2] and JVET-L0124 [3] triangular partitioning using merge mode based motion information coding is presented. The current method has the drawback of limited motion vector precision due to the nature of the merging process. Motion vectors cannot be updated. In JVET-H0087 [4], JVET-K0270 [5] and JVET-L0125 [6] a possible solution is presented by enabling AMVP for triangular partitions. In the latest proposal JVET-L0125, BD-rate changes of -0.47% for RA and -0.27% for LB are reported against the BMS-2.0.1 anchor. The encoder complexity increases to 175% for RA and 157% for LB.

In the current VVC draft JVET-L1001 [7] and the VTM software, an apparent syntax redundancy exists, where the triangle merge mode flag is signaled independent of the value of the MMVD mode flag, even though the combination of the two approaches is not possible.

Instead of removing the redundancy, it is proposed to capitalize on this and enable the combination of triangular partitioning and MMVD-based motion coding.

# Proposed method

The proposed method is implemented on top of VTM-3.0 [8]. For the combined approach, three syntax changes are necessary compared to the current VVC draft:

- If the triangle merge mode is enabled and if the MMVD mode is enabled, the decoding of an additional triangle_direction_flag syntax element is necessary, to specify the direction of the diagonal split (top-left to bottom-right or bottom-left to top-right). This syntax element is necessary as the direction of the diagonal split is otherwise inferred from the triangle_merge_index and a look-up table.

- If the triangle merge mode is enabled and if the MMVD mode is enabled, the syntax decoding processes associated with MMVD are invoked a second time, to derive the motion information of the second triangle.

- If the triangle merge mode is enabled and if the MMVD mode is disabled, the triangle_merge_index is decoded as currently specified.

Figure 1: Decoder loop changes for the integration of MMVD and triangular partitioning (red).

Figure 1 shows a simple block diagram, how the MMVD mode is integrated into the decoder loop relating to triangular prediction. It is noted that the triangle merge list is reused for the basis predictors of MMVD, which can be either the first or the second candidate of the merge list. This ensures that only uni-directional motion vectors are used for triangular prediction.

At the encoder side, a full search of all 64 possible motion vectors for both triangles (2 predictors * 4 directions * 8 magnitudes) is performed. It is noted that this differs from the approach used for regular, rectangular blocks, where a fast skipping of unlikely motion vector directions is used.

Motion vector estimation is achieved by performing a motion compensation of the luma channel for every possible motion vector, followed by computing the sum of absolute differences (SAD) for each triangle and updating the lowest RD-cost per triangle. Finally, the combination of two MMVD-based motion vectors is chosen, which results in the lowest overall RD-cost.

# Experimental results

The following experiment was conducted using the VTM-3.0 software with the VTM configuration and according to the JVET common test conditions JVET-L1010 [9] to collect the BD-rate, encoding and decoding time of the tested techniques in the Random Access (RA) and Low Delay B (LB) configurations. The anchor used was VTM-3.0 with Git hash 10a973eb47959ef56d8029b81c17538392ff4126. Additionally, memory bandwidth measurements were made in a separate decoder run according to JVET-K0451 [10].

|  | Random Access Main 10 |  |  |  |  |  |  | Random Access Main10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |  |  | Tool off |  |
|  | Y | U | V | EncT | DecT | ave mem BW | max mem BW | Impacted Pixels | Adj DecT |
| Class A1 | -0.02% | -0.17% | -0.21% | 142% | 101% | 100% | 101% | 1.51% |  |
| Class A2 | -0.19% | -0.26% | -0.39% | 154% | 101% | 100% | 100% | 1.39% |  |
| Class B | -0.14% | -0.19% | -0.07% | 157% | 99% | 100% | 101% | 1.71% |  |
| Class C | -0.19% | -0.30% | -0.31% | 171% | 101% | 100% | 100% | 3.64% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.14% | -0.23% | -0.23% | 157% | 100% | 100% | 101% | 2.12% |  |
| Class D | -0.30% | -0.41% | -0.52% | 176% | 100% | 99% | 100% | 2.91% |  |
| Class F | -0.08% | -0.13% | -0.17% | 158% | 101% | 100% | 103% | 2.56% |  |

|  | Low delay B Main10 |  |  |  |  |  |  | Low delay B Main10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |  |  | Tool off |  |
|  | Y | U | V | EncT | DecT | ave mem BW | max mem BW | Impacted Pixels | Adj DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.27% | -0.24% | -0.42% | 204% | 100% | 100% | 101% | 3.71% |  |
| Class C | -0.44% | -0.89% | -0.57% | 233% | 97% | 100% | 101% | 6.09% |  |
| Class E | -0.65% | -0.26% | -0.58% | 219% | 99% | 100% | 100% | 3.90% |  |
| Overall | -0.33% | -0.48% | -0.54% | 224% | 100% | 100% | 101% | 4.55% |  |
| Class D | -0.65% | -0.26% | -0.58% | 219% | 99% | 100% | 100% | 7.56% |  |
| Class F | -0.37% | -0.61% | 0.71% | 184% | 103% | 100% | 100% | 2.88% |  |

With the proposed method, a BD-rate change of -0.14% can be achieved for RA with an increase in encoder complexity to 157%. For LB, a BD-rate change of -0.33% can be achieved with an encoder complexity increase to 224%. No increase in decoder complexity or memory bandwidth is reported.

# Conclusion

It is proposed to further study the integration of MMVD with triangular partitioning, as no additional decoder complexity is added when the two tools are combined. It is recommended to investigate how fast-decision heuristics at the encoder side can limit the additional encoder runtime.

# References

[1]	 T. Toma, T. Nishi, K. Abe, R. Kanoh, C. Lim, J. Li, R. Liao, S. Pavan, H. Sun, H. Teo, V. Drugeon, "Description of SDR video coding technology proposal by Panasonic", Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J0020, 10th Meeting, San Diego, US, April 2018

[2]	R.-L. Liao, C. S. Lim, "CE10: Triangular prediction unit mode (CE10.3.1 and CE10.3.2)", Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0144, 11th Meeting, Ljubljana, SI, July 2018

[3]	R.-L. Liao, C. S. Lim, "CE10.3.1.b: Triangular prediction unit mode", Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L0124, 12th Meeting, Macao, CN, October 2018

[4] 	Y. Ahn, H. Ryu, D. Sim (Digital Insights), "Diagonal motion partitions on top of QTBT block structure", Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-H0087, 8th Meeting, Macao, CN, October 2017

[5]	Y. Ahn, D. Sim (Digital Insights), "CE10-related: Diagonal motion partitions on top of MTT block structure", Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0270, 11th Meeting, Ljubljana, SI, July 2018

[6]	Y. Ahn, D. Sim (Digital Insights), "CE10: Diagonal motion partitions with uni-prediction constraint (Test 10.3.3)", Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L0125, 12th Meeting, Macao, CN, October 2018

[7]	B. Bross, J. Chen, S. Liu (editors), "Versatile Video Coding (Draft 3)", Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L01001, 12th Meeting, Macao, CN, October 2018

[8]	VVC Reference Software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

[9] 	F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1010, 12th Meeting, Macao, CN, October 2018.

[10] R. Hashimoto, S. Mochizuki, "AHG5: How to use the software to evaluate memory bandwidth", Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0451 11th Meeting, Ljubljana, SI, July 2018

# Acknowledgments

Simulations were performed with computing resources granted by RWTH Aachen University under project rwth0333.

# Patent rights declaration(s)

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).