| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0783-v1 |
| --- | --- |

| Title: | CE3-related: Modification of MPM list order |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo Jangwon Choi Jungah Choi Sunmi Yoo Ling Li Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 jin78.heo@lge.com jangwon84.choi@lge.com jungah.choi@lge.com sunmi.yoo@lge.com l.li@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a modification of MPM list order for zero reference line. The proposed method maintains the same MPM structure and gives the priority to a planar mode under certain conditions. Simulation results for this test reportedly showed average luma BD-rate changes of -0.08% and X.XX% compared to VTM3.0-AI and VTM3.0-RA, respectively.

Proposed method

The proposed method changes MPM list order for the following two cases for zero reference line.

- If two neighboring modes are different and two neighboring modes are greater than DC mode.

- If one neighboring mode is planar or DC, and the other neighboring mode is angular mode.

First case, MPM list is derived as follows:

mpm[0] = leftIntraDir mpm[1] = Planar_IDX mpm[2] = aboveIntraDir mpm[3] = DC_IDX If maxAB − minAB is in the range of 2 to 62, mpm[4] = 2 + ( ( maxAB + 61 ) % 64 ) mpm[5] = 2 + ( ( maxAB − 1 ) % 64 ) otherwise mpm[4] = 2 + ( ( maxAB + 60 ) % 64 ) mpm[5] = 2 + ( ( maxAB ) % 64 )

Second case, MPM list is derived as follows:

mpm[0] = PLANAR_IDX mpm[1] = maxAB mpm[2] = DC_IDX mpm[3] = 2 + ( ( maxAB + 61 ) % 64 ) mpm[4] = 2 + ( ( maxAB − 1 ) % 64 ) mpm[5] = 2 + ( ( maxAB + 60 ) % 64 )

Experimental results

This CE test has been implemented based on VTM3.0 reference software and experimentally evaluated for VTM test according to common test conditions defined in JVET-L1010 [1].

Table 1 Results of CE3-1.3 over VTM3.0-AI

Table 2 Results of CE3-1.3 over VTM3.0-RA

Reference

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L1010, 12th meeting, Macao, CN, 3-12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).