| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0815-v2 |
| --- | --- |

| Title: | CE3-related: Harmonization on MPM list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li Jin Heo Jangwon Choi Jungah Choi Sunmi Yoo Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 | Tel: Email: | +82-2-6912-6561 {l.li, jin78.heo, jangwon84.choi,sunmi.yoo, jaehyun.lim, seunghwan3.kim }@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

JVET-M0783 proposed a difference ordering of MPM list for zero reference index. On top of that, MPM list harmonization for zero reference index and non-zero reference indices are proposed. When non-zero reference indices are used, mpm list is generated by extracting four intra angular modes from the zero reference index MPM list. The experiment result reportedly shows -0.05% and x.xx% bit rate changes for AI and RA.

Proposed Method

In current VTM 3.0 as well as JVET-M0783, Planar and DC modes are always included in the MPM list. However, those two modes are not used when non-zero reference indices are applied. Thus a simple way to harmonize MPM list, is to simply extract four angular modes from the list.

| MPM list for zero reference index | MPM list for non-zero reference index |
| --- | --- |
| mpm[0] = PLANAR_IDX mpm[1] = DC_IDX mpm[2] = VER_IDX; mpm[3] = HOR_IDX; mpm[4] = VER_IDX - 4; mpm[5] = VER_IDX + 4; | mpm[0] = VER_IDX; mpm[1] = HOR_IDX; mpm[2] = VER_IDX - 4; mpm[3] = VER_IDX + 4; |

Figure 1. Example of extracting four angular intra modes from MPM list

Experimental Results

The proposed method is implemented on top of VTM 3.0, and evaluated for VTM configuration according to the common test conditions [2].

Table 1 Experiment result for all intra (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.23% | -0.07% | 98% | 103% |
| Class A2 | -0.05% | 0.04% | 0.00% | 99% | 102% |
| Class B | -0.04% | -0.07% | 0.02% | 100% | 102% |
| Class C | 0.06% | 0.01% | -0.02% | 99% | 101% |
| Class E | -0.05% | 0.06% | 0.05% | 99% | 102% |
| Overall | -0.05% | -0.04% | 0.00% | 99% | 102% |
| Class D | -0.03% | -0.09% | -0.02% | 99% | 100% |
| Class F | 0.36% | 0.27% | 0.37% | 99% | 102% |

Summary

The proposed harmonization on MPM list shows -0.05% and x.xx% bit rate changes for AI and RA. It is recommended to adopt this proposal to VVC.

References

- VTM 3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, Document of Joint Video Experts Team (JVET), JVET-L1010, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).